3GPP TSG-RAN WG4 Meeting #117	R4-2521672

# Dallas, TX, USA, November 17-21, 2025

Source:	Ericsson

Title:	Removal of UTRA TDD from MSR BS specifications

Agenda item:	10.3

Document for:	Approval

# Background

UTRA TDD has been included in the MSR BS specifications since Rel-9. It seems however that UTRA TDD MSR BS are not anymore produced and there are not any UTRA TDD deployments.

This contribution explores the possibility to remove the UTRA TDD RAT and protection of UTRA TDD from MSR BS specifications. Accompanying CRs for TS 37.104 [1] and TS 37.141 [2] would implement the proposed removal.

# Discussion

There are two aspects of UTRA TDD in the MSR BS specifications:

- MSR BS capability of operating UTRA (TDD) in unpaired BC3 bands. Note that this capability is limited to cover the 1.28 Mcps UTRA TDD option as stated in section 4.5 of TS 37.104 [1] and TS 37.141 [2].

- Co-existence with UTRA TDD operation in adjacent frequencies (ACLR, Tx IM, Rx blocking and Rx IM) and co-existence and co-location with UTRA TDD in other bands. Related requirements cover all UTRA TDD options (1.28 Mcps, 3.84 Mcps and 7.68 Mcps)

The two points are discussed separately below.

## MSR BS capable of UTRA TDD

UTRA TDD (1.28 Mcps) was included from the first release (Rel-9) of the MSR BS core specification TS 37.104 [1]. It is reflected through listing of UTRA TDD bands for BC3, applicability of requirement for UTRA TDD operation, and specific UTRA TDD RF and demodulation requirements, mostly through references to the UTRA TDD specification. There are also additional UTRA TDD requirements for BS output power, Output power dynamics, Dynamic range, Modulation quality, Frequency error and Time alignment.

In the MSR BS conformance specification TS 37.141, the same UTRA TDD entries exist as in the core specification. In addition, there are UTRA TDD specific Test Configurations (TC1b, TC3b and TC6c), Capability Sets for BC3 that include UTRA TDD capability (CS1, CS3 and CS14) and specific UTRA TDD test signals.

All aspects listed above can be removed, if UTRA TDD capable BS are not any more needed.

## Co-existence with UTRA TDD

As in all other specifications, specific requirements are defined to protect and co-exist with operation in adjacent frequencies and other bands. Several of those are specific to co-existing with UTRA TDD for MSR BS operation (TS 37.104 [1] references):

- BS Spurious emissions limits for co-existence with systems operating in other frequency bands (6.6.1.3):
The UTRA TDD operating bands named a) to f) are listed as “System type to co-exist with“.

- BS Spurious emissions limits for BS co-located with another BS (6.6.1.4):
The UTRA TDD operating bands named a) to f) are listed also here as “Type of co-located BS”.

- Base Station ACLR in unpaired spectrum with synchronized operation (6.6.4.1):
All UTRA TDD bandwidths (1.28 Mcps, 3.84 Mcps and 7.68 Mcps) are listed as “Assumed adjacent channel carrier”. This leads to unnecessary testing.

NOTE: 	None of the UTRA bandwidths are listed as “Assumed adjacent carrier” for NR in TS 38.104 [7] and can thus be seen as obsolete at this time.

- Additional minimum requirement (BC3) for Transmitter intermodulation (6.7.3) 
For colocation with UTRA TDD BS.

- Additional BC3 blocking minimum requirement (7.4.5)
For operation in the same area as UTRA TDD.

- Receiver intermodulation (7.7.1 and 7.7.2)
1.28 Mcps UTRA TDD is listed as “RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or sub-block edge”.

The requirements listed above are not relevant in areas where UTRA TDD is not anymore in operation and UTRA TDD would thus not need specific protection. Note that even if the specific protection requirements are removed, the level of protection for adjacent frequencies and other bands would remain in principle unchanged, since there are multiple such requirements remaining. Removing the UTRA TDD specific requirements would however simplify testing and declaration of conformance.

## Deployment of UTRA TDD

UTRA TDD was never widely adopted by major operators. The UTRA TDD mode saw only very limited commercial deployment and today has been almost entirely superseded by other standards. Most mobile networks deployed for 3G were using UTRA FDD, and operators who wanted TDD operation in unpaired bands moved to E-UTRA TDD and later 5G NR TDD.

In Europe, ETSI concluded in 2024 that UTRA TDD was no longer deployed and its removal from the harmonised standard was considered [3]. Based on this conclusion, all aspects of UTRA TDD including related BC3 requirement and co-existence limits were removed. The latest version (Release 17) of the MSR BS harmonised standard in EN 301 908-18 [4] does thus not contain any UTRA TDD-related requirements.

# Proposals

Based on the conclusion that UTRA TDD is no longer deployed, the following is proposed:

Proposal 1: 	All UTRA TDD requirements are removed from the MSR BS specifications, including UTRA TDD bands for BC3, applicability of requirement for UTRA TDD operation, specific UTRA TDD RF requirements and additional UTRA TDD requirements for Tx IM, dynamic range, blocking and Rx IM. In addition, for the MSR BS test specification, UTRA TDD specific Test Configurations, Capability Sets for BC3 that include UTRA TDD capability and UTR TDD test signals are removed.

Proposal 2:	Protection of UTRA TDD are removed from BS spurious emission limits for co-existence and co-location, and from BS ACLR in unpaired spectrum with synchronized operation. This concerns all UTRA TDD bandwidths (1.28 Mcps, 3.84 Mcps and 7.68 Mcps).

CRs (Cat F) for implementation of the above proposals are presented in [5] and [6]. It is proposed to implement the changes in Rel-17, for alignment with European Harmonised Standards.

If a more thorough review of the proposals above and related CRs is needed, decisions can be postponed until RAN4#118.

# References

- 3GPP TS 37.104 V17.14.0 (2024-12), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception (Release 17)”.

- 3GPP TS 37.141 V17.15.0 (2024-12), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing (Release 17)”.

- TFES(24)078019, “Requirements for UTRA TDD and BC3 in EN 301 908 part 18”.

- ETSI EN 301 908-18 V17.1.1 (2025-02), “IMT cellular networks; Harmonised Standard for access to radio spectrum; Part 18: NR, E-UTRA, UTRA and GSM/EDGE Multi-Standard Radio (MSR) Base Station (BS) Release 17”.

- R4-2521673, "(RInImp9-Rfmulti,TEI) CR to 37.104: Removal of UTRA TDD from MSR BS specifications" (Ericsson).

- R4-2521676, "(RInImp9-Rfmulti,TEI) CR to 37.141: Removal of UTRA TDD from MSR BS specifications" (Ericsson).

- 3GPP TS 38.104 V17.16.0 (2024-12), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Base Station (BS) radio transmission and reception (Release 17)”.