3GPP TSG-RAN WG4 Meeting #117	R4-2521685

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.15.2.2

Source: 	Samsung

Title: 	Discussion on L1-RSRP test case for SSB adaptation

Document for:	Discussion

- Introduction

In this meeting, we will continue giving some analyses on L1-RSRP TC design based on WF [1] agreed in last meeting.

- Discussion

Agreements achieved in previous meetings

The following scope/principle was agreed to assist defining test cases for SSB adaptation (only L1 measurement related)

Issue 1-1: TC(s) to be specified besides L3 intra-frequency measurement Agreement: - Besides L3 intra-frequency measurement, specify the following test case for SSB adaptation - L1-RSRP measurement - L1-SINR measurement Issue 1-2: General principle Agreement: - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only - Both FR1 and FR2 - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement - Only test non-DRX mode - without SSB time index detection - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. Issue 1-3: SMTC and SSB configuration Agreement (in AH): - For L1-RSRP measurement test case: - SMTC periodicity from 20ms to 80ms, and from 80ms to 40ms. - SSB periodicity from 20ms to 80ms, and from 80ms to 40ms. - For other test cases: - SMTC periodicity from 80ms to 20ms. - SSB periodicity from 80ms to 20ms. Agreement - TC1: L3 intra-frequency measurement - FR1 - TC2: L1-RSRP measurement - PCell FR1 and SCell FR2 - TC3: L1-SINR measurement - FR1 Issue 1-4: How to verify SSB adaption Agreement: - For L3 Intra-frequency serving Cell measurement: - UE is configured with Event A1 or A2. - Condition of serving Cell is changed to satisfy the Event threshold at the instance after DCI 2-9 received + the transition period. - It is verified whether UE can trigger event within the delay based on the mapped additional SMTC.

Test requirements

Different from the legacy L1-RSRP test case design, for DCI format 2_9 indication based SSB adaptation, besides m+d slots defined in RAN1, additional processing time to determine the first SSB burst for L1-RSRP measurement requirement defined in RAN4, i.e., 2 ms and 3 ms for FR1 and FR2 respectively, are considered, namely, the highlighted part in 9.5.7 Minimum requirement at transitions. In this sense, the additional processing time met should be represented in the UE behaviour definition in the test requirements.

9.5.7 Minimum requirement at transitions [Omit] For UE supporting SSB adaptation, when UE receives DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission during one mesauremnet period, the L1-RSRP measurement period requirements based on the indicated SSB burst periodicity applies from the first SSB burst after the time point as defined in TS 38.213 plus 2 ms and 3 ms for FR1 and FR2 respectively.

Proposal 1: For DCI format 2_9 indication based SSB adaptation, additional processing time (i.e., 2 ms or 3 ms) to determine the first SSB burst for L1-RSRP measurement should be considered in the test requirements.

Test purposes

From our understanding, how to design the L1-RSRP test case depends on what functionality, in other words, what purpose would like to be verified by the test. The following two purposes can be considered: Purpose 1: To verify the UE supporting ssb-BurstPeriodicityAdaptation-r19 makes correct reporting of L1-RSRP measurement when SSB periodicity changes due to SSB adaptation plus to verify the UE supporting ssb-BurstPeriodicityAdaptation-r19 can track the changes in the Tx beams that UE is configured to measure, so that the NW can select the best one to schedule data . Purpose 2: To verify the UE supporting ssb-BurstPeriodicityAdaptation-r19 makes correct reporting of L1-RSRP measurement when SSB periodicity changes due to SSB adaptation. The TC design of purpose 1 is relatively complicated, and the Tx beam tracking functionality was already verified in the basic TC.

To make purpose 2 come true, one SSB configured with different periodicities is considered, and the SSB#0 with updated periodicity is right next to the m+d+additional processing time. To be specific, during T1/T2/T3, only SSB #0 is transmitted but with different periodicities triggered by DCI format 2_9 SSB adaptation indication. And a correct UE should report accurate L1-RSRP measurements at the configured reporting instances no matter how the periodicity is changed.

Purpose 2: To verify the UE supporting ssb-BurstPeriodicityAdaptation-r19 makes correct reporting of L1-RSRP measurement when SSB periodicity changes due to SSB adaptation

  - Only one SSB (i.e., SSB#0) is configured

- Conclusions

In this contribution, we provided our viewpoints on L1-RSRP test case design for NES SSB adaptation. The following proposals are obtained:

Proposal 1: For DCI format 2_9 indication based SSB adaptation, additional processing time (i.e., 2 ms or 3 ms) to determine the first SSB burst for L1-RSRP measurement should be considered in the test requirements.

Purpose 2: To verify the UE supporting ssb-BurstPeriodicityAdaptation-r19 makes correct reporting of L1-RSRP measurement when SSB periodicity changes due to SSB adaptation

  - Only one SSB (i.e., SSB#0) is configured

References

[1] R4-2514809, WF on Netw_Energy_NR_enh_RRM_Part2, Huawei