3GPP TSG-RAN WG4 Meeting # 117                                                               R4-2521687

Dallas, USA, 17th – 21st November, 2025

Source:	Samsung

Title:	Discussion on Rel-20 RRM Enhancements redcap

Agenda item:			7.4.3.2

Document for:	Discussion

1	Introduction

In this meeting, we will give continue discussions on Rel-20 RRM Enhancements redcap based on agreed WF [1].

2	Relaxed RLM/BFD RRM requirements

The feasibility and performance impact of relaxing UE measurements for RLM and/or BFD was discussed in Rel-17, targeting good serving cell quality and UE low mobility state with short DRX cycle. Regarding the criteria:

- Entry Criteria (Rel-17 RLM/BFD relaxation criteria)

- Case 1： provided UE is configured the explicit signalling (goodServingCellEvaluationBFD) to enable RLM/BFD relaxation and UE has fulfilled good serving cell criterion, if low mobility criteria is NOT configured - Case 2：provided UE is configured the explicit signalling to enable RLM/BFD relaxation and UE has fulfilled both good serving cell criterion (goodServingCellEvaluationBFD) and low mobility criterion (lowMobilityEvaluationConnected) if low mobility criteria is configured

- Exit Criteria (Rel-17 RLM/BFD relaxation criteria)

- RLM: - The UE is no longer allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met: - The UE sends out-of sync indications to the higher layers, - The timer T310 is running. - No DRX is configured or configured DRX cycle is longer than 80 ms - BFD: - The UE is no longer allowed to relax BFD measurements and apply the relaxed link recovery procedures provided that at least one of the following conditions is met: - The timer beamFailureDetectionTimer is running. - No DRX is configured or configured DRX cycle is longer than 80 ms

Based on the background above, for the Rel-17 RLM/BFD relaxation, low mobility and good serving cell criterion are considered, and the corresponding requirements are defined based on such criterions, accordingly. The same criteria will be reused in our case.

Proposal 1: For RLM/BFD RRM requirement relaxation for (e)Redcap, criteria defined in Rel-17 RLM/BFD relaxation including exit and entry criteria can be reused.

Since in the current Spec., it is observed that the following two aspects: 1) The RLM requirement (i.e., evaluation period) defined for 2Rx RedCap UE, 2) UE capability of PDCCH monitoring (i.e., PDCCH transmission parameters for OOS/IS evaluation) considered for 2Rx RedCap UE are the same as the ones specified for non-RedCap UE, if RLM and/or BFD requirements can be relaxed for non-RedCap UE under certain conditions, the corresponding requirements can be relaxed for 2Rx RedCap UE, and in theory, the same principle can be reused.

Regarding the relaxed factor, the following Tables give an example regarding how to specify the relaxed RLM/BFD measurement requirement for non-RedCap UE.

Table 8.1.2.4-1: Evaluation period TEvaluate_out_SSB_Relax for FR1

| Configuration | TEvaluate_out_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80 ms | Max(200 K3 NOTE3, Ceil(15 K1NOTE2 P) Max(TDRX,TSSB)) |
| 80 ms < Max(TDRX,TSSB) ≤160 ms | Ceil(15 P) Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80 ms. NOTE 2: K1 = 4 for Max(TDRX,TSSB) ≤40 ms and K1 = 2 for 40 ms<Max(TDRX,TSSB) ≤80 ms. NOTE 3: K3 = K1, if K1 ≤ 2; otherwise K3 = 1. |  |

Combining the analysis above, from our understanding, for the Redcap UE with 2Rx in Rel-20, the current relaxed factor K3 and K1 can be reused when the DRX cycle is no longer than 80 ms for RLM/BFD RRM requirement relaxation.

Proposal 2: For RLM/BFD RRM requirement relaxation for (e)Redcap UE with 2Rx, the existing relaxation factor K1 and K3 can be reused

3	Conclusion

In this contribution, we provided our viewpoints on RRM requirements on relaxed RLM/BFD RRM requirements for (e)RedCap UEs. The following observations and proposals are obtained:

Proposal 1: For RLM/BFD RRM requirement relaxation for (e)Redcap, criteria defined in Rel-17 RLM/BFD relaxation including exit and entry criteria can be reused.

Proposal 2: For RLM/BFD RRM requirement relaxation for (e)Redcap UE with 2Rx, the existing relaxation factor K1 and K3 can be reused

Reference

[1] R4-2514821, WF on NR_RRM_Ph6