3GPP TSG-RAN WG4 Meeting #117	R4-2521718

Dallas, United States, 17th – 21st November 2025

Source:	Nokia

Title:	Simulation results for demodulation performance requirements for MIMO_Ph5

Agenda item:	6.12.3

Document for:	Information

Introduction

This document presents simulation results regarding demodulation performance and CSI reporting requirements for the MIMO_Ph5 based on the earlier WF [1] to reach the alignment.

Simulation results for 3Tx

Test assumptions

Table 1 Simulation parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0, 1, 2} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 15kHz SCS: 5 and 50MHz 30kHz SCS: 10 and 100MHz |
|  | Frequency hopping | Disabled |
| TPMI index for 3Tx three layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| Channel model and MCS |  | TDLB100-400: MCS2 TDLC300-100: MCS16 TDLA30-10: MCS20 |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. |  |  |

Simulation results

Table 2 Simulation results with 70% of normalized throughput for PUSCH Mapping Type A and 15kHz SCS

| BW | Rx Antennas | MCS | Channel | SNR [dB] |
| --- | --- | --- | --- | --- |
| 5MHz | 4 | MCS2 | TDLB100-400 | -3.69 |
|  |  | MCS16 | TDLC300-100 | 8.45 |
|  |  | MCS20 | TDLA30-10 | 12.07 |
|  | 8 | MCS2 | TDLB100-400 | -6.33 |
|  |  | MCS16 | TDLC300-100 | 4.09 |
|  |  | MCS20 | TDLA30-10 | 6.39 |
| 50MHz | 4 | MCS2 | TDLB100-400 | -3.65 |
|  |  | MCS16 | TDLC300-100 | 8.89 |
|  |  | MCS20 | TDLA30-10 | 12.91 |
|  | 8 | MCS2 | TDLB100-400 | -5.82 |
|  |  | MCS16 | TDLC300-100 | 4.35 |
|  |  | MCS20 | TDLA30-10 | 7.18 |

Table 3 Simulation results with 70% of normalized throughput for PUSCH Mapping Type B and 15kHz SCS

| BW | Rx Antennas | MCS | Channel | SNR [dB] |
| --- | --- | --- | --- | --- |
| 5MHz | 4 | MCS2 | TDLB100-400 | -3.61 |
|  |  | MCS16 | TDLC300-100 | 8.7 |
|  |  | MCS20 | TDLA30-10 | 12.12 |
|  | 8 | MCS2 | TDLB100-400 | -6.16 |
|  |  | MCS16 | TDLC300-100 | 4.12 |
|  |  | MCS20 | TDLA30-10 | 6.54 |
| 50MHz | 4 | MCS2 | TDLB100-400 | -3.55 |
|  |  | MCS16 | TDLC300-100 | 8.91 |
|  |  | MCS20 | TDLA30-10 | 12.85 |
|  | 8 | MCS2 | TDLB100-400 | -5.77 |
|  |  | MCS16 | TDLC300-100 | 4.36 |
|  |  | MCS20 | TDLA30-10 | 7.25 |

A 50 MHz bandwidth requires an SNR target that is 0.1 to 0.8 dB higher than a 5 MHz bandwidth, with greater performance loss seen at higher MCS levels.

There is no significant difference in performance between type A and B mapping for the simulated scenarios.

Conclusion

This contribution contains the Nokia’s simulation results for MIMO_Ph5

We have made the following observations based on our results:

Simulation results for 3Tx

- A 50 MHz bandwidth requires an SNR target that is 0.1 to 0.8 dB higher than a 5 MHz bandwidth, with greater performance loss seen at higher MCS levels.

There is no significant difference in performance between type A and B mapping for the simulated scenarios.

- References

- R4-2514806 - WF on NR_MIMO_demod