3GPP TSG RAN WG4 #117	R4-2521779

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.15.2.3

Source: 	Ericsson

Title:	Remaining issues on OD-SIB1 performance requirements

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 discussed about OD-SIB1 test case design with the following WF.

Issue 2-1: Whether to define TC for OD-SIB1 Agreement: - RAN4 to define cell reselection TC with longer TSI-NR for OD-SIB1. Issue 2-2: Detail test case design Way Forward - Option 1: RAN4 to define a new test case to verify the cell reselection behaviour together with longer TSI-NR in case of OD-SIB1. - UE transmits UL-WUS transmission - UE receives and decodes RAR - UE receives OD-SIB1 within OD-SIB1 window - Further discuss the exact value on the longer TSI-NR and the ending point - Option 2: - Target Cell (Cell2) is a NES Cell. - The ending point of the TC is UL WUS transmission. - UE shall be able to transmit UL WUS no later than: Tdetect, NR_Intra + TSI-NR + TUL-WUS, where TSI-NR is 1280ms as defined in existing test cases, and TUL-WUS is the time to the first UL WUS occasion after TSI-NR.

In this contribution, we’ll discuss the related test case design for OD-SIB1 procedure[1].

# OD-SIB1 test case

In last RAN4 meeting, RAN4 discussed how to test cases for OD-SIB1 acquisition with cell reselection in IDLE mode. RAN4 agrees to define a test case with longer TSI-NR in performance part. To perform the new OD-SIB1 acquisition procedure in IDLE mode, the following steps can be follows.

- Step 0: Two cells are configured, cell 1 and cell 2. Cell 2 is an OD-SIB1 cell.

- Step 1: The UE evaluates cell reselection based on measurements towards SSB of target cell2.

- Step 2: After cell reselection delay, the UE initiates OD-SIB1 procedure. (The total delay should be defined in the test)

  - Step 2-1: UE transmits the UL-WUS(PRACH)

  - Step 2-2: UE receives the RAR

  - Step 2-3: UE monitors OD-SIB1 window to decode the SIB1 and OSIs

However, to facilitate the test, after UE finishes the OD-SIB1 reading, UE needs to trigger another RACH to NW similar as legacy test case. Therefore, to verify the UE successful access the OD-SIB1 cell, the step 2-4 is still needed: UE triggers the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility.

Generally, a typical RACH procedure is less than 100ms in NR. Considering the test margin for cell reselection(from 33.28s to 34s), the margin is enough to guarantee UE performing the OD-SIB1(RACH) procedure.

Proposal 1:  RAN4 to define a new OD-SIB1 test case to verify the cell reselection behaviour together with the same TSI-NR as legacy.

  - UE transmits UL-WUS transmission

  - UE receives and decodes RAR

  - UE receives OD-SIB1 within OD-SIB1 window with TSI-NR

  - UE triggers the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility

- Summary

In this contribution, we have discussed the performance part of OD-SIB1 procedure. Based on the discussions, we made the following proposals and observations:

Proposal 1:  RAN4 to define a new OD-SIB1 test case to verify the cell reselection behaviour together with the same TSI-NR as legacy.

  - UE transmits UL-WUS transmission

  - UE receives and decodes RAR

  - UE receives OD-SIB1 within OD-SIB1 window with TSI-NR

  - UE triggers the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility

- References

- R4-2514809, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei