3GPP TSG-RAN WG4 Meeting #117						         	       		R4-2521792

Dallas, USA, Nov. 17-21, 2025

Title: 	On UL/DL high order modulation for 6G

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.2.2	Channel bandwidth [FS_6G_Radio]

Document for:	Approval

# 1	Background

In RAN#116bis, a way forward [1] captured several guidelines/agreements for the 6G system parameters study. In his contribution, we cover the UE PA and PA transceiver baseline performance for 6G and its impact on higher order modulations like UL 1024QAM. Additionally, we discuss the EVM budget to support 1024QAM in 4096QAM in DL.

# Discussion

## 2.1 Way forward input

In RAN4#116bis, a way forward [1] captured several guidelines and agreements for the 6G system parameter study, the parts relevant to channel bandwidth aspects are copied here in italics.

Max Channel Bandwidth

- RAN4 evaluation work focus on the feasibility study and RF requirements impact

- To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1

  - Identify the main affected requirements for modulation evaluations

    - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions.

  - Align on the evaluation assumptions.

  - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation.

  - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed.

  - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1.

- Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations

  - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies.

  - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

In this contribution, we will cover the aspects highlighted in yellow and more specifically the baseline UL256 EVM budget and the needed evolution for 1024QAM accounting for the PA limitations.

## 2.2 EVM budget for UL 256QAM and 1024QAM

As a starting point, we first need to establish the baseline support in 6G. Our proposal in the UERF thread is copied here.

Default supported UL QAM:

- UL 256QAM for band ≤5GHz

- UL 64QAM for band >5GHz and <16GHz

- FFS for >16GHz

- For NTN bands it may not be needed to specify A-MPR for the highest QAM

- UL 1024QAM is considered optionally

  - The Tx impairments should be further improved accordingly, and Tx EVM should be based on assuming improved Rx EVM on the BS side.

For the discussion on modulation, it is fair to start from a baseline 6G support of 256QAM which already provides a starting point in terms of EVM budget, TRX impairments, and required PA back-off:

- The link EVM target was 5% with equal contribution of the UE and BS at 3.5%

- This requires TRX IQ impairments to be at least 34dB

- This requires a PA back-off of 4.5dB and 6.5dB for DFT-s-OFDM and CP-OFDM respectively

Current PA EVM performance

Our Wi-Fi PAs are already designed for 1024QAM on the client side and 4096QAM on the access point side, so sub 1% EVM contribution from a PA is feasible up to 7GHz.

However, this comes at the expense of large backoff and large current as:

- The PA is polarised in class A since, even with digital pre-distortion (DPD), the intrinsic PA linearity needs to be exceptional, especially for AM-PM and memory effects that are very critical and are not fully compensated by DPD

  - This results in critical thermal drifts which are manageable for the short WiFi bursts

- There is no ET or APT based efficiency enhancement.

- This is only possible because there is no power control in Wi-Fi and the DPD only works at maximum power and has a limited number of waveforms to be trained for: only OFDM with mostly fully allocated subcarriers (no or limited partial allocation) always operating at max power.

For all the above reasons, it is not reasonable to base the 6G UL1024QAM study with Wi-Fi PAs in mind and rather see how far the current cellular PAs can be improved with all the state-of-the-art techniques using ET and DPD.

A higher backoff than the one required for UL256QAM might be needed for 1024QAM to make sure the intrinsic PA linearity is already sufficient for the DPD to be effective in all corners, a good Tx SNR shall be maintained over a large output power dynamic range, and one should not overestimate the performance by looking at max power behaviour.

From experience of various ET and APT implementations with various DPD algorithms, bandwidth and training strategies, the overall performance when using the same PA can vary significantly over dynamic range, frequency and temperature so the system performance should not be dimension based on ideal linearization behaviour.

Based on this experience, we suggest that a state of the art and reasonable in band SNR for the PA+ET+DPD can target up to 38dB at least for the upper 20dB of output power and >6dB PA backoff to ensure enough intrinsic PA performance. This is state of the art at 100MHz modulation BW and can probably be extended to 200MHz modulation BW but ET and DPD will be challenged to go beyond while the PA AM/PM and memory effect will be more significant.

EVM budget

In Table 1, we present a potential EVM budget for 6G UL1024QAM based on 5G UL256QAM experience and extrapolating based on:

- What can be aided by digital techniques and/or calibration: digital and analog BB dynamic range and and IQ impairments

- What is an intrinsic analog performance that is more difficult to improve and difficult to maintain at high frequencies:

  - LO phase noise (especially at high frequencies)

  - PA intrinsic linearity (AM/AM, AM/PM and memory effects some of which may not be compensated by DPD and are not represented in PA models for simulations)

It should be noted that the below budget is a static EVM budget. However, unlike Wi-Fi, power control in cellular induces a further contribution with the transient EVM especially in FDD bands.

Table 1: Tentative UL1024QAM EVM budget based on UL256QAM 5G budget

Observations:

- Even with allocating a 1% contribution for the UE contributors that can be digitally aided and the BS, the system budget is 2.5% which may prove insufficient.

  - This will be further degraded with transients and at low power when the RF noise floor will start to contribute

- The BS support the largest improvement, then TRX IQ (assuming very good calibration) and then LO and PA.

- The PA(ET+DPD) only contributes 0.4% to the UE EVM.

- The BS Rx EVM of 1% may sound challenging. However, if DL4096QAM is foreseen this should be feasible for a BS if a UE can achieve it.

The EVM budget above is not meant to be an exact proposal but rather an exercise to show what ballpark number should be targeted for the different contributors. Further elaboration of the UL1024QAM EVM budget can be discussed based on the below proposals.

Proposal on EVM budget for UL1024QAM:

- Baseline 6G performance is UL256QAM at least up to 5GHz requiring IQ impairments at 34dB

- PA performance target for 6G UL1024QAM cannot be based on Wi-Fi PAs as their operation is simpler with no power control, limited waveform variety and short bursts.

- It is reasonable to assume 38dB in band SNR for an ET PA with DPD assuming:

  - Enough power backoff so that the intrinsic PA distortions are already good

  - A modulation BW no higher than 200MHz

  - RF frequency up to 10GHz, FFS up to 15GHz

  - Proper DPD training over a large set of waveforms and power range and a DPD BW of at least 3x the modulation BW.

- The UE/BS EVM split cannot be of equal contribution and the BS EVM should be significantly better than the UE EVM

- Additional margin is needed in the system to account for transients and RF chain noise floor to enable UL1025 QAM over a n acceptable output power dynamic range

- To improve the scope of the UL1024QAM study it would be beneficial for RAN4 to agree on:

  - Link level EVM target for 1024QAM

  - Maximum UL modulation bandwidth targeted

  - Maximum UL frequency targeted.

## 2.3 Higher order modulation in DL

For DL our assumption is that 1024QAM is also targeted and that DL4096QAM may be an optional target for FWA for example.

As, an exercise, similar than what we did for UL1024QAM, Table 2 presents some potential EVM budget for DL1024QAM and DL4096QAM.

For the DL1024QAM case, we have assumed the following:

- TRX IQ impairment and LO phase noise is the same in Rx as in Tx

- BS Tx EVM has a similar budget as the PA for UL1024QAM

- Link EVM target is the same as for UL

Table 2: Tentative EVM budgets for DL1024QAM and DL4096QAM

Observations:

- For DL1024QAM reusing the UE UL1024QAM budget, provided a BS Tx EVM is <1.5% the same link EVM of 2.5% can be achieved

- For DL4096QAM significant improvement is needed for all contributors, assuming that the UE LO phase noise is the most critical parameter, a link EVM of 1.4% is achieved which may prove insufficient.

The EVM budget above is not meant to be an exact proposal but rather an exercise to show what approximate number should be targeted for the different contributors. Further elaboration of the DL4096QAM EVM budget can be discussed based on the below proposals.

Proposal for DL1024QAM and DL4096QAM

- DL1024QAM is considered as baseline for 6G

  - Assuming a link target of 2.5% and a contribution of the BS of 1.5%, a UE contribution of 2% EVM is achievable with the same Rx phase noise and IQ impairment than in the UL.

- DL4096QAM is studied as an optional modulation for 6G

  - All contributors need to improve significantly (at least 2x versus 1024QAM) and, assuming that the most critical is the UE LO phase noise especially at higher RF frequencies, Sub 1% EVM will be needed from the BS Tx and most contributors.

- To improve the scope of the DL1024QAM and DL4096QAM study it would be beneficial for RAN4 to agree on:

  - Link level EVM target for 1024QAM and 4096QAM

  - Maximum DL modulation bandwidth targeted

  - Maximum DL frequency targeted.

# Conclusion

In this contribution, we provide studies on the dimensioning of EVM budgets for 6G to support 256QAM and 1024QAM in UL and 1024QAM and 4096QAM in DL.

Proposal on EVM budget for UL1024QAM:

- Baseline 6G performance is UL256QAM at least up to 5GHz requiring IQ impairments at 34dB

- PA performance target for 6G UL1024QAM cannot be based on Wi-Fi PAs as their operation is simpler with no power control, limited waveform variety and short bursts.

- It is reasonable to assume 38dB in band SNR for an ET PA with DPD assuming:

  - Enough power backoff so that the intrinsic PA distortions are already good

  - A modulation BW no higher than 200MHz

  - RF frequency up to 10GHz, FFS up to 15GHz

  - Proper DPD training over a large set of waveforms and power range and a DPD BW of at least 3x the modulation BW.

- The UE/BS EVM split cannot be of equal contribution and the BS EVM should be significantly better than the UE EVM

- Additional margin is needed in the system to account for transients and RF chain noise floor to enable UL1025 QAM over a good output power dynamic range

- To improve the scope of the UL1024QAM study it would be beneficial for RAN4 to agree on:

  - Link level EVM target for 1024QAM

  - Maximum UL modulation bandwidth targeted

  - Maximum UL frequency targeted.

Proposal for DL1024QAM and DL4096QAM

- DL1024QAM is considered as baseline for 6G

  - Assuming a link target of 2.5% and a contribution of the BS of 1.5%, a UE contribution of 2% EVM is achievable with the same Rx phase noise and IQ impairment than in the UL.

- DL4096QAM is studied as an optional modulation for 6G

  - All contributors need to improve significantly (at least 2x versus 1024QAM) and, assuming that the most critical is the UE LO phase noise especially at higher RF frequencies, Sub 1% EVM will be needed from the BS Tx and most contributors.

- To improve the scope of the DL1024QAM and DL4096QAM study it would be beneficial for RAN4 to agree on:

  - Link level EVM target for 1024QAM and 4096QAM

  - Maximum DL modulation bandwidth targeted

  - Maximum DL frequency targeted.

# References

[1] R4-2514641 WF on [116bis][101] 6G system parameter, Huawei RAN4#116b

[2] R4-2513115 Inputs to RAN4 6G study on general RF and UERF, Skyworks Solutions Inc., RAN4#116b

[3] R4-2513114 Inputs to RAN4 6G study on system parameters, Skyworks Solutions Inc., RAN4#116b

[4] R4-2513062 Inputs to RAN4 6G study on general aspects, Skyworks Solutions Inc., RAN4#116b