# 3GPP TSG RAN WG4 Meeting #117 														R4-2521866

# Dallas, USA , November 17th – 21st, 2025

Agenda Item:	6.13.3

Source: 	Ericsson

Title:	RRM performance requirement for Rel-19 Subband full duplex evolution

Document for:	Discussion

# Introduction

The agreements on SBFD operation during RAN4#116 bis meeting are captured in the approved WF [1].

# Discussion

## 2.1 RRM performance requirements for SBFD operation

### Issue 2-1-1: Measurement accuracy for CSI-RS based L1 measurements - Background - Agreement from RAN4#114bis - Note: The following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE. - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - Way forward - Define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding [TBD] relaxation on top of existing accuracy (based on 48 RB) - For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB), - for a UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB apply. - Option 2: accuracy requirements based on 24 RB apply. - for UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB apply

Based on our simulation results which are provided in [2], we see minor differences less than 0.5 dB with all SCS being simulated between 24 and 48 PRB for delta RSRP, which can also be seen from the simulation results compared with other companies [2].

Since there is no measurement delay extension due to RAN1 final agreement for simultaneous reception of the 2DL subband, which implicitly means that at one measurement occasion, the UE will have in total more than 48PRBs, we don’t see why we need accuracy relaxation is needed.

Proposal 1: For case 3 , RAN4 specify accuracy requirement based on 24PRBs and no relaxation is needed.

# Summary

The following have been proposed in the current contribution:

Proposal 1: For case 3 , RAN4 specify accuracy requirement based on 24PRBs and no relaxation is needed.

# References

- R4-2515054 “WF on SBFD RRM”, Huawei, August ,2025

- R2-2521867 “Simulation results for CSI-RS based L1-RSRP”, Ericsson, November ,2025