3GPP TSG-RAN WG4 Meeting #117	R4-2521890

Dallas, USA, 17 – 21th November 2025

Source:	BT

Title:	On Demodulation and Performance Requirement Framework for 6G

Agenda item:	8.7

Document for:	Discussion

Introduction

This contribution extends BT’s earlier input [1] outlining specific study items that can operationalise the principles proposed therein based on the conclusions in [2]. The goal is to guide RAN4’s 6G demodulation study toward a structured, physically grounded, and deployment-aligned performance requirement framework.

Discussion

General Aspects

Experience from previous releases has shown that overly compressed study timelines may lead to inconsistent requirement coverage and fragmented topic progress. A structured and realistic schedule is needed to allow sufficient time and scope for meaningful technical discussions. First phase of such discussions should start with a review of current 5G NR air performance aspects, without waiting for key agreements on any enhancements.

adopt a realistic, phased study schedule that begins with a review of current 5G NR air performance aspects, ensuring sufficient time for comprehensive technical discussions before pursuing enhancement agreements.

For waveform and modulation, a pragmatic starting point is to retain NR-based waveforms, i.e. CP-OFDM for downlink and both CP-OFDM and DFT-s-OFDM for uplink, while extending to higher modulation orders up to 1024QAM to reflect expected 6G capabilities. Initial study numerologies can reuse well-understood SCS values (15/30 kHz for sub-6 GHz, 30/60 kHz around 7 GHz) to maintain early feasibility.

5G NR air interface to be used as a baseline for evaluations of proposed enhancements to demod framework.

Channel Models

Legacy TDL channel models lack spatial richness and realism for large-antenna and multi-link operation. Those should not be used for MIMO use cases, but can be referred to for 5G-6G comparison. While simplicity of TDL may be attractive for SISO cases, we believe that fragmentation would just cost the group in terms of time that could have been used to focus on use cases that matter to end customers and monetisation.

Building on Rel-19 SCM outcomes, RAN4 should adopt rCDL as the default propagation model for 6G performance requirement work, with extensions for higher frequency ranges where frequency-dependent cluster or angular characteristics influence DUT behaviour.

Use rCDL as baseline channel model for MIMO performance requirements in 6G.

Use TDL models only for comparison of performance cases between 5G and 6G within the study.

Additional channel types (e.g. NTN, ISAC) may be justified through specific study contributions that demonstrate necessity and feasibility. We believe that AI/ML-based receiver evaluations could be naturally supported through a physically robust channel models. In other words, we believe that expert’s effort in the group should be focussed on creation of a unified model that is sufficiently robust in terms of physical characteristics (time, frequency domains, correlation, temporal behaviour, time evolution)

Avoid creation of use-case specific channel models, rather focus on robustness of a single accepted model.

Receiver Assumptions

Baseline receiver definitions in previous generations were often overly conservative, limiting representativeness of field performance. Modern commercial devices commonly employ advanced detectors such as MMSE-IRC and R-ML. 6G demodulation requirements should therefore be referencing such state-of-the-art receivers. This does not mean that simple detection solutions cannot be used – it means that we need to have requirements for the state of technology that is representative of the modern commercial technology.

Define MMSE-IRC and R-ML as baseline receiver assumptions for UE demodulation studies.

Interference Modelling

Field deployments exhibit complex interference patterns where multi-TRP and inter-cell interactions are involved. Not having test cases for such use cases may result in unnecessary risks to network performance, as has been observed in 5G. We believe 6G demodulation requirements should explicitly address interference from both intra- and inter-cell sources, considering realistic network configurations and spatial domain characteristics.

RAN4 to study downlink and uplink interference profiles for intra-cell and inter-cell scenarios, ensuring correct representation of multi-TRP and heterogeneous deployments.

Performance Testing and Requirement Definition

Many current test methodologies use indirect metrics such as throughput ratios (such as gamma-values for PMI reporting), which adds a layer of ambiguity that may not correlate with field outcomes. Simplifying these to direct throughput/SNR or BLER-based criteria will improve traceability and test efficiency. This approach aligns requirement metrics with observable user experience while maintaining statistical robustness.

Simplify PMI reporting tests by adopting throughput/SNR-based criteria instead of γ-value measurement in the 6G demodulation framework.

New TE Functionalities

Modern demodulation validation must reflect adaptive properties of RAN operation. To that end, Outer-Loop Link Adaptation (OLLA) and CSI/SRS-based precoding represent use cases where introducing network-like feedback mechanisms into test equipment behaviour would be beneficial to validate 6G performance. Including these mechanisms early in the study phase will ensure coherent evolution of performance metrics and testing feasibility.

RAN4 demod to include at least studies on OLLA with link adaptation and CSI/SRS-based precoding as part of the 6G demodulation SI to enable realistic dynamic test environments.

Conclusion

This contribution makes the following proposals:

- adopt a realistic, phased study schedule that begins with a review of current 5G NR air performance aspects, ensuring sufficient time for comprehensive technical discussions before pursuing enhancement agreements.

- 5G NR air interface to be used as a baseline for evaluations of proposed enhancements to demod framework.

- Use rCDL as baseline channel model for MIMO performance requirements in 6G.

- Use TDL models only for comparison of performance cases between 5G and 6G within the study.

- Avoid creation of use-case specific channel models, rather focus on robustness of a single accepted model.

- Define MMSE-IRC and R-ML as baseline receiver assumptions for UE demodulation studies.

- RAN4 to study downlink and uplink interference profiles for intra-cell and inter-cell scenarios, ensuring correct representation of multi-TRP and heterogeneous deployments.

- Simplify PMI reporting tests by adopting throughput/SNR-based criteria instead of γ-value measurement in the 6G demodulation framework.

- RAN4 demod to include at least studies on OLLA with link adaptation and CSI/SRS-based precoding as part of the 6G demodulation SI to enable realistic dynamic test environments.

References

- R4-2513348, On demodulation and performance requirements in 6G, Oct 2025

- R4-2514588, WF on [116bis][106] 6G Demod, Mediatek, Oct 2025