3GPP TSG-RAN WG4 Meeting #117	R4-2521899

Dallas, USA, Nov 17th – Oct 21st, 2025

Source:	Nokia

Title:	RRM core requirements and testing framework for AI/ML positioning accuracy enhancement

Agenda item:	6.11.5

Document for:	Discussion

Introduction

The work item on artificial intelligence (AI)/machine learning (ML) for NR air interface (NR_AIML_Air) approved at RAN#102, was revised at RAN#108 [1] to refine the normative work of the general AI/ML framework for air interface and to enable the recommended use cases as depicted in the TR for the preceding study [2].

In this paper, open issues on RRM core and performance requirements for UE-based positioning (Case 1) are discussed and corresponding proposals are made.

Discussion

The open issues related to Cases 1 are discussed in this section according to RAN4#115 WF [3].

Requirements for Case1

This section discusses the requirements for the use cases with a UE-side model.

The following agreements were reached for Case 1 at RAN4#115 [3].

Issue 3-1: Requirements for case 1 Agreement: - Discuss how to define a measurement delay requirement (reporting delay) - Do not define any positioning accuracy requirement

The following agreements were also reached for Case 1 at RAN4#116 [4].

Issue 3-1: Reporting Delay Requirements for case 1 Agreement: Take the framework of the existing reporting delay requirements - Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report - Check until next meeting on the number of samples needed for the measurements

In the RAN4#116bis meeting, an agreement was made on the CRs [5].

Issue 3-1: Requirements for case 1 To be clarified in the requirements: - Request location information message should refer to “case 1” positiniong - Reuse the legacy RSTD measurement delay for all the cases (with gap, w/o gap, PRS aggregation) - Beam sweeping factor for FR2 including sweeping reduction factor - Take the legacy values - Rx TEG: check on signaling, take the legacy(used in RSTD measurements) approach if it is signaled - Cover all RRC states CR structure: - Introduce new clause for AI/ML based positioning case 1 - Reference existing requirements wherever they are reused to avoid duplicating the same requirement Introduce requirements for each RRC state in the corresponding clauses: Draft for RRC idle: Nokia Draft for RRC connected: E/// Draft for RRC inactive: Nokia

We agree to reuse the legacy RSTD measurement delay for all cases: with measurement gap, without gap, and with PRS aggregation. In RRC_IDLE and RRC_INACTIVE states, gap configuration is not available. Therefore, the measurement period is not impacted by gap and can be reused from clause 4.5.2.4 or 4.5.2.5 for RRC_IDLE, and clause 5.6.2.5 or 5.6.2.6 for RRC_INACTIVE, depending on whether bandwidth aggregation is configured. These clauses define the measurement period as a function of PRS periodicity, UE Rx beam sweeping factor, Rx TEG scaling factor, and sample count. Therefore, the new subclauses under clauses 4.5 and 5.6 will reuse these legacy definitions and values. Additionally, the four exceptions to reporting delay due to RRC state transitions are captured identically to the legacy RSTD reporting requirements.

There are four exceptions defined in the legacy RSTD reporting delay clause that apply when deriving measurement delay from the measurement period. Since RAN4 agreed to reuse clause 4.5.2 for RRC_IDLE and clause 5.6.2 for RRC_INACTIVE, these exceptions are also applicable to AI/ML positioning case 1.

Reusing clauses 4.5.2 and 5.6.2 enables AI/ML positioning to inherit all legacy features including sample count and beam sweeping.

- The number of samples (N_sample) directly affects the measurement period and total positioning delay. Legacy specifications allow N_sample from 1 to 4, with 4 as default. For AI/ML positioning, the reporting delay includes an additional inference delay, which can increase the total reporting delay. This inference delay could arise from machine learning-related operations, such as inference, which may require non-negligible compute time highly depending on model complexity and UE capability. In RRC_IDLE and RRC_INACTIVE, where no gap is configurable, reducing N_sample to 1 or 2 can help minimize delay. Clause 5.6A.4 allows N_sample = 2 if signaled, and further reduction to 1 may be considered for optimization. Considering that AI/ML positioning may introduce longer reporting latency, reducing the number of samples could be a valid approach to mitigate delay. Therefore, RAN4 should discuss the selection of N_sample for AI/ML positioning and evaluate the trade-off between delay and positioning accuracy.

RAN4 to discuss whether to allow the full range of N_sample values (1 to 4), or to restrict N_sample to 1 or 2 for delay reduction in AI/ML positioning.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to AI/ML based positioning. In the paper, the following observations and proposals were made:

Case 1

- There are four exceptions defined in the legacy RSTD reporting delay clause that apply when deriving measurement delay from the measurement period. Since RAN4 agreed to reuse clause 4.5.2 for RRC_IDLE and clause 5.6.2 for RRC_INACTIVE, these exceptions are also applicable to AI/ML positioning case 1.

Reusing clauses 4.5.2 and 5.6.2 enables AI/ML positioning to inherit all legacy features including sample count and beam sweeping.

- RAN4 to discuss whether to allow the full range of N_sample values (1 to 4), or to restrict N_sample to 1 or 2 for delay reduction in AI/ML positioning.

- References

- RP-251186, Revised WID on AI/ML for NR air interface, RAN#108, Prague, Czech Republic, June 9-13, 2025

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm, RAN4#115

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm, RAN4#116

- R4-2514631, WF on AI/ML for air interface for [116][131], Qualcomm, RAN4#116bis