3GPP TSG RAN WG4 Meeting #117	 																R4-2521922

Dallas, USA, 17 Nov. 2025 – 21 Nov. 2025

Agenda Item:	6.17.2.2

Source: 	Ericsson

Title:	Discussion on CSI-RS event triggered reporting test configurations for Mobility Ph 4

Document for:	Discussion

# Introduction

In this contribution we discuss the event triggered reporting test configurations for MIMO Ph 5.

# Discussion

## Event-driven beam management

Test configuration:

For the event-triggered reporting delay requirements, the end point of the measurement period is defined as the first UL transmission, as per the core requirements listed below. According to RAN2, the UE may transmit the first UL either by sending an SR, a special purpose SR, or by using an existing UL grant.

The event triggered measurement reporting delay is defined as the time between an event that will trigger an event triggered measurement report and the point when the UE starts to transmit the first UL transmission to report the measurement results over the air interface. This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

In the test configuration, using SR as the end point is not verifiable, since the UE can send the SR at any arbitrary time, and the test equipment (TE) cannot determine when the measurement period has actually ended. While using a special purpose SR could be an alternative, but not all UEs may support it. To ensure consistent testing across all UEs, we propose scheduling a DCI with UL grant in every slot once the event is triggered.

- RAN4 to agree that TE sends DCI in every slot after the event condition is changed in the test.

The test procedure: As we discussed in the last meeting, we suggest specifying the event triggered and periodic reporting as two separate tests (test 1 and test 2) in the same test case. The UE supporting both has to pass the event triggered test case and UE supporting only one capability, UE need to pass the test as per its capability.

In both tests, LTM candidate configured at the start of T1 but for the event triggered reporting, there is NO report is expected till the T2 is started.

The test procedure is divided into two parts:

Test 1: Periodic Reporting -> it can follow Rel-18 LTM procedure

Test 2: Event-Triggered Reporting

- UE reports CSI-RS-based L1-RSRP only when the event condition is met, using MAC CE.

- Similar to Test 1, it spans T1 and T2 with different CSI-RS_RP values for Cell 2.

- Only one report is expected after T2 starts, assuming the event condition is met and event triggered periodic reporting is not configured.

- Configuration prior to T1 is similar to Test 1, except reporting is event-triggered.

Test metric:

For the CSI-RS based L1-RSRP without SSB based L1-RSRP, in the test, event conditions are met at the start of T2. However, since the UE may need time to detect the event, it is assumed that the measurement is performed after T2 begins. Considering the beam sweeping factor, the UE may realize the event condition after completing a full sweep, which could take up to 8 × 20 ms = 160 ms. Given that DCI is scheduled every slot, the UE is expected to report the event-triggered L1-RSRP within 160 ms from the start of T2. There is NO beam sweeping time of SSB is considered in the event triggered reporting delay of CSI-RS based L1-RSRP.

- For the CSI-RS based L1-RSRP with skipping SSB based L1-RSRP, the UE is expected to report the event-triggered L1-RSRP within 160 ms from the start of T2

# Summary

The following have been proposed in this contribution.

- RAN4 to agree that TE sends DCI in every slot after the event condition is changed in the test.

- For the event triggered CSI-RS based L1-RSRP by skipping SSB based L1-RSRP, the UE is expected to report the event-triggered L1-RSRP within 160 ms (assuming CSI-RS periodicity of 20ms) from the start of T2.

# References

[1] 		R4-2508271, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

[2]		R4-2507014, On RRM performance aspects and test cases for MIMO Ph 5, Ericsson

[3]		R4-250xxxx, Discussion on CJTC reporting and asymmetric UL TRP requirements, Ericsson

[4]		R4-2317371