3GPP TSG-RAN WG4 Meeting #117	R4-2521980

# Dallas, USA, 17th Nov 2025 – 21st Nov 2025

Agenda item:	10.7

Source: 	Viasat, Thuraya

Title: 	Missing ETSI emission requirements for IoT NTN bands 253 and 255 and NR NTN band n255

Document for:	Approval

## 1.	Introduction

During the RAN4#116 meeting, as part of the combined L-Band WI [1], it was agreed to capture the two sets of emissions to address the existing as well as ongoing work on the ETSI harmonized standards [2].

A discussion paper [3] along with draft CRs [4] were submitted during the RAN4#116bis meeting for other previously specified NTN bands (n255 and bands 253/255) for which the same ETSI harmonized standards are applicable. During the meeting, the discussion paper was briefly presented and discussed with companies agreeing to evaluate the feasibility and backwards compatibility issues of specifying new NS values from Rel-17 onwards in support of the ETSI requirements for the L-Band frequency ranges. The draft CRs received some comments and were noted.

Based on the prior submissions and current 3GPP specifications, this paper expands on operators’ views [3] on updating the specification from Rel-17 onwards for NTN bands n255, b255 and b253.

## 2.	Background on treatment of L-Band ETSI requirements in 3GPP RAN4

Treatment of ETSI requirements for IoT NTN band has undergone an interesting journey over the past three years.

In prior RAN4 meetings (2023), as part of deliberations related to NTN bands b255/n255, there were submissions and discussions related to existing ETSI harmonised standard related to L-Band frequency ranges. An LS was sent to ETSI inquiring about the applicability of the requirements for 3GPP based earth stations operating in L-Band frequencies.

At the RAN4#109 meeting, measurements for band 255 were submitted in [5]. Measurements showed that the ETSI in-band and out of band emissions requirements can be met by Cat-M1 band 255 UEs. They also showed that ETSI in-band emissions requirements can be met by NB1/NB2 band 255 UEs. However, the out of band emissions requirements for both one-tone and twelve-tone transmissions over an NB-IoT channel may require a guard band of up to 100 kHz to operate without additional power backoff.

In RAN4#114, it was determined that the ITU-R M.1480 unwanted emissions requirements apply globally [6]. As a result, additional out of band/spurious frequencies ranges were included in Table 6.4A.4.3-1 of TS 36.102 [10] (duplicated in Annex D) and a guard band of 190 kHz was mandated from the NB-IoT RF channel edge and the lower limit of band 255 in Clause 5.2B. However, the update does not clarify why unwanted emissions above 1660.5 MHz specified in Table 2b of ITU-R M.1480 and the in-band emissions in Clause 3.2 of M.1480 are not required to be met. Furthermore, similar updates were not introduced for NR NTN band n255.

The ITU-R M.1480 unwanted emissions requirements are comparable to ETSI unwanted emission requirements. If 3GPP has concluded that M.1480 applies globally, then even if ETSI adopts a new harmonised standard in lieu of EN 301 681, it is possible that ITU-R M.1480 requirements would still have to be met by bands b251/n251 and b255/n255.

In RAN4#116, an LS was received from ETSI TC SES [7], providing progress update on further refinement of the harmonised standards related to the mobile earth station operation in the L-band. As a result, during RAN4#116, for n250, n251 and n253, it was agreed to include two sets of emissions requirements and corresponding NS flags and A-MPR values [2]. It was also agreed that the requirements in ETSI EN 301 681 [8] are still in force and are applicable to 3GPP spectrum bands specified in the L-band spectrum ranges listed in [1]. Furthermore, it was also agreed that once the new ETSI standards are ratified, the emission requirements in the 3GPP specifications may be updated to reflect that the newer ETSI standard what would be in force.

Observation 1: Two sets of requirements have been specified for NR NTN bands n250, n251 and n253 (Annex B) – set 1 based on current ETSI requirements in EN 301 681 [4, Annex A] and set 2 based on requirements likely to be included in the new ETSI standard inferred from [7].

Observation 2: RAN4 has agreed that given that the new ETSI standard has not been completed, the emissions requirements and the NS flags and A-MPR values associated with set 1 requirements (current ETSI requirements in EN 301 681 [8]) are in force.

The uplink frequency range of the existing L-bands (NR NTN band - n255, IoT NTN bands - 253/255) specified as part of 3GPP releases 17 and 18 are identical to the new combined L-bands n250 and n251. Therefore, the same ETSI emission requirements are also applicable to those IoT and NR NTN 3GPP bands.

| IoT/NR NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| b255/ n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| b253/n253 | 1668 MHz – 1675 MHz | 1518 MHz – 1525 MHz | FDD |
| b251/n251 | 1626.5 MHz – 1660.5 MHz | 1518 MHz – 1559 MHz | FDD |
| b250/n250 | 1668 MHz – 1675 MHz | 1518 MHz – 1559 MHz | FDD |

Observation 3: The ETSI emissions requirements specified for bands n251 also apply to NR NTN band n255 but have not been incorporated in TS 38.101-5 [9].

Observation 4: The ETSI emissions requirements specified for bands n251 and n253 also apply to IoT NTN bands 255 and 253 respectively but have not been incorporated in TS 36.102 [10].

## 3.	Addressing ETSI requirements for IoT NTN band 253 (same uplink frequency range as band n250)

There are currently no UEs in the market that support IoT NTN band 253. Therefore, there is no backwards compatibility issue/impact if Release 18 specifications were updated to capture the ETSI emissions requirement set and any associated power back-off required to meet those requirements.

Observation 5: Operators are unaware of any commercial UEs that support IoT NTN band 253.

Evaluating the unwanted emissions specified for band 253 (Annex B-1), unlike band 255, the out of band emissions requirements within 2 MHz from the band edge is same as the in-band emission requirements for band 255. Measurements in [5] (duplicated in Annex C) show that there is adequate margin to the emissions requirements and no guard band or additional power backoff is required for PC3 M1, NB1 and NB2 NTN band 253 UEs. Therefore, the emission requirements in TS 36.102 be updated from Release-18 specification onwards as proposed in R4-2521981 and its associated Cat-A CR.

Observation 6: Relative measurements indicate there is adequate margins to meet the ETSI requirements for NTN band 253 PC3 M1, NB1, NB2 UEs without the need for specifying additional guard band or power backoff.

Proposal 1: Update the IoT NTN band 253 specifications in TS 36.102 to capture the two sets of requirements and associated NS flags NS_09N and NS_13N for PC3 IoT NTN UE from Rel-18 specifications onwards as proposed in R4-2521981 and its associated Cat-A CR.

## 4.	Addressing ETSI requirements for NR NTN band n255 (same uplink frequency range as band n251)

Like IoT NTN band 253, there are currently no UEs in the market that support NR NTN band n255. Therefore, there is no backwards compatibility issue/impact if the specifications were updated from Release-17 onwards as submitted in t-doc numbers R4-2521983 and its associated Cat-A CRs.

Observation 7: Operators are unaware of any commercial UE that support NR NTN band n255.

Proposal 2: Update the NR NTN band n255 specifications in TS 38.101-5 to capture the two sets of requirements and corresponding NS flags NS_10N and NS_14N along with the A-MPR identified for n251 for PC3 NR NTN UE from Rel-17 specifications onwards as proposed in R4-2521983 and its associated Cat-A CRs.

Furthermore, as part of the HPUE NTN UE Rel-19 work, RAN4 has specified PC2/PC1 NR NTN UE for n255 [9]. However, since the ETSI emissions requirements were not captured and the NS flags associated with those requirements not specified, and A-MPR evaluations for these additional requirements were also not performed.

Observation 8: Unlike for PC3, the A-MPR assessment for PC2 and PC1 n255 NR NTN UEs is unavailable.

Given that the HPUE work was recently completed (September 2025), there are no known PC2 and PC1 n255 commercial UEs. It is therefore proposed that the ETSI requirement sets for n255 PC2 and PC1 UEs be specified in Release 19 along with any A-MPR that is necessary to meet the requirements. Since the A-MPR assessment is unavailable, a note can be captured to indicate that the A-MPR for PC2 and PC1 n255 UEs for NS_10N and NS_14N are not supported as proposed in R4-2521988. Companies are further encouraged to present A-MPR evaluations at the RAN4#118 meeting with the objective of capturing the A-MPR requirements in TS 38.101-5 during that meeting.

Observation 9: There are no PC2 or PC1 n255 commercial UEs.

Proposal 3: Update note 5 in Table 6.2.3.1-1 as proposed in R4-2521988.

Proposal 4: Companies are encouraged to submit A-MPR evaluations for NS_10N and NS_14N for PC2 and PC1 UEs at the RAN4#118 meeting with the objective of finalizing the CR for Release 19 TS 38.101-5 during that meeting.

## 5.	Addressing ETSI requirements for IoT NTN band 255 (similar to n250)

Unlike NTN bands 253 and n255, there are Release 17 compliant NB-IoT NTN band 255 PC3 UEs that are commercially available. However as discussed above in clause 2, measurements show that they are likely able to comply with the ETSI in-band requirements without additional power backoff. It is quite possible that the out of band emissions requirements are being met through a combination of the 190 kHz guard band specified in TS 36.102 for band 255 and leveraging the nominated bandwidth concept of the ETSI standard.

In fact, the out of band ETSI requirements near the lower edge of the band are the same as those specified for band 255 in Table 6.5A.4.3-1 [10] (duplicated in Annex D). Given that no concerns were raised in meeting these requirements for band 255 during the Release-19 HPUE deliberations, it stands to reason that the same ETSI out of band requirements can be met by band 255 PC2/PC1 NB-IoT UEs with the existing provision of 190 kHz guard band.

With respect to ETSI in-band emission limits, except for a very small region of the spectrum, 3GPP SEM is more stringent than the ETSI requirements. Within that small region of spectrum, measurement show there is adequate margin and seems to be adequate to not require additional power backoff for PC3 band 255 IoT NTN UEs.

Observation 10: There are Release 17 compliant NB-IoT UEs operating in NTN band 255.

Observation 11: PC3 IoT NTN UEs are likely able to comply with the current ETSI emissions requirements without additional power backoff through a combination of the 190 kHz guard band specified in TS 36.102 for band 255, inherent design margins available and use the nominated bandwidth concept of the ETSI standard.

Observation 12: Based on measurements in [5] (Annex C) and considering the guard band of 190 kHz at the lower limit of band 255, there seems to be adequate margin to the ETSI requirements, and no additional power backoff is required for NS_10N and NS_14N for PC3 IoT NTN band 255 UEs.

TS 36.331 describes the behaviour of a UE when multiple NS values are signalled. Per TS 36.331, the UE will apply the 1st NS value from the list broadcast in SIB1 and ensure that the emissions associated with that NS value are met.

By prioritising broadcast of new NS value over the older NS value, newer UE implementations will be able to select the cell and comply with the ETSI requirements. The legacy UEs that do not recognise the newer NS value will continue to step through the NS list and utilize the default NS value, NS_01 and select the cell without getting barred. Therefore, incorporating NS values NS_10N and NS_14N should not cause any backwards computability issues.

Observation 13: There do not seem to be any backwards compatibility issues if the newer NS values are signalled at the top of the NS value list broadcast in SIB1 and the channel is configured at least 190 kHz away from the lower limit of the band edge. The legacy PC3 UEs that do not recognize the new NS values will continue to use the NS_01 included in SIB1. PC3 UEs that recognize the NS value (NS_10N/NS_14N) will use the new NS value and comply with the ETSI requirement.

Proposal 5: Update the IoT NTN band 255 specifications in TS 36.102 to capture the two sets of requirements and associated NS flags (NS_10N and NS_14N) for PC3 IoT NTN UE from Rel-18 specifications onwards as proposed in R4-2521986 and associated Cat-A CRs.

Furthermore, as part of the HPUE NTN UE Rel-19 work, RAN4 has specified PC2 UEs for Cat-M1 and NB1/NB2 NTN band 255 UEs and PC1 NB1/NB2 NTN band 255 UEs in [10]. Since the ETSI emissions requirements were not captured and the NS values, NS_10N/NS_14N, associated with those requirements not specified, A-MPR evaluations for these additional requirements were not performed.

Observation 14: The A-MPR assessment for PC2 and PC1 band 255 IoT NTN UEs were performed without considering the ETSI requirement sets.

During Release-19 HPUE work, no A-MPR was specified to meet the out of band requirements for PC2/PC1 band 255 IoT NTN UEs to meet the M.1480 requirements. Since the ETSI out of band requirements are comparable to ITU-R M.1480 out of band requirements which have been captured in the specifications, one can conclude that the ETSI out of band requirements can be met by PC2/PC1 band 255 IoT NTN UEs.

Observation 15: It was concluded that no additional A-MPR is required for band 255 PC2 Cat-M1, NB1, NB2 NTN UEs or PC1 NB-NTN UE to meet ITU-R M.1480 out of band requirements. It can therefore be concluded that no additional A-MPR is required to meet ETSI out of band requirements.

With respect to ETSI in-band emission limits, except for a very small region, 3GPP SEM is more stringent than the ETSI requirements. For the small region where the ETSI limits are more stringent, higher MPR of PC2 and PC1 and available margin are likely adequate to not require additional A-MPR. Moreover, UE manufacturers will be able to get devices ETSI certified without additional power backoff through the use of nominated bandwidth, allowed to be used for showing compliance to the ETSI in-band emissions requirements.

Observation 16: Higher MPR and available margin based on measurement are likely adequate to meet ETSI in-band emission requirements and not require additional power backoff for band 255 PC2 Cat-M1, NB1, NB2 NTN UEs or PC1 NB-NTN UE.

Observation 17: Manufacturers will be able to leverage use of nominated bandwidth without requiring additional power backoff to meet the in-band emissions requirements for band 255 PC2 Cat-M1, NB1, NB2 NTN UEs or PC1 NB-NTN UE.

Therefore, it is proposed to update the A-MPR tables with N/A for NS_10N and NS_14N for PC2 and PC1 band 255 IoT NTN UEs.

Proposal 6: Update the IoT NTN band 255 specifications for PC2 and PC1 UEs in TS 36.102 as proposed in R4-2522010.

## 3.	Conclusion

Given the observations below,

Observation 1: Two sets of requirements have been specified for NR NTN bands n250, n251 and n253 (Annex B) – set 1 based on current ETSI requirements in EN 301 681 [4, Annex A] and set 2 based on requirements likely to be included in the new ETSI standard inferred from [7].

Observation 2: RAN4 has agreed that given that the new ETSI standard has not been completed, the emissions requirements and the NS flags and A-MPR values associated with set 1 requirements (current ETSI requirements in EN 301 681 [8]) are in force.

Observation 3: The ETSI emissions requirements specified for bands n251 also apply to NR NTN band n255 but have not been incorporated in TS 38.101-5 [9].

Observation 4: The ETSI emissions requirements specified for bands n251 and n253 also apply to IoT NTN bands 255 and 253 respectively but have not been incorporated in TS 36.102 [10].

Observation 5: Operators are unaware of any commercial UEs that support IoT NTN band 253.

Observation 6: Relative measurements indicate there is adequate margins to meet the ETSI requirements for NTN band 253 PC3 M1, NB1, NB2 UEs without the need for specifying additional guard band or power backoff.

Observation 7: Operators are unaware of any commercial UE that support NR NTN band n255.

Observation 8: Unlike for PC3, the A-MPR assessment for PC2 and PC1 n255 NR NTN UEs is unavailable.

Observation 9: There are no PC2 or PC1 n255 commercial UEs.

Observation 10: There are Release 17 compliant NB-IoT UEs operating in NTN band 255.

Observation 11: PC3 IoT NTN UEs are likely able to comply with the current ETSI emissions requirements without additional power backoff through a combination of the 190 kHz guard band specified in TS 36.102 for band 255, inherent design margins available and use the nominated bandwidth concept of the ETSI standard.

Observation 12: Based on measurements in [5] (Annex C) and considering the guard band of 190 kHz at the lower limit of band 255, there seems to be adequate margin to the ETSI requirements, and no additional power backoff is required for NS_10N and NS_14N for PC3 IoT NTN band 255 UEs.

Observation 13: There do not seem to be any backwards compatibility issues if the newer NS values are signalled at the top of the NS value list broadcast in SIB1 and the channel is configured at least 190 kHz away from the lower limit of the band edge. The legacy PC3 UEs that do not recognize the new NS values will continue to use the NS_01 included in SIB1. PC3 UEs that recognize the NS value (NS_10N/NS_14N) will use the new NS value and comply with the ETSI requirement.

Observation 14: The A-MPR assessment for PC2 and PC1 band 255 IoT NTN UEs were performed without considering the ETSI requirement sets.

Observation 15: It was concluded that no additional A-MPR is required for band 255 PC2 Cat-M1, NB1, NB2 NTN UEs or PC1 NB-NTN UE to meet ITU-R M.1480 out of band requirements. It can therefore be concluded that no additional A-MPR is required to meet ETSI out of band requirements.

Observation 16: Higher MPR and available margin based on measurement are likely adequate to meet ETSI in-band emission requirements and not require additional power backoff for band 255 PC2 Cat-M1, NB1, NB2 NTN UEs or PC1 NB-NTN UE.

Observation 17: Manufacturers will be able to leverage use of nominated bandwidth without requiring additional power backoff to meet the in-band emissions requirements for band 255 PC2 Cat-M1, NB1, NB2 NTN UEs or PC1 NB-NTN UE.

the following are proposed:

Proposal 1: Update the IoT NTN band 253 specifications in TS 36.102 to capture the two sets of requirements and associated NS flags NS_09N and NS_13N for PC3 IoT NTN UE from Rel-18 specifications onwards as proposed in R4-2521981 and its associated Cat-A CR.

Proposal 2: Update the NR NTN band n255 specifications in TS 38.101-5 to capture the two sets of requirements and corresponding NS flags NS_10N and NS_14N along with the A-MPR identified for n251 for PC3 NR NTN UE from Rel-17 specifications onwards as proposed in R4-2521983 and its associated Cat-A CRs.

Proposal 3: Update note 5 in Table 6.2.3.1-1 as proposed in R4-2521988.

Proposal 4: Companies are encouraged to submit A-MPR evaluations for NS_10N and NS_14N for PC2 and PC1 UEs at the RAN4#118 meeting with the objective of finalizing the CR for Release 19 TS 38.101-5 during that meeting.

Proposal 5: Update the IoT NTN band 255 specifications in TS 36.102 to capture the two sets of requirements and associated NS flags (NS_10N and NS_14N) for PC3 IoT NTN UE from Rel-18 specifications onwards as proposed in R4-2521986 and associated Cat-A CRs.

Proposal 6: Update the IoT NTN band 255 specifications for PC2 and PC1 UEs in TS 36.102 as proposed in R4-2522010.

## 4.		References

[1]	RP-241689, "Revised WID on New NR NTN bands to support Extended L-band and combined MSS L-band and Extended L-band ranges ", Inmarsat, Viasat

[2]	R4-2511861, CR to TS 38.101-5: Introduction of combined L-bands, Apple, et. al.

[3]	Missing ETSI emission requirements for IoT NTN bands 253 and 255 and NR NTN band n255, Viasat, et. al.

[4]	R4-2513842/3843/3844/3858/3859, Draft CRs by Viasat, et. al.

[5]	R4-2318360, Maintenance on IoT NTN UE RF – ETSI issue, Sony

[6]	R4-2502991, CR to TS 36.102: B255 emissions, Qualcomm

[7]	R4-2509023, LS on Harmonised Standard for NTN capable UE, ETSI TC SES

[8]	ETSI EN 301 681, Harmonised Standard for Mobile Earth Stations (MES) ofGeostationary mobile satellite systems, including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) under the Mobile Satellite Service (MSS), operating in the 1,5 GHz and 1,6 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU

[9]	TS 38.101-5, ver 19.2.0

[10]	TS 36.102, ver 19.1.0

- Annex A: Current ETSI requirements for L-bands [4]

Table A-1: In-band emission requirements for a NTN UE with UL operation in 1626.5-1660.5MHz frequency range

Table A-2: In-band emission requirements for a NTN with UL operation in 1668-1675MHz frequency range

Table A-3: Out-of-band emission requirements for a NTN UE with UL operation in 1626.5-1660.5MHz frequency range

Table A-4: Out-of-band emission requirements for a NTN UE with UL operation in 1668-1675MHz frequency range

- Annex B: RAN4 specifications for n250 and n251 [5]

## B-1: Two NS Values for the two sets of emission requirements for two L-band uplink frequency ranges: 1626.5 to 1660.5 MHz (NS_10N, NS_14N) and 1668 to 1675 MHz (NS_09N, NS_13N)

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_09N | 6.5.2.3.3 6.5.3.3.8 | n250, n253 | 5 |  | Clause 6.2.3.7 |
| NS_10N | 6.5.2.3.3 6.5.3.3.9 | n251 | 5, 10, 15, 20 |  | Clause 6.2.3.8 |
| NS_13N5 | 6.5.3.3.10 |  | 5 |  |  |
| NS_14N5 | 6.5.3.3.10 |  | 5, 10, 15, 20 |  | Clause 6.2.3.9 |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. NOTE 4: The NS_07N and NS_08N is signalled based on coordination between operators and subject to regional/national regulation NOTE 5: NS_09N (n253, n250), NS_10N (n251) are currently in force. NS_13N (n253, n250), NS_14N (n251) may supersede NS_09N, NS_10N when the new ETSI requirements get ratified. |  |  |  |  |  |

## B-2: Emission requirements for two L-band uplink frequency ranges: 1626.5 to 1660.5 MHz and 1668 to 1675 MHz

##### 6.5.2.3.3	Requirements for network signalling value “NS_09N" and “NS_10N"

When "NS_09N" or "NS_10N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3-1 for any channel bandwidth configured within the corresponding band.

Table 6.5.2.3.3-1: Additional requirements for "NS_09N" and "NS_10N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Note (measurement method) |
| --- | --- | --- | --- |
| ± 0-25 | 30 to 15 | 3 kHz | Average |
| ± 25-125 | 15 to -20 |  |  |
| ± 125-425 | -20 |  |  |
| ± 425-1500 | -20 to -35 |  |  |
| ± 1500-36000 | -25 | 30 kHz |  |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

##### 6.5.3.3.8	Requirement for network signalling value "NS_09N"

When "NS_09N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.8-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.8-1: Additional out-of-band requirements for "NS_09N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1666.5 | -25 | 30 kHz | Average |
| 1666.5 to 1667.575 | -20 to -35 | 3 kHz | Average |
| 1667.575 to 1667.875 | -20 | 3 kHz | Average |
| 1667.875 to 1667.975 | 15 to - 20 | 3 kHz | Average |
| 1667.975 to 1668 | 30 to 15 | 3 kHz | Average |
| 1675 to 1675.025 | 30 to 15 | 3 kHz | Average |
| 1675.025 to 1675.125 | 15 to - 20 | 3 kHz | Average |
| 1675.125 to 1675.425 | -20 | 3 kHz | Average |
| 1675.425 to 1676.5 | -20 to -35 | 3 kHz | Average |
| 1676.5 to 1677 | -25 | 30 kHz | Average |
| 1677 to 1680 | -30 | 30 kHz | Average |
| 1680 to 1685 | -30 | 100 kHz | Average |
| 1685 to 1695 | -30 | 300 kHz | Average |
| 1695 to 1705 | -30 | 1 MHz | Average |
| 1705 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

##### 6.5.3.3.9	Requirement for network signalling value "NS_10N"

When "NS_10N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.9-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.9-1: Additional out-of-band requirements for "NS_10N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1660.525 | 30 to 15 | 3 kHz | Average |
| 1660.525 to 1660.625 | 15 to - 20 | 3 kHz | Average |
| 1660.625 to 1660.925 | -20 | 3 kHz | Average |
| 1660.925 to 1662 | -20 to -35 | 3 kHz | Average |
| 1662 to 1662.5 | -25 | 30 kHz | Average |
| 1662.5 to 1665.5 | -30 | 30 kHz | Average |
| 1665.5 to 1670.5 | -30 | 100 kHz | Average |
| 1670.5 to 1680.5 | -30 | 300 kHz | Average |
| 1680.5 to 1690.5 | -30 | 1 MHz | Average |
| 1690.5 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

##### 6.5.3.3.10	Requirement for network signalling value "NS_14N"

When "NS_14" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.10-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.10-1: Additional out-of-band requirements for "NS_14N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

- Annex C: Evaluating Cat-M1 and NB1 UE measurements against ETSI requirements in EN 301 681 [5]

(a)

(b)

Figure C-1: The comparison of measured emission mask for an actual cat-M1 device vs. 3GPP SEM and ETSI (a) IB and (b) OOB emission limit.

(a)

(b)

Figure C-2: The comparison of measured emission mask for an actual NB-IoT device vs. 3GPP SEM and ETSI (a) IB and (b) OOB emission limit.

- Annex D: Emission Limits Specified for IoT NTN band 255 [10]

Table 6.5A.4.3-1: Requirements for spurious emissions for UE co-existence

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 253 | E-UTRA Band 5, 26, 31, 41, 48, 72 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n65, n67, n74, n75, n76, n79, n91, n92, n93, n94, n105, n109 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| 254 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 31, 41, 48, 54, 66, 70, 71, 72, 85, 87, 88, 103 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n53, n65, n67, n74, n75, n76, n77, n78, n90, n91, n92, n93, n94, n105, n106, n109 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| 255 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 31, 41, 48, 66, 70, 71, 72, 85, 103 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n53, n65, n67, n74, n75, n76, n90, n91, n92, n93, n94, n105, n106, n109 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1621.5 |  | - | 1624.5 | -30 | 0.03 | 5 |
|  | Frequency range | 1624.5 |  | - | 1625 | -30 to -27.5 | 0.03 | 4, 5 |
|  | Frequency range | 1625 |  | - | 1625.125 | -27.5 to -27.2 | 0.03 | 4, 5 |
|  | Frequency range | 1625.125 |  | - | 1625.8 | -27.2 to -20 | 0.03 | 4, 5 |
|  | Frequency range | 1625.8 |  | - | 1626 | -20 to -17 | 0.03 | 4, 5 |
|  | Frequency range | 1626 |  | - | 1626.2 | -17 to -10 | 0.03 | 4, 5 |
|  | Frequency range | 1626.2 |  | - | 1626.5 | -10 | 0.03 | 5 |
| 256 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 33, 32, 35, 38, 40, 41, 42, 43, 50, 51, 54, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n12, n13, n14, n24, n29, n30, n39, n48, n53, n66, n67, n71, n78, n79, n85, n90, n91, n92, n93, n94, n101, n105, n106, n109 |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n2, n25, n70 |  | FDL_low | - | FDL_high | NA | NA | 3 |
| 252 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 33, 32, 35, 38, 40, 41, 42, 43, 50, 51, 54, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n12, n13, n14, n24, n29, n30, n39, n48, n53, n66, n67, n68, n71, n78, n79, n85, n87, n88, n90, n91, n92, n93, n94, n101, n105, n106, n109, n110 |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.4A.2-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5A.4.2-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: The co-existence between 256 and band 2, 25 and 70 is subject to regional/national regulation. NOTE 4: Linearly interpolated in dBm vs. frequency. NOTE 5: Requirement is a regional regulatory requirement. |  |  |  |  |  |  |  |  |