3GPP TSG-RAN WG4 Meeting #117                                        R4-2521984

# Dallas, TX, USA, November 17-21, 2025

Title:	Qualcomm views on 6G UE RF spec

Source:	Qualcomm Incorporated

Agenda item:	8.12.2.1

Release:	Rel-20

SI: 	FS_6G_Radio

Document for:	Approval

# Introduction

In this paper we discuss two aspects regarding UE RF requirements that could be improved. First we discuss the presence of test modes and environmental conditions in cure requirements and secondly we discuss feature agnostic approach to requirements i.e. how to writ requirements so that they can be valid regardless of presence of other features.

# 2	Discussion

## 2.1	Test modes and environmental conditions

Test-related functions (‘test modes’) exist to facilitate testing in some way (RAN5 domain). UE core requirements (RAN4 domain) on the other hand are meant to establish design goals for field performance for UEs.

Observation 2.1-1: UE RF core requirements are design goals for the UE and represent a minimum performance expectation that can be used for network design.

In keeping with intent, RAN4 core requirements should be defined without limitation on environmental conditions beyond what is expected for operation, and without reliance on test modes. Test mode definition can be left to RAN5.

Proposal 2.1-1:  Assume in the study item that evaluations are applicable for all environmental conditions with no test mode applied.

## 2.2	Feature orthogonality

In this section we discuss about a problem of orthogonality of the requirements between two features. The issue can be described with a question: When requirements for UL MIMO and Carrier Aggregation were defined independently of each other, what requirements apply when UE is configured for UL MIMO and CA simultaneously? For 5G both were defined in Rel-15 and it could have been assumed that for combined operation of these features the possible relaxation or dependencies between operation that might be needed in specifications would have been brought up during Rel-15. The section 4 of 38.101-1 implies this but is not perfectly clear since it states the relations between general requirement and additional requirement.

In the absence of a specific requirement for the combined operation, UE has to meet both requirements and Rel-15 UE can support the combination of CA with UL MIMO. This could be made clear for example by amending the clause 4 by sentence in the lines of:

“In case UE supports features where requirements are under more than one suffix providing additional requirements, UE has to meet general and all the additional requirements simultaneously. In case there are overlapping requirements, UE has to meet the tighter requirements.”

The problem RAN4 will face is that is also well explained by continuing the example case: Despite the UL MIMO and CA requirements both were in Rel-15,  For Rel-17 a WI objective was agreed for “CA with UL MIMO” and this then made the combination of CA and UL MIMO a Rel-17 feature. This is unique characteristics of RAN4. Other WGs typically define requirements for features at a time and intend to solve issues of the combined features at the time of introduction. It maybe so by design since RAN4 deals with the implementation challenges and problems may only come up in RAN4.

Observation 2.2-1: RAN4 is the WG that in many cases concludes requirements for combinations of features.

To avoid ambiguity in 6G era, RAN4 should discuss what combinations of features are discussed and confirmed to be feasible and study how the specifications could be written with better flexibility. This would also help with the discussions on WI prioritization that have reflections of roadmap discussion.

Proposal 2.2-1: RAN4 should study how to define requirements in such manner that dependencies are minimised or at least are unambiguously understood and specified when multiple features are simultaneously configured during the introduction of new features

# 3. 	Conclusion

Observation 2.1-1: UE RF core requirements are design goals for the UE and represent a minimum performance expectation that can be used for network design.

Proposal 2.1-1:  Assume in the study item that evaluations are applicable for all environmental conditions with no test mode applied

Observation 2.2-1: RAN4 is the WG that in many cases concludes requirements for combinations of features.

Proposal 2.2-1: RAN4 should study how to define requirements in such manner that dependencies are minimised or at least are unambiguously understood and specified when multiple features are simultaneously configured during the introduction of new features

# References

[1]	RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2]	TR 38.914, “Study on 6G Scenarios and Requirements”, v0.1.0, June 2025

[3]	R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple), 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

[4]	R4-2513020, “Qualcomm Views on System parameters”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13-17, 2025

[5]	R4-2513022, “Qualcomm views on 6G spectrum”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13-17, 2025

[6]	R4-157052, “On Introduction of Antenna Isolation for RF Tests”, Rohde & Schwarz, RAN4-77

[7]	R4-2514642, “WF for [116bis][102] 6G general RF and UE RF”, Feature lead (Qualcomm), 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, 13th – 17th October, 2025