3GPP TSG-RAN WG4 Meeting #117	R4-2521987

Dallas, US, November 17th – 21st, 2025

Source:	KDDI, Qualcomm, Samsung

Title:	TP to TR 38.719-03-01 for adding CA_n40-n41-n77

Agenda item:			5.3.4

Document for:	Approval

- Introduction

This contribution is a text proposal for TR 38.719-03-01 to include CA_ n40A-n41A-n77A as defined in WID [1].

Fallbacks are already specified in TS 38101-1[2].

- References

[1] RP-252440. Revised WID NR_CADC_SUL_R19, Ericsson

[2] 3GPP TS 38101-1 V19.3.1(2025-09)

Text Proposal

<Start of Text Proposal>

5.x	CA_ n40A-n41A-n77A

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.0.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | FDD |
| n41 | 2469 MHz – 2690 MHz | 2469 MHz – 2690 MHz | TDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n40A-n41A-n77A | CA_n40A-n41A CA_n40A-n77A CA_n41A-n77A | n40 | n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |

5.x.1.3	∆TIB,c and ∆RIB,c values

The ∆TIB,c and ∆RIB,c values are borrowed from CA_ n20-n41-n77 and given in the tables below.

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration |  |  |
| CA_n40-n41-n77 | 0.5 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration |  |  |
| CA_n40-n41-n77 | 0.2 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. |  |  |  |

5.x.2	Specific for 2 bands UL CA

5.x.2.1	UE co-existence studies

5.x.2.1.1	Co-existence studies for 2UL band with 1CC per band

For the three 2-band UL cases, IMDs exist:

-UL CA_n40A-n41A has IMD5, IMD6 and IMD7 into n77 downlink.

-UL CA_n40-n77A has IMD4, IMD6 and IMD7 into n41 downlink.

-UL CA_n41-n77A has IMD4, IMD6 and IMD7 into n40 downlink.

5.x.2.2	REFSENS requirements

Based on evaluation

-For UL CA_n40A-n41A with IMD5 into n77 downlink, the PC3 IMD5 MSD is 5.2dB.

-For UL CA_n40-n77A with IMD4 into n41 downlink, and the PC3 IMD4 MSD is 13.1dB

-For UL CA_n41-n77A with IMD4 into n40 downlink, and the PC3 IMD4 MSD is 15dB

.The test points and MSD values are shown in Table 5.x.2.2.

Table 5.x.2.2: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| n40-n41-n77 | n40 | 2305 | 10 | 50 | 2305 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3445 | 5.2 | TDD | IMD5X |
|  | n40 | 2305 | 10 | 50 | 2305 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2600 | 13.1 | TDD | IMD4 X |
|  | n77 | 3605 | 10 | 50 | 3605 | N/A | TDD | N/A |
|  | n40 | N/A | 10 | N/A | 2350 | 15 | TDD | IMD4 X |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | TDD | N/A |
|  | n77 | 3725 | 10 | 50 | 3725 | N/A | TDD | N/A |

NOTE X:	This band is subject to IMD6 also which MSD is not specified.

<End of Text Proposal>