3GPP TSG-RAN WG4 Meeting #117	R4-2521989

Dallas, US, November 17th – 21st, 2025

Source: 	KDDI, Murata, Samsung

Title:	TP to TR 38.746 for adding HPUE feature to CA_n18-n40 with BCS 4 and 5

Agenda item:	5.7.3

Document for:	Approval

- Introduction

This paper is a Text Proposal to TR 38.746 for adding HPUE UL notes to CA_ n18A-n40A with BCS 4 and 5.

The Draft-CR for the PC3 case for this 2-band combination was submitted to this meeting in R4-2521298[3].

The CA_n18-n40 configuration with uplink CA has been specified in TS 38.101-1 for Release 18. Although UL CA_n18-n40 causes IMD5(4,-1) interference in the victim band n18 under BCS0, the resulting IMD5 products do not fall within the downlink of n18. Therefore, in Release 18, RAN4 concluded that no MSD test point exists for this band configuration, and the MSD for PC3 is set to 0.

In the existing BCS0 configuration, the n40 channel bandwidth is limited to 10–50 MHz. Introducing BCS 4 and 5 to CA_n18-n40 adds a 5 MHz bandwidth to n40. Analysis shows that the IMD5 products fall at the edge of the band, and the IMD5 edge is 20 dB lower than the IMD5 center. As a result, the MSD for PC3 under BCS 4 and 5 in CA_n18-n40 can be considered negligible, similar to the BCS0 case.

Analysis indicates the following MSD values for PC2 and PC1.5 in DL CA_n18A-n40A with UL CA_n18A-n40A:

- 0.9 dB for PC2 Case a

- 1.6 dB for PC2 Case b

- 2.2 dB for PC1.5 Case b

These MSD values for PC2 and PC1.5 are relatively small. Therefore, in accordance with the approved WF R4-2507935, MSD requirements for CA_n18-n40 HPUE PC2 and PC1.5 do not need to be specified.

- Text proposal

< Start of Text Proposal>

5.X	CA_n18-n40

5.X.1	Configurations

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n40A | CA_n18A-n40A | n18 | 5, 10, 15 | 0 |
|  |  | n40 | 10, 15, 20, 30, 40 |  |
|  | n408,9 CA_n18A-n40A8,9 | n18 | n18 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink CA configuration according to clause 6.2A.1.1 or 6.2A.1.2 or 6.2A.1.3 or single uplink carrier configuration according to clauses 6.2.1 or 6.2D.1 or 6.2G.1 in this downlink/uplink combination. NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink CA configuration according to clause 6.2A.1.3 or single uplink carrier according to clauses 6.2.1 or 6.2D.1 or 6.2G.1 in this downlink/uplink combination. NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink CA configuration according to clause 6.2H.3.1 or 6.2L.3.1. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink CA configuration according to clause 6.2H.3.1 or 6.2L.3.1. |  |  |  |  |

5.X.2	Maximum output powers for inter-band CA

The UE Power class 2 and Power class 1.5 for uplink Configuration for CA_n18-n40 is as shown in Table 5.X.2-1 and Table 5.X.2-2 respectively.

Table 5.X.2-1: UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n18-n40 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

Table 5.X.2-2 UE Power Class 1.5 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 1.5 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n18-n40 | Case a | 29dBm | 26dBm | 26dBm |
|  | Case b | 29dBm | 23dBm | 29dBm |
|  | Case c | 29dBm | 29dBm | 23dBm |
|  | Case d | 29dBm | 26dBm | 29dBm |
|  | Case e | 29dBm | 29dBm | 26dBm |

5.X.3	REFSENS requirements.

Based on analysis,

- there is no IMD issue and no harmonic issue for UL n40 to DL CA_n18-n40.

- For UL CA_n18A-n40A with BCS 4 and 5, IMD5(4,-1) interference falls at the edge of n18 but not in n18 downlink, resulting in negligible MSD, similar to the BCS 0 case for PC3.Technical analysis indicates the MSD values for PC2 and PC1.5 can be regarded as small.  PC2 and PC1.5 in DL CA_n18A-n40A with UL CA_n18A-n40A:

Therefore, in accordance with the approved WF R4-2507935, MSD requirements for CA_n18-n40 HPUE PC2 and PC1.5 do not need to be specified.

5.X.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 case.

< End of Text Proposal>

- References

[1] 3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[2] 	RP-241679_New WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR SUL (supplementary uplink)

[3] R4-2521298 Draft CR to TS38.101-1 Rel-19 for adding BCS 4 and 5 into CA_n18-n40

[4] R4-2505143, WF on MSD simplification for HPUE

[5] R4-2514599, Draft CR for 38.101-1 on enabling HPUE support for band combinations without HPUE applicability notes in clause 5