3GPP TSG-RAN WG4 Meeting #117	R4-2522021

Dallas, USA,  17th – 21st November 2025

Agenda item:	10.2.2

Source:	Apple

Title:	On ACLR for band n104

WI/SI:	NR_6GHz-Core

Release:	Rel-17

Document for:	Discussion

# 1	Introduction

In previous meeting it has been proposed to reduce the MPR for n104 by relaxing the ACLR. A previous Release 17 study on IMT parameters for the 6.425-7.125 GHz range agreed to specify 26 dB ACLR instead of the current 30 dB for UE Power Class 3, but this has not yet been incorporated into the technical specifications. Since frequency band n104 (6.425-7.125 GHz) has not been deployed yet and no UEs implement it, there seems to be options to modify requirements which needs further discussion. The WF [1] captures the following tasks:

<Way forward/Agreement> - RAN4 introduces a band specific ACLR of 26 dB to n104 with a dedicated MPR under the following conditions. - PC3 for n104 and CA_n104C and Band Combinations (BCs) including n104 - 1Tx and 2Tx are considered

# 2  Discussion

In RAN4#116 RAN4 agreed to relax ACLR of n104 to 26dB for PC3. The goal is to evaluate MPR changes based on the relax requirement. The band was originally introduced in Rel-17. Changes shall be made for starting from this release. Relaxations to requirements are always possible as it does not harm deployments and UE development cycles. Since the band has not been deployed yet, RAN4 understanding is that the requirements can be modified and MPR tightened without causing disruptions in the field. However, it might be the better choice to not change MPR for single CC and CA and leave optimisation to the UE vendor.

Observation 1: RAN4 considers requirement changes to Rel-17 n104 band possible as it has not been deployed in the field yet. ACLR relaxations are fine as it does not harm deployments and UE development cycles. However, it might be the better choice to not change MPR for single CC and CA and leave optimisation to the UE vendor. This avoid tightening requirements retroactively in frozen releases.

Proposal 1: To avoid tightening requirements in frozen releases consider to only modify ACLR and leave MPR optimisation to UE vendors.

MPR for band n104 could potentially tightened for active release which is currently Rel-20. To evaluate the output power potential for PC3 single CC on band n104 simulations were made with 26dBc ACLR instead of 30dBc. The amplifier calibration procedure is not modified. The legacy calibration waveform (DFT-s, QPSK, 20MHz, 100RB0) has been used with 30dBc ACLR for PC3. The power back-off simulations have been conducted using 26dBc. This approach will provide decrease power back-off need for outer allocations as those are mainly affected by ACLR. Inner allocations are either limited by MOP of the power class or the EVM/IBE requirement.

Our simulations showed that gains are possible for QPSK and 16QAM. These results are provided in Table 1. The modulations 64QAM and 256QAM are either EVM or IBE limited as shown in Table 2. Therefore, the ACLR relaxation cannot provide any MPR improvement for higher order modulations.

Table 1: Power back-off results for QPSK and 16QAM

| Modulation | DFT-s-OFDM | CP-OFDM |
| --- | --- | --- |
| QPSK |  |  |
| 16QAM |  |  |

Table 2: Limiting factor for output power for 64QAM

| Modulation | DFT-s-OFDM | CP-OFDM |
| --- | --- | --- |
| 64QAM (30dBc ACLR) |  |  |
| 64QAM (26dBc ACLR) |  |  |

Based on the observations made in Table 1 and 2 and the contribution [1] from we are proposing to consider the MPR improvements shown in Table 3. The orange-coloured values mark the changes to regular PC3 MPR.

Table 3 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | ≤ 0.51 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.51 | 0 | 0 |
|  | QPSK | ≤ 1 | ≤ 0.5 | 0 |
|  | 16 QAM | ≤ 2 | ≤ 1 | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 | ≤ 2.0 | ≤ 1.5 |
|  | 16 QAM | ≤ 3 | ≤ 2.5 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB |  |  |  |  |

Proposal 2: Do not modify MPR of edge RB allocations as those are limited by SEM and will not observe improvements.

Proposal 3: Consider Table 3 when discussion potential MPR improvements for n104 with 26dBc ACLR.

Intra-band CA MPR was constructed by careful measurements including various steps, such as single and dual PA. If RAN4 consider modifying CA MPR for n104 the same analysis needs to be conducted. Running simulations alone isn’t feasible because they don’t consider real-world factors like reverse intermodulation and memory, which are crucial for high bandwidth (e.g., 200MHz). It is therefore proposed to either run extensive measurement campaigns to verify any CA MPR tightening or to keep allowance as is. The latter one would imply that UE vendors shall identify their own MPR improvements.

Observation 2: Intra-band CA MPR was constructed by careful measurements including various steps, such as single and dual PA. If RAN4 consider modifying CA MPR for n104 the same analysis needs to be conducted. Running simulations alone isn’t feasible because they don’t consider real-world factors like reverse intermodulation and memory, which are crucial for high bandwidth (e.g., 200MHz).

Proposal 4: Either conduct extensive measurement campaigns to confirm any CA MPR tightening or maintain the current allowance and let UE vendors optimise MPR.

# 4	Conclusions

This contribution provides simulation results n104 with ACLR of 26dBc and has the following observation and conclusion:

Observation 1: RAN4 considers requirement changes to Rel-17 n104 band possible as it has not been deployed in the field yet. ACLR relaxations are fine as it does not harm deployments and UE development cycles. However, it might be the better choice to not change MPR for single CC and CA and leave optimisation to the UE vendor. This avoid tightening requirements retroactively in frozen releases.

Proposal 1: To avoid tightening requirements in frozen releases consider to only modify ACLR and leave MPR optimisation to UE vendors. MPR improvements can be considered from Rel-20.

Proposal 2: Do not modify MPR of edge RB allocations as those are limited by SEM and will not observe improvements.

Proposal 3: Consider Table 3 when discussion potential MPR improvements for n104 with 26dBc ACLR.

Observation 2: Intra-band CA MPR was constructed by careful measurements including various steps, such as single and dual PA. If RAN4 consider modifying CA MPR for n104 the same analysis needs to be conducted. Running simulations alone isn’t feasible because they don’t consider real-world factors like reverse intermodulation and memory, which are crucial for high bandwidth (e.g., 200MHz).

Proposal 4: Either conduct extensive measurement campaigns to confirm any CA MPR tightening or maintain the current allowance and let UE vendors optimise MPR.

# 4	References

- R4-2511799, “Way forward for MPR reductio for n104”, Huawei, 3GPP TSG-RAN WG4 Meeting #106