3GPP TSG-RAN WG4 Meeting #117				                          R4-2522039

Dallas, USA, Nov. 17-21, 2025

Source:	Sony

Title:	Maintenance on IoT NTN UE RF -- ETSI issue band255.

Agenda item:	10.7

Document for:	Discussion

# Background

In RAN4#116bis, discussion on introducing ETSI emission requirement were discussed [1]. In this paper, we re-submit some analysis for b255 against ETSI in-band and out-of-band mask based on PC3 IoT commercial devices.

# IB and OOB emission requirements in ETSI

The IB emission and OOB emission requirement in the EN 301 681 specification [2] vs. 3GPP SEM for cat-M1 and NB-IoT have been plotted in Fig. 1 and Fig. 2, respectively. The results are scaled to dBm/1MHz. Generally speaking, some discrepancies between ETSI and 3GPP emission requirements exist, and the ETSI requirement is more stringent than 3GPP requirements at some frequency offset range.

Observation 1: There is a discrepancy between 3GPP and ETSI SEM requirements, and ETSI SEMs are tighter than 3GPP SEMs at some frequency offsets.

## IB and OOB emission requirement in ETSI for cat-M1

For cat-M1 (Fig. 1), the ETSI requirement is more stringent than 3GPP SEM within a wide frequency offset range. However, such a discrepancy is mainly caused by different SEM specification methods used by 3GPP and ETSI: the stepwise SEM is adopted in 3GPP but linearly dBW-interpolated SEMs are used in ETSI. In our view, these discrepancies may not be critical since the SEM of real devices is closer to the ETSI method (linearly interpolated).

Observation 2: The discrepancies in CatM1 masks are caused by different SEM specification methods used by 3GPP and ETSI (stepwise SEM in 3GPP vs. linearly dBW-interpolated SEM in ETSI) – these discrepancies may not be critical since the real SEM performance is closer to ETSI method.

- (b)

Fig. 1. The comparison of ETSI (a) IB and (b) OOB emission limit vs. 3GPP SEM for cat-M1.

## IB and OOB emission requirement in ETSI for NB-IoT

For NB-IoT (Fig. 2), The more stringent IB emission limit from ETSI might be tolerated at the expense of the actual SEM margins. However, the OOB emission mask (see Fig. 2(b)) becomes the most concerning case when NB-IoT carrier is located at the edge of the operating frequency band (e.g., b255). Therefore, 3GPP shall consider specifying a guard band or A-MPR to help the 3GPP devices meet ETSI requirements unless ETSI could consider aligning their OOB emission limit requirement with 3GPP specification.

Observation 3: The discrepancies in NB-IoT masks are mostly concerning if NB-IoT is located at the edge of the operating frequency band b255, and a sufficient guard or A-MPR to help NB-IoT devices to meet the ETSI OOB emission limit when the carrier is on the edge of the operating frequency band.

- (b)

Fig. 2. The comparison of ETSI (a) IB and (b) OOB emission limit vs. 3GPP SEM for NB-IoT.

To ensure the NB-IoT devices to meet the ETSI OOB emission limit, a guard band on the edges of band 255 can be assigned. It is worth noting that that a mask defined as spurious emission requiremetn in 36.102 which is identical to the out of band mask as in ETSI has been captured in 36.102, although the reson is that such a mask also adopted by ITU.

Meanwhile, a guardband eqvuilently to ensure 200 kHz guardband has also been defined for b255 to accommnodate such a supuroouse emission requiurement.

Observation 4: the mask equivelent to ETSI out of band mask has been defined in 36.102 as supuriouse emission requiremetn for b255 and a corresponding guardband has been defined.

## Measured emission performance of actual devices for cat-M1 and NB-IoT

To verify our analysis above, measured results for a cat-M1 and NB-IoT devices are in Fig. 3 and 4, respectively. All plots are scaled to dBm/30kHz in this case, and the correponding allowed MPR for each test case has been applied. For NB-IoT, both singel tone and 12 tone cases are measured. As expected and approved by measurements of the typical CatM1 and NB-IoT devices at Tx frequency close to the b255 UL band - all discrepancies between the ETSI and 3GPP masks may be tolerated at the expense of the actual SEM margins except for ETSI OOB requirement for NB-IoT at 100kHz to ~200kHz offset from channel center.Moreover, the NB-IoT 1-Tone with max Pout (MPR0) has been identified a worst case, which needs guard band must be larger than 100kHz to meet the ETSI OOB emission requirement with no margins.

Observation 5: Approved by measurements of the typical CatM1 and NB-IoT devices at Tx frequency close to the b255 UL band - all discrepancies between the ETSI and 3GPP masks may be tolerated at the expense of the actual SEM margins except for ETSI OOB requirement for NB-IoT at ~200kHz offset from channel center.

- (b)

Fig. 3. The comparison of measured emission mask for an actual cat-M1 device vs. 3GPP SEM and ETSI (a) IB and (b) OOB emission limit.

- (b)

Fig. 4. The comparison of measured emission mask for an actual NB-IoT device vs. 3GPP SEM and ETSI (a) IB and (b) OOB emission limit.

# Conclusion

In this contribution, we make the following observations and conclusions:

Observation 1: There is a discrepancy between 3GPP and ETSI SEM requirements, and ETSI SEMs are tighter than 3GPP SEMs at some frequency offsets.

Observation 2: The discrepancies in CatM1 masks are caused by different SEM specification methods used by 3GPP and ETSI (stepwise SEM in 3GPP vs. linearly dBW-interpolated SEM in ETSI) – these discrepancies may not be critical since the real SEM performance is closer to ETSI method.

Observation 3: The discrepancies in NB-IoT masks are mostly concerning if NB-IoT is located at the edge of the operating frequency band b255, and a sufficient guard or A-MPR to help NB-IoT devices to meet the ETSI OOB emission limit when the carrier is on the edge of the operating frequency band.

Observation 4: the mask equivelent to ETSI out of band mask has been defined in 36.102 as supuriouse emission requiremetn for b255 and a corresponding guardband has been defined.

Observation 5: Approved by measurements of the typical CatM1 and NB-IoT devices at Tx frequency close to the b255 UL band - all discrepancies between the ETSI and 3GPP masks may be tolerated at the expense of the actual SEM margins except for ETSI OOB requirement for NB-IoT at ~200kHz offset from channel center.

# References

- R4-2513841		Missing ETSI emission requirements for IoT NTN bands 253 and 255 and NR NTN band n255

- ETSI EN 301 681 V2.1.2 (2016-11): " Satellite Earth Stations and Systems (SES); Harmonised Standard for Mobile Earth Stations (MES) of Geostationary mobile satellite systems, including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) under the Mobile Satellite Service (MSS), operating in the 1,5 GHz and 1,6 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU" 2016-11