3GPP TSG-RAN WG4 Meeting #117				                           R4-2522047

Dallas, USA, Nov. 17-21, 2025

Source:	Sony

Title:	Further views on channel arrangement of 6GR

Agenda item:	8.2.4

Document for:	Approval

# Background

In RAN4#116bis, initial discussion on system parameter for 6GR from RAN4 side was carried out [1] and a WF was agreed [2].  In this contribution, we share our further views on channel arrangement based on the scope that was agreed in the WF.

# Sync raster

In 5G NR, a sync raster around 1.2 MHz/1.4 MHz is used to ensure that for any channel position for a given channel raster, there always a sync raster position available so that offer sufficient deployment flexibility. The principle of sync raster design is illustrated in Fig. 2, while The sync channel raster spacing ΔFSC,Raster is defined according to the following equation:

ΔFSC,Raster ≤ BWConfig – BWPBCH + ΔFCH,Raster

However, only very little defined sync raster is in use as discussed in [3], while UE still needs to search for all possible positions which cause longer delay and power consumption.

Observation 1: large number of sync raster defined in 5G NR is not used in the field.

The delay due to scanning through a large number of sync raster is expected to be more prominent in 6GR design as SSB periodicity may even be reduced (less frequent) and longer delay and more complicated searching algorithm may be needed on UE side. Therefore, to mitigate such a burden, a sparse sync raster design would be benefit in 6GR.

Observation 2: The delay due to scanning through a large number of sync raster is expected to be more prominent in 6GR design as SSB periodicity may even be reduced, and thus a sparse sync raster design would be benefit in 6GR.

Fig. 2. Sync raster design principle in 5G NR (source: TR38.817)

As a starting point, for the frequency bands where the minimum bandwidth is larger than 5MHz (e.g., 20 MHz minimum bandwidth in n96, n102, n104 based on 5G NR), a sparser sync raster can be designed if we take the same principle of 5G NR in those frequency bands. For frequency bands where the minimum bandwidth may still require to be 5MHz or even 3MHz, RAN4 may consider taking a subset of existing sync raster in 5G NR, e.g., every 3rd,  and further discuss if additional sync raster would be needed in a band-by-band manner.

Proposal 1: RAN4 study the approach to reducing the number of sync raster in 6GR.

# Channel raster

For channel raster design, the key is to ensure a high utilization of the operators spectrum and also ensure the compatibility with legacy RAT. For 5G NR, 100 kHz channel raster was defined initially but an enhanced channel raster with 10 kHz was introduced in the later stages of 5G NR to offer more efficient spectrum usage. The channel raster should not significantly impact the UE hardware implementation. On the other hand, it needs to be considered whether a 10 kHz channel raster would affect coexistence with 5G NR or LTE systems, as many legacy systems still operate with a 100 kHz channel raster. One possibility is to adopt a 5kHz channel raster. Therefore, we propose that 6G consider enhanced channel raster (than 100 kHz) from the beginning to ensure that the spectrum usage of 6G can be more efficient. Meanwhile, further study is needed to determine if different channel raster between 5G NR/LTE and 6GR would cause any issues in terms of MRSS and/or in-band coexistence.

Proposal 2: It is proposed 6G consider an enhanced channel raster (finer than 100 kHz) from the beginning to ensure the spectrum usage of 6G can be more efficient than 5G. Meanwhile, further study if a different channel raster between 5G NR/LTE and 6GR would cause any issue in terms of MRSS and/or in-band coexistence

# Conclusion

In this contribution, we share our initial view on the system design of 6GR from the RAN4 perspective, where the following observations and proposals are given:

Observation 1: large number of sync raster defined in 5G NR is not used in the field.

Observation 2: The delay due to scanning through a large number of sync raster is expected to be more prominent in 6GR design as SSB periodicity may even be reduced, and thus a sparse sync raster design would be benefit in 6GR.

Proposal 1: RAN4 study the approach to reducing the number of sync raster in 6GR.

Proposal 2: It is proposed 6G consider an enhanced channel raster (finer than 100 kHz) from the beginning to ensure the spectrum usage of 6G can be more efficient than 5G. Meanwhile, further study if a different channel raster between 5G NR/LTE and 6GR would cause any issue in terms of MRSS and/or in-band coexistence

# References

- R4-2514508		Topic Summary for [116bis][101] 6G system parameter	Moderator (Huawei)

- R4-2514618		WF on [116bis][101] 6G system parameter	Huawei

- R4-2513075		Views on 6G system parameters	MediaTek inc.