3GPP TSG-RAN WG4 Meeting #117				                           R4-2522048

Dallas, USA, Nov. 17-21, 2025

Source:	Sony

Title:	Views on device type of 6GR

Agenda item:	8.2.5

Document for:	Approval

# Background

In RAN4#116bis, initial discussion on system parameters for 6GR from the RAN4 side was carried out [1] and a WF was agreed [2].  In this contribution, we share our further views on device type based on the scope that was agreed in the WF.

# Device types

The 6GR design targets a scalable and forward-compatible design for diverse device types [3]. Therefore, it is essential to clarify the device types that the 6GR needs to support before we dive into the detailed system parameters. Each device type may represent a set of common assumptions about UE hardware implementation and can contain a set of UE parameters/capabilities/features with different values, e.g., UE CBW, number of antennas, TX power, etc.

Observation 1: Each device type may represent a set of common assumptions about UE hardware implementation and can be specified as a set of UE parameters/capabilities/features e.g., UE CBW, number of antennas, TX power, etc., with different values

The basically methodology for device type definition can be considered as below:

- Common mandatory baseline parameters/capabilities/features set for all 6G devices to ensure basic interoperability

- Device-type-specific parameters/capabilities/features to cater for the diverse performance, cost, and power consumption targets

- Optional parameters/capabilities/features can be supported for each device type, e.g., AI, sensing, NTN etc.

With the above methodology, introducing UE device types in 6GR can help to streamline the UE capability framework and avoid excessive UE capability reporting. Additionally, with a clear set of features to be defined for each device type, it can also help to reduce the effort on Interoperability and Development Testing (IoDT) among UE vendors, network vendors and operators.

Observation 2: Introducing device type in 6GR can help to avoid excessive UE capability reporting and the effort on Interoperability and Development Testing (IoDT).

To guarantee the above benefit of device type reporting, it would require clear core requirements to be defined for each feature in RAN4, to ensure the UE functions according to the reported device type. In other words, minimum requirements for each device type should be mapped to UE capabilities associated with the device type, i.e. if a capability is supported/indicated, then specific minimum requirements must be applied.

Observation 3: It is critical for RAN4 to define clear core requirements for features of each device type to ensure the UE functions according to the reported device type.

It is clear that RAN4 shall define the core requirements for each device type based on certain implementation assumptions as mentioned in the beginning. In RAN4#116bis, the following agreements were made regarding the device type discussion:

The following aspects could be further studied

  - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range.

    - Study on the NW impact should also be considered

  - Study how to better support variety devices from RAN4 perspective

    - E.g. differentiate devices, use cases, features

  - Other aspects are not precluded

- Collaborate on the framework with other WGs:

  - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

The intended usage scenario or use case of a device is inevitably one of the key factors to be considered first when it comes to determining the device type in 6GR in our view. The main differentiator in terms of the usage scenario when it comes to device type discussion is whether the device is for massive IoT or mobile broadband.  An IoT device would have a lower device complexity and cost structure than mobile broad band, which leads to different assumptions for UE hardware capabilities regardless of formfactor of the device.

Observation 4: The targeted usage scenario, e.g., if it is massive IoT or if it is a mobile broadband device, is a key factor to be considered when determining the hardware assumption for device type, due to the different cost and complexity considerations.

On top of the usage scenario, formfactor is a critical aspect to be considered as well when determining the device type as it plays a major role in constraining the device implementation. For example, the number of antennas assumption can be different between a handheld device, e.g., smartphone, and a non-handheld device, e.g., FWA/CPE devices, although the cost/complexity assumption could be similar.

Observation 5: For a given usage scenario, e.g., mobile broadband, the formfactor limitation plays a key role to differentiate device types.

With the consideration above, it is proposed to consider the following device types as a starting point and further discuss whether more device types would be needed.

- Massive IoT device (e.g., tracker, wearable devices, meters)

  - Aim for device capability above 4G IoT and below 5G NR (e)Redcap,

  - with reduced device complexity than legacy devices (e.g., 5G (e)Redcap)

- Mobile broadband device:

  - Broadband device with limited formfactor (e.g., smartphone, glass type XR)

    - Improved data rate but limited additional complexity to legacy devices (e.g., 5G eMMB)

    - considering the formfactor limitation when determining the device parameters/capabilities

  - Ultra Broadband devices (e.g., FWA/CPE, HMD XR)

    - focus on fulfilling the extraordinary performance bounds

    - no major constraints due to device formfactor but with realistic implementation assumption

Proposal 1: Considering 1) Massive IoT device, 2) Broadband device with limited formfactor e.g., smartphone, 3) Ultra Broadband devices (e.g., FWA/CPE, XR) as starting point for definition, further discuss if more device types need to be defined in 6GR.

The 6GR supports a diverse range of service classes, from immersive multimedia and massive IoT to integrated communications and sensing [3]. To avoid over-fragmentation, the device type definitions in 6GR should consider grouping devices that share similar hardware capabilities or performance requirements. For example, both FWA devices and HMD XR devices may need to offer superior throughput to the end user while facing fewer constraints in terms of the device's form factor, which can be categorized into the same device type.

Proposal 2: Devices that share similar hardware capabilities or performance requirements shall be categorized into the same device types.

The detailed parameters/capabilities of those devices are listed in Table I below.

Table I. 6GR device type and associated device capability

|  | Massive IoT device, e.g., tracker, wearable devices, meters | Broadband device with formfactor limitations, e.g., smartphone, glass type XR devices | Ultra broadband device e.g., FWA/CPE, Vehicle, HMD type XR devices. |
| --- | --- | --- | --- |
| Design considerations | - Aim for device capability above 4G IoT and below 5G NR, - with reduced device complexity than legacy devices (e.g., 5G (e)Redcap) | - Improved data rate but limited additional complexity to legacy devices (e.g., 5G eMMB) - consider the formfactor limitation when determining the device parameters/capabilities | - focus on fulfilling the extraordinary performance bounds - no major constraints due to device formfactor but with realistic implementation assumption |
| Frequency bands/range | Focus on frequency bands < 3 or 4 GHz | All bands in FR1, FR2-1, “FR3” | All bands in FR1, FR2-1, “FR3” |
| Minimum UE CBW | 3MHz, further discuss if the system should be optimized based on 3MHz or 5MHz |  |  |
| Maximum UE CBW (single CC) | [5-20] MHz But with consideration of 3MHz max UE CBW in UL direction to enable SAW-less implementation | - 100 MHz in FR1 (same as 5G NR) - 200 MHz in “FR3” - 400 MHz in FR2-1(same as 5G NR) | - 200 MHz in FR1 - 400 MHz in “FR3” - 400 MHz in FR2-1 |
| Duplex mode | Focus on HD-FDD, FD-FDD/TDD can be supported | FD-FDD/TDD | FD-FDD/TDD |
| Number of antennas | 1T/1R | FR1&FR2-1 antenna assumption to be the same as in 5G NR, 2T/4R in FR3. | 4T/8R in FR1 and “FR3” FR2-1 antenna assumption to be the same as in 5G NR. |
| Default power class | 23 dBm | 23 dBm in FR1 26 dBm in “FR3” FFS in FR2-1, Considering enhancement on PC3 in FR2-1, without precluding other possible power classes. | 26 dBm in FR1 31 dBm in “FR3” PC4 in FR2-1 |
| Mobility | Idle mode mobility as baseline | Connected mode mobility |  |
| Maximum Modulation order | 64QAM UL/64QAM DL | 256 QAM UL/1024 QAM DL in all frequency ranges | 1024 QAM UL/1024 QAM DL in FR1, “FR3” 256 QAM UL/1024 QAM DL in FR2 |
| Note: “FR3” refers to the spectrum between FR1 and FR2-1 in this contribution. |  |  |  |

# Number of Antennas

As many other parameters in the device type discussion have been addressed in our accompanying papers in agenda items 8.2.1 to 8.2.4, in this paper, we only dive into the number of antennas discussion.

For Massive IoT devices, considering the device complexity and possible form factor limitations for some use cases, such as wearable devices and smaller trackers, it is proposed to adopt 1T/1R as the baseline.

Proposal 3: For massive IoT devices, it is proposed to adopt 1T/1R as a baseline.

For broadband devices with form factor limitations, such as smartphones, their form factor is unlikely to change in the 6G era. Therefore, it is proposed to retain the same number of antennas as in 5G NR as the starting point in FR1. For FR3, it is proposed to adopt 2T/4R as the starting point and allow companies to investigate whether a larger number of antennas can be fitted or needed. In FR2-1, a device with two antenna panels, each with 4*1 antenna elements, can be used as a baseline to define the spherical coverage without compromising the minimum requirements of single-panel implementations.

Proposal 4: For broadband devices with form factor limitations, like smartphones, it is proposed to retain the same number of antennas as in 5G NR as the starting point for FR1. For FR3, it is proposed to adopt 2T/4R as the starting point. In FR2-1, a device with two antenna panels, each with 4*1 antenna elements, can be used as a baseline to define the spherical coverage without compromising the minimum requirements of single-panel implementations.

For ultra broadband devices, due to higher demand on the device performance and fewer constraints on the device's form factor, it is proposed to adopt 4T/8R in FR1 and FR3, while for FR2-1, three antenna panels with at least 4*1 antenna elements can be used as a baseline assumption to define the minimum requirements of the device type.

Proposal 5: For ultra broadband devices, it is proposed to adopt 4T/8R in FR1 and FR3, while for FR2-1, three antenna panels with at least 4*1 antenna elements can be used as a baseline assumption to define the minimum requirements of the device type.

Last but not least, what is critical is the number of MIMO layers that a device needs to support. In 5G NR, different numbers of MIMO layers than the number of antennas was defined, for example a smartphone can support 6 RX but only needs to support 4 layers of MIMO communication. Therefore, this is also a point RAN4 needs to investigate and clarify.

Observation 6: in 5G NR, the number of antennas can be different to the number of MIMO layers that the UE can support.

Proposal 6: RAN4 shall also study and clarify if the supported number layer of MIMO layers equals the number of antennas.

# Conclusion

In this contribution, we share our view on the device type of 6GR from the RAN4 perspective, where the following observations and proposals are given:

Observation 1: Each device type may represent a set of common assumptions about UE hardware implementation and can be specified as a set of UE parameters/capabilities/features e.g., UE CBW, number of antennas, TX power, etc., with different values

Observation 2: Introducing device type in 6GR can help to avoid excessive UE capability reporting and the effort on Interoperability and Development Testing (IoDT).

Observation 3: It is critical for RAN4 to define clear core requirements for features of each device type to ensure the UE functions according to the reported device type.

Observation 4: The targeted usage scenario, e.g., if it is massive IoT or if it is a mobile broadband device is key factor to be considered when determining the hardware assumption for device type, due to the different cost and complexity considerations.

Observation 5: For a given usage scenario, e.g., mobile broadband, the formfactor limitation plays key role to differentiate device types.

Observation 6: in 5G NR, the number of antennas can be different to the number of MIMO layer that UE can support.

Proposal 1: Considering 1) Massive IoT device, 2) Broadband device with limited formfactor e.g., smartphone, 3) Ultra Broadband devices (e.g., FWA/CPE, XR) as starting point to define, further discuss if more device types need to be defined in 6GR.

Table I. 6GR device type and associated device capability

|  | Massive IoT device, e.g., tracker, wearable devices, meters | Broadband device with formfactor limitations, e.g., smartphone, glass type XR devices | Ultra broadband device e.g., FWA/CPE, Vehicle, HMD type XR devices. |
| --- | --- | --- | --- |
| Design considerations | - Aim for device capability above 4G IoT and below 5G NR, - with reduced device complexity than legacy devices (e.g., 5G (e)Redcap) | - Improved data rate but limited complexity additional to legacy devices (e.g., 5G eMMB) - consider the formfactor limitation when determining the device parameters/capabilities | - focus on fulfilling the extraordinary performance bounds - no major constraints due to device formfactor but with realistic implementation assumption |
| Frequency bands/range | Focus on frequency bands < 3 or 4 GHz | All bands in FR1, FR2-1, “FR3” | All bands in FR1, FR2-1, “FR3” |
| Minimum UE CBW | 3MHz, further discuss if the system should be optimized based on 3MHz or 5MHz |  |  |
| Maximum UE CBW (single CC) | [5-20] MHz But with consideration of 3MHz max UE CBW in UL direction to enable SAW-less implementation | - 100 MHz in FR1 (same as 5G NR) - 200 MHz in “FR3” - 400 MHz in FR2-1(same as 5G NR) | - 200 MHz in FR1 - 400 MHz in “FR3” - 400 MHz in FR2-1 |
| Duplex mode | Focus on HD-FDD, FD-FDD/TDD can be supported | FD-FDD/TDD | FD-FDD/TDD |
| Number of antennas | 1T/1R | FR1&FR2-1 antenna assumption to be the same as in 5G NR, 2T/4R in FR3. | 4T/8R in FR1 and “FR3” FR2-1 antenna assumption to be the same as in 5G NR. |
| Default power class | 23 dBm | 23 dBm in FR1 26 dBm in “FR3” FFS in FR2-1, Considering enhancement on PC3 in FR2-1, without precluding other possible power classes. | 26 dBm in FR1 31 dBm in “FR3” PC4 in FR2-1 |
| Mobility | Idle mode mobility as baseline | Connected mode mobility |  |
| Maximum Modulation order | 64QAM UL/64QAM DL | 256 QAM UL/1024 QAM DL in all frequency ranges | 1024 QAM UL/1024 QAM DL in FR1, “FR3” 256 QAM UL/1024 QAM DL in FR2 |
| Note: “FR3” refer to the spectrum between FR1 and FR2-1 in this contribution. |  |  |  |

Proposal 2: Devices that share similar hardware capabilities or performance requirements shall be categorized into the same device types.

Proposal 3: For massive IoT devices, it is proposed to adopt 1T/1R as a baseline.

Proposal 4: For broadband devices with form factor limitations, like smartphones, it is proposed to retain the same number of antennas as in 5G NR as the starting point for FR1. For FR3, it is proposed to adopt 2T/4R as the starting point. In FR2-1, a device with two antenna panels, each with 4*1 antenna elements, can be used as a baseline to define the spherical coverage without compromising the minimum requirements of single-panel implementations.

Proposal 5: For ultra broadband devices, it is proposed to adopt 4T/8R in FR1 and FR3, while for FR2-1, three antenna panels with at least 4*1 antenna elements can be used as a baseline assumption to define the minimum requirements of the device type.

Proposal 6: RAN4 shall also study and clarify if the supported layer of MIMO equals the number of antennas.

# References

- R4-2514508		Topic Summary for [116bis][101] 6G system parameter	Moderator (Huawei)

- R4-2514618		WF on [116bis][101] 6G system parameter	Huawei