3GPP TSG-RAN WG4 Meeting #117	R4-2522092

Dallas, USA,  17th – 21st November 2025

Agenda item:	12

Source:	Apple

Title:	On band combination database improvements

WI/SI:

Release:	Rel-20

Document for:	Decision

# 1	Introduction

The agreement to transition from a table-based format capturing thousands of band combinations to a database in Rel-20 time frame[1] is a major leap forward for RAN4. During Rel-20 time frame a database will be finalised containing all CA and DC combinations from TS 38.101-1/2/3. This will allow simplified handling and data processing. The database consists of individual JSON files storing the data in a dedicated structured format. Proposal is made for referencing the technical reports for band notation. Additionally, a proposal is made to consider a dedicated database for A-MPR requirements.

# 2	Improvements to JSON file format and notation

While working on the existing JSON schema it has been found that certain modification could be needed for accuracy against RAN4 agreements and for processing purposes.

##### 2.1 Carrier Aggregation Notation

When RAN4 discussed the band combination handling and notation in 2018 it was agreed that band combination notations follow a specific format. This notation system is specified in TR 38.817-01 [2]. In clause 4.1 the technical report defines the standardized representation methods for NR-only configurations, EN-DC and SUL combinations. It establishes the fundamental notation principles, using Arabic numerals for LTE bands and "n"-prefixed numerals for NR bands, along with specific formatting conventions to distinguish between intra-band/inter-band and contiguous/non-contiguous deployments. The specification includes detailed examples and bandwidth class representations for different combination types, providing fundamental reference for notation of band combinations.

Part of the essential notation format is the use of band combination prefixes such as ‘CA’, ‘DC’ and ‘SUL’. Those are necessary to differentiate between various combination types (i.e. NR-CA, NR-DC, EN-DC etc.). It is a fundamental part of the notation which is repeated and expended upon in TR 38.846 clause 6.1 [3] and will also be part of the new PRD document. The TR defines the prefix notation for band combinations which are "CA_", "DC_", "SUL_", "V2X_" and makes them mandatory. Below is an excerpt from clause 6.1B listing the first four rules for the notation of band combinations. The focus lies on the first bullet which makes it mandatory for a configuration to start with either “CA_”, “DC_” or any other prefix.

Removing band combination prefixes from JSON files creates issues going beyond simple reader confusion. The prefixes serve as essential semantic identifiers providing the type of the band combination. Dropping the prefix in the database breaks the notation rules set up in TR 38.817-01 [2], TR 38.846 [3] and TR 38.862 [4]. Furthermore, the removal creates overhead by requiring custom parsing logic and exception handling since the source repository needs to be included as “part” of the band combination notation. The reliance on external context to complete the context of a band combination (e.g. whether it is a NR-CA or NR-DC combo) is not a good approach and RAN4 should not take this direction for 5G or 6G specification. Instead, the band combination prefix should be included in the database according to the specified notation rules.

Observation 1: RAN4 established mandatory band combination prefixes (CA, DC, SUL, V2X) in TR 38.817-01 and TR 38.846 to create a standardized notation system that differentiates between various combination types using clear semantic identifiers.

Proposal 1: We propose that the TRs laying out the rules for band combination notation should be listed in the tdoc capturing the progress and next steps to provide proper reference for future development of the database. Those TRs are TR 38.817-01, TR 38.846 and TR 38.862.

##### 2.2 Database for A-MPR

The JSON based database offers improved machine readability allowing more efficient and faster data crunching. Analog to band combinations the A-MPR tables (regions and values) need to be transferred from word-based specification. Lots of effort is needed to guarantee error free conversion from word tables into internal databases. Major improvements would be expected if A-MPR requirements would also be provided as database.

Observation 2: Analog to band combinations the A-MPR tables (regions and values) need to be transferred from word-based specification. Lots of effort is needed to guarantee error free conversion from word tables into internal databases. Major improvements would be expected if A-MPR requirements would also be provided as database.

Proposal 2: RAN4 should discuss whether A-MPR requirements could be moved into a database for 6G.

# 3	Conclusion

This contribution discusses certain challenges and shortcomings of the current JSON database implementation and suggests improvements. As final suggestion it is proposed to consider a database approach for A-MPR requirements. The following observations and proposal are made:

Observation 1: RAN4 established mandatory band combination prefixes (CA, DC, SUL, V2X) in TR 38.817-01 and TR 38.846 to create a standardized notation system that differentiates between various combination types using clear semantic identifiers.

Proposal 1: We propose that the TRs laying out the rules for band combination notation should be listed in the tdoc capturing the progress and next steps to provide proper reference for future development of the database. Those TRs are TR 38.817-01, TR 38.846 and TR 38.862.

Observation 2: Analog to band combinations the A-MPR tables (regions and values) need to be transferred from word-based specification. Lots of effort is needed to guarantee error free conversion from word tables into internal databases. Major improvements would be expected if A-MPR requirements would also be provided as database.

Proposal 2: RAN4 should discuss whether A-MPR requirements could be moved into a database for 6G.

# 4	References

- R4-2511804, “WF for the ETSI MCC band combination database”, Nokia, 3GPP TSG-RAN WG4 Meeting #116

- TR 38.817-01, “General aspects for User Equipment (UE) Radio Frequency (RF) for NR”, v16.4.0, 3GPP

- TR 38.846, “Study on simplification of band combination specification for NR and LTE”, v18.1.0, 3GPP