3GPP TSG-RAN WG4 Meeting #116bis	R4-2522094

Dallas, US, 17th – 21st November 2025

Source:	Nokia

Title:	On remaining LBCA requirements

Agenda item:	4.8.2

Document for:	Discussion

Introduction

In this contribution we discuss the remaining issues related to core part requirements for LBCA via switching.

Discussion

SCell activation delay

In the last meeting, RAN4 agreed the following about TLBCA for SCell activation:

Issue 1-1-3b: switching pattern based carrier switching in SCell activation Agreement: - TLBCA =1ms for MAC CE based, 0 ms for RRC based. For the same issue, the chair meeting minutes contains the following tentative agreement: Tentative Agreement: - For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(THARQ+3ms+TLBCA)/slot-length. Switching is placed at the end of the slot. - For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(TRRC_Process +TLBCA)/slot-length. Switching is placed at the end of the slot.

Because the value for TLBCA is non-zero, this delay needs to be taken into account in the SCell activation delay. For example, when the SCell to be activated is known, the first step of SCell activation after receiving and processing the SCell activation command and sending HARQ/RRC response is DL synchronization through the first SSB. The first SSB on the SCell may be sent immediately after THARQ+3ms or it may be sent later, depending on when the SCell activation command is sent in relation to the SSB. If the SSB is sent on the SCell during TLBCA i.e. before the pattern is activated, not including TLBCA in the SCell activation delay or the definition of TfirstSSB would mean that the UE would need to receive the SSB on the SCell before the switching pattern is activated. This would lead to an interruption on PCell and for the need for RAN4 to define a related requirement.

Therefore, we think it is clearer to assume that the UE can receive the first SSB or perform any measurements related to unknown SCell activation when the pattern is already activated. Therefore, TLBCA parameter should be added in the SCell activation delay and any measurements related to activation (Tactivation_time) would happen only after the switching pattern is activated. Consequently, with the same justification, we support the above tentative agreement from the last meeting.

Add TLBCA component in SCell activation delay before Tactivation_time.

Agree the tentative agreement:

  - For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(THARQ+3ms+TLBCA)/slot-length. Switching is placed at the end of the slot.

  - For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(TRRC_Process +TLBCA)/slot-length. Switching is placed at the end of the slot.

Impact of C-DRX on LBCA RRM Requirements

RAN4 introduced LBCA specific measurement delay requirements where the impact of LBCA switching pattern is taken into account in PCell and SCell measurement requirements.

Issue 1-3-1: Requirement design of activated SDL SCell L3 measurement Agreement in online session: - if the activated SCC measurement is MG-based in LB-CA, the legacy intra-frequency identification/measurement requirement with MG can be applied. - reuse the existing measurement requirements with gap for inter-frequency measurement and inter-RAT measurement in LB-CA. - Not consider the scenario and not define the requirement for measurement without gap for inter-frequency measurement and inter-RAT measurement in LB-CA. Agreement in online session: agree on the following approach, and further update on the wording is not precluded. Further discuss on the CR drafting. - if the activated SCC measurement is MG-less in LB-CA, Kp factor can be updated for activated SCC identification/measurement requirement (intra-frequency measurement without MG) for LB-CA. - Kp factor for MG-less activated SCC identification/measurement requirement in LB-CA can be updated as following: - Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(SMTC period, xRP_max, switch pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and switch pattern periodicity is the periodicity of the NW configured switch pattern for LB CA via switching, and starting from the beginning of any SMTC occasion: - Ntotal is the total number of SMTC occasions within the window, including those overlapped with MG and those overlapped with FDD PCC duration based on the switching pattern within the window, and - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions and not overlapped with FDD PCC duration based on the switching pattern within the window W. - No requirement when Navailable = 0.

Regarding DRX, RAN4 never discussed how the switching pattern is applied. The agreement that was made about the Kp factor is general and applies to both non-DRX and DRX measurement delay, meaning that even when UE is in DRX and not scheduled, the UE is expected to follow the switching pattern for measurements. It is logical to assume that the pattern is active when the UE is being scheduled, but we think the pattern does not need to be followed for measurements during DRX.

It is defined in the measurement requirements that while the UE is being scheduled, non-DRX requirements apply. Hence, this would directly lead to UE following non-DRX requirements where the pattern is taken into account. However, when UE is in DRX and not scheduled, we think there is no need to take the pattern into account in the measurement delay.

Switching pattern is not applied when UE is in DRX.

  - Remove the impact of switching pattern for Kp factor for measurements during DRX.

If Proposal 3 is agreeable, inform RAN1 about the agreement in an LS

UL TCI state activation

RAN4 introduced the impact of LBCA switching to DL TCI state switching delay requirements. Tfirst-SSB definition was updated so that the SSB that is overlapping with SCell active period is taken into account. RAN4 also defined PL-RS switching requirements with LBCA switching, where the PL-RS that are overlapping with PCell active periods are taken into account.

However, RAN4 has not defined requirements for UL TCI state activation/switching for PCell, when switching pattern is activated. Here, if the PL-RS of the target TCI state is not maintained, PL-RS estimation is part of the TCI state switching delay, and it will clearly be impacted if LBCA switching pattern is activated. Therefore, for completeness of the requirements, we have proposed in our CR R4-2520470 how to capture these requirements.

RAN4 has not defined UL TCI state switching delay requirements for PCell with LBCA switching.

Introduce UL TCI state switching delay requirements for PCell by taking the LBCA switching pattern impact into account in PL-RS estimation delay.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Add TLBCA component in SCell activation delay before Tactivation_time.

Proposal 2: Agree the tentative agreement:

  - For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(THARQ+3ms+TLBCA)/slot-length. Switching is placed at the end of the slot.

  - For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(TRRC_Process +TLBCA)/slot-length. Switching is placed at the end of the slot.

Proposal 3: Switching pattern is not applied when UE is in DRX.

- Remove the impact of switching pattern for Kp factor for measurements during DRX.

Proposal 4: If Proposal 3 is agreeable, inform RAN1 about the agreement in an LS

Observation 1: RAN4 has not defined UL TCI state switching delay requirements for PCell with LBCA switching.

Proposal 5: Introduce UL TCI state switching delay requirements for PCell by taking the LBCA switching pattern impact into account in PL-RS estimation delay.