3GPP TSG-RAN WG4 Meeting #117	R4-2522095

Dallas, US, 17th – 21st November 2025

Source:	Nokia

Title:	Discussion on remaining RRM Ph5 requirements

Agenda item:	4.15.1

Document for:	Discussion

Introduction

In this paper we discuss remaining issues for RRM ph5 related to beam sweeping.

Fast L3 beam sweeping

The issue about re-entering FBS after timer expiration is still open in the last meeting’s WF:

Issue 1-1-1: Condition of re-entering FSB after exiting <Way Forward>: - Further discuss in next meeting considering the below options: - Option1: Define T2 for re-evaluation after the UE exits FBS mode. - Option2: Not to define T2 for re-evaluation after the UE exits FBS mode. - Option 2.1: Need to update core requirements. - Option 2.2: Not need to update core requirements.

In our view, defining a requirement for re-evaluating the triggering condition does not seem needed, as the UE is anyways continuously performing measurements and knows whether the condition is met or not.

The following cases exist:

- UE stops FBS at timer expiration and the condition after some time is not met any longer (UE moves back to cell center)

- UE exits FBS at timer expiration and the condition continues to be met (UE stays at the cell edge)

In the first scenario, UE shall re-enter FBS when the condition becomes met again (UE moves back to cell edge). However, we do not think this counts as re-entry as it is the same scenario as entry condition becoming fulfilled (again), where the UE would naturally need to restart FBS. Hence, this scenario is already covered.

After timer expiration, if the FBS condition stops being met and is met again, the UE shall re-enter FBS. This is already covered and has no specification impact.

Regarding the second scenario, it has been proposed that after a second timer, the UE would re-evaluate the condition and re-enter FBS. In our view it is questionable how a timer would benefit the UE or network in this case. If the UE stops and is stationary and network does not send a handover command, it does not matter whether the UE performs fast or normal beam sweeping. UE re-entering FBS would be useful rather if the UE moves even closer to the cell edge and handover becomes even more urgent. However, a second timer does not help to define such event. Therefore, we think such condition is not mandatory.

After timer expiration, if the FBS condition continues to be met, we prefer not to define a re-entry condition for FBS.

Issue 1-1-2: Multi-band supported by FBS <Agreement>: - The serving carrier shall be a FR2-1 carrier. - For inter-frequency measurement, when serving carrier and target carrier are in different bands, use the FBS value reported for the band of the target carrier in the requirements. Issue 1-1-4: Applicability of FBS when the target and serving carrier frequency are on different <Way Forward>: - Further discuss the options: - Option 1: When the target and serving carrier frequency are different, FBS applies to target carrier only. - Option 2: When the target and serving carrier frequency are different, FBS applies to the serving carrier and target carrier. - Option 3: Introduce UE capability indicating UE can support FBS on both target and serving carrier frequency.

About issue 1-1-4, some distinction might need to be done about the scenario, and whether the UE is expected to perform the measurements simultaneously in the serving and target cell. When measurements with gaps are considered, the CSSF is calculated such that the UE is not expected to measure 2 carriers simultaneously. Therefore, for measurements with gaps it is clear that the UE should be able to use FBS on the serving and target carriers.

For measurements without gaps, the assumption is that 2 searchers are used, and PCell has a dedicated searcher. So, that means that the UE might be performing measurements simultaneously for the serving and target carriers if the SMTC is overlapping. As pointed out in the beginning of this work item, some companies had concerns on using multi-Rx on 2 carriers simultaneously. For this reason, we think it is reasonable to assume that FBS is applied only on one carrier if the SMTC for the target and serving frequencies overlap.

FBS applicability when target and serving carrier frequency are different:

  - FBS applies to target carrier only

    - when the target and serving carrier frequency are different,

    - and when the SMTC for measurements without gaps overlaps for serving and target carrier,

  - FBS applies to the serving and target carrier otherwise.

Conclusion

The following Observations and Proposals were made:

Proposal 1: After timer expiration, if the FBS condition stops being met and is met again, the UE shall re-enter FBS. This is already covered and has no specification impact.

Proposal 2: After timer expiration, if the FBS condition continues to be met, we prefer not to define a re-entry condition for FBS.

Proposal 3: FBS applicability when target and serving carrier frequency are different:

- FBS applies to target carrier only

  - when the target and serving carrier frequency are different,

  - and when the SMTC for measurements without gaps overlaps for serving and target carrier,

- FBS applies to the serving and target carrier otherwise.