3GPP TSG-RAN WG4 Meeting #117	R4-2522098

Dallas, US, 17th – 21st November 2025

Source:	Nokia

Title:	Discussion on requirements for CJTC-C

Agenda item:	6.12.2.2

Document for:	Discussion

Introduction

In the last RAN4 meeting [1], the following agreements were made

Issue 1-1: Accuracy requirements for delay and frequency offset reporting for CJT calibration Way forward: Alternatives for further discussion: Alt 1: Define one set of [tighten] accuracy requirements based on SNR >= 0 or 3dB Encourage further input on the exact performance requirement Note: “tighten” means it is tighten compared to -3dB side condition Alt 2: Define two sets of requirements with UE capability Encourage input on details of the UE capability Issue 1-2: Test scope for UE reporting enhancement for CJT calibration? Agreement: - Define 2 TCs - TC1: FR1 CJTC-Dd measurement period and accuracy - TC2: FR1 CJTC-F measurement period and accuracy

Discussion

As discussed in our previous papers [2] and [3], it is important that accuracy requirements cover low SNR conditions. As highlighted in [2], the CJT gains are not so significant at cell center as when compared to cell edge. Therefore, in order to take advantage of the full benefit of CJTC, RAN4 should evaluate the test cases at low SNR conditions.

Simulations show that CJT provides better gains in cell edge than cell center.

Define one set of tighten accuracy requirements based on SNR = 0.

There is an important issue that could be discussed as part of the tests for CJT calibration, and this is the impact of invalid measurements in the test. The invalid conditions are defined as a possible reporting state for CFO and PO, however the RAN 1 agreements did not include a definition of the invalid state. UE is not configured in anyway to detect such invalid states, therefore the decision making associated with invalid state reporting is left to implementation of the device. The latter may be a problem during the test because if different UE devices under test, report invalid measurements in different amounts and by unknown reasons, the number of valid measurements out of the total of measurements is affected by that, and that could bias the test statistics.

The TC should be designed to be able measure reporting accuracy and also include both valid and invalid measurements in specified proportions, allowing verification of the UE's ability to identify valid and invalid results. Thus, the UE must detect measurement validity based on defined conditions.

One option is to add a test mode that lets testing equipment set conditions for valid or invalid UE measurements, or have the UE report the thresholds it uses. This allows tests to cover specific sequences of valid and invalid measurements. However, adding a test mode now is challenging due to limited time remaining before the final RAN 4 meetings.

Another plausible approach is to specify conditions in the documentation that identify invalid states. For example, if the SNR of a received reference signal falls below -3 dB (the minimum acceptable value), the UE may report an invalid measurement. Additionally, negative values should be avoided where quantization does not support them; for instance, if the UE selects an anchor TRP not based on minimal FO, it cannot report other TRPs with lower FO using negative values.

For DO and FO reporting, out-of-range measurements should not be considered invalid. An out-of-range result does not indicate an incorrect measurement; rather, it shows that the quantization range is insufficient to represent the value. In such cases, instead of indicating an invalid measurement, the UE should assign out-of-range values to the last (for DO) or penultimate (for FO) quantization state, which correspond to the highest value within the quantization range. Thus, this step represents the upper limit for valid quantized measurements that exceed the established range.

Out-of-range is not a condition for reporting an invalid measurement.

The UE may report invalid FO or PO measurement value if the TRS measurement SNR < -3 dB

RAN1 has captured the reporting values for DO reporting as [38.214]:

##### 5.2.1.4.9 CJTC-Dd reporting of delay offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-Dd', the interval [δin,δin+1) in which the delay offset, Dn,offset, falls into, for CSI-RS resource set n=0,…,NTRP-1, n≠nref, is indicated by iDO=i0⋯iNTRP-1 in∈{0,1,…,MD-1} where the mapping from in to δin is given by δin=in⋅ADMD-1 and δMD=∞, and the values of AD∈{CP/2, CP}, where CP is the length of the cyclic prefix for symbols other than 0 and 7⋅2μ in the subframes associated with the NTRP configured CSI-RS resource sets for tracking, and MD∈{32, 64, 128, 256} are configured by the higher layer parameters valueOfAD and valueOfMD, respectively. The index value in=MD-1, corresponding to the interval [AD,∞), represents an out-of-range delay offset Dn,offset.

The reporting mapping table for CJTC-Dd reported values should follow RAN1 agreement as specification defines exactly how the measured delay offset values are quantised into reportable indices. Specification defines how each index value 𝑖n corresponds to a specific range of measured delay offset with the mapping determined by the parameters 𝐴𝐷 (last quantization step boundary) and 𝑀𝐷 (number of quantisation levels). Specification defines index values from 0 to 𝑀𝐷−2 each to be representing a finite interval of delay offsets measurements, while the highest index value 𝑖𝑛=𝑀𝐷−1 is reserved for out-of-range values, specifically when measured delay offset is greater than 𝐴𝐷 configured by the higher layer parameters valueOfAD.

For delay offset, the quantisation boundaries are defined directly in the specification, which means these boundaries can be immediately adopted into RAN4 reporting mapping tables without further modification.

RAN4 to adopt the CJTC reporting mapping table for delay offset, where AD defines the quantization step size, configured by the higher layer parameter valueOfAD, and 𝑀D (MD) is the number of possible reported values, configured by the higher layer parameter valueOfMD as

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC-Dd_0 | 0 ≤ CJTC-Dd <ADMD-1 | Fraction of CP |
| CJTC-Dd_1 | ADMD-1≤ CJTC-Dd <2 ADMD-1 | Fraction of CP |
| CJTC-Dd_2 | 2 ADMD-1≤ CJTC-Dd <3 ADMD-1 | Fraction of CP |
| … | … | Fraction of CP |
| CJTC-Dd_{MD-2} | MD-2 ADMD-1≤ CJTC-Dd <AD | Fraction of CP |
| CJTC-Dd_{MD-1} | AD≤ CJTC-Dd | Fraction of CP |

RAN1 has captured the reporting values for FO reporting as in [38.214]:

##### 5.2.1.4.10 CJTC-F reporting of frequency offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-F', the frequency offset value, FOn , for CSI-RS resource set n=0,…,NTRP-1, n≠nref, is indicated by kFO=k0⋯kNTRP-1 kn∈{0,1,…,MFO-1} where the mapping from kn to FOn is given by FOn=kn⋅AFOMFO-1,kn=0,1,…,MFO-2'invalid',kn=MFO-1 and the values of AFO∈{0.1ppm, 0.2ppm} and MFO∈{16, 32, 256} are configured by the higher layer parameters valueOfAFO and valueOfMFO, respectively.

For the FO reporting for CJTC RAN1 defined the reporting of frequency offset values with different approach compared to delay offset reporting. Frequency offset is introduced in the specification as a set of reported values rather than explicit quantisation boundaries. This means that RAN4 is required to establish new quantisation boundaries when adopting the reporting mapping tables for frequency offset, ensuring that each reported value corresponds to a clearly defined measurement range. Furthermore, frequency offset reporting includes support for an ‘invalid’ value, which must also be represented clearly in the CJTC reporting mapping table.

For frequency offset (FO) reporting, the quantisation boundaries should be set between the reporting values defined by Ran1. This approach ensures that reported value is always the closest one to the actual measured quantity value.

RAN4 to adopt the CJTC reporting mapping table for frequency offset, where AFO defines the quantization step size, configured by the higher layer parameter valueOfAFO, and 𝑀FO (MFO) is the number of possible reported values, configured by the higher layer parameter valueOfMFO as

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC-F_0 | 0 ≤ CJTC-F <AFO2MFO-1 | ppm |
| CJTC-F_1 | AFO2MFO-1≤ CJTC-F <3 AFO2MFO-1 | ppm |
| CJTC-F_2 | 3 AFO2MFO-1≤ CJTC-F <5 AFO2MFO-1 | ppm |
| … | … | ppm |
| CJTC-F_{MFO-2} Note 1 | 2 MFO-5AFO2MFO-1≤ CJTC-F | ppm |
| CJTC-F_{MFO-1} Note 2 | ‘Invalid’ | NA |
| Note 1: CJTC-F_{MFO-2} is also used to report measurements above AFO i.e. out of range. Note 2: CJTC-F_{MFO-1} excludes report indication of out of range, where the UE may report invalid value if TRS measurement SNR is such that SNR < -3 dB. |  |  |

RAN1 has captured the reporting values for PO reporting as [38.214]:

##### 5.2.1.4.11 CJTC-P reporting of phase offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-P', if subbandSize is set to 'wideband', the phase offset value, Φn , for CSI-RS resource n=0,…,NTRP-1, n≠nref, is indicated by pΦ=p0⋯pNTRP-1 pn∈{0,1,…,MΦ-1} otherwise, the phase offset values, {Φn,s}, for the NSB-P configured subbands s=0,1,…,NSB-P-1 and CSI-RS resource n=0,…,NTRP-1, n≠nref, is indicated by pΦ=p0,0⋯p0,NSB-P-1⋯pNTRP-1,0⋯pNTRP-1,NSB-P-1 pn,s∈{0,1,…,MΦ-1} where the mapping from pn to Φn is given by Φn=pn⋅2πMΦ-1,pn=0,1,…,MΦ-2'invalid',pn=MΦ-1 the mapping from pn,s to Φn,s is given by Φn,s=pn,s⋅2πMΦ-1,pn,s=0,1,…,MΦ-2'invalid',pn,s=MΦ-1 and the value of MΦ∈{16, 32} is configured by the higher layer parameter valueOfMPhi.

For the phase offset reporting RAN1 has agreed that invalid values should be explicitly represented by reserving the highest index (𝑝𝑛=𝑀Φ−1) as 'invalid'. Since phase is inherently a circular quantity (unlike frequency or delay), meaning it wraps around at 2𝜋 and cannot exceed this range, there is no need for an unbounded (“out-of-range”) quantization step.

For PO per subband reporting, it is important to note that TRSs may be received with low SNR on certain subcarriers due to fading dips. In such cases, only those specific subcarriers should be marked as ‘invalid’, rather than invalidating the entire report. This targeted approach will ensure that the quantisation and reporting remain accurate for the valid subbands, while appropriately handling instances where TRS power is insufficient for reliable measurement.

In our view to capture the RAN1 agreement, the boundaries between reported values in the mapping table should be positioned midway between adjacent measured quantities. This approach minimises the distance between a measured phase offset and its reported value, improving the accuracy of quantisation and reducing potential reporting errors.

RAN4 to adopt the CJTC reporting mapping table for phase offset, where 𝑀Φ (MPhi) is the number of possible reported values, configured by the higher layer parameter valueOfMPhi as

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC-P_0 | 2MΦ-3πMΦ-1 ≤ CJTC-P <πMΦ-1 or 0 ≤ CJTC-P <3πMΦ-1 | rad |
| CJTC-P_1 | 3πMΦ-1≤ CJTC-P <5πMΦ-1 | rad |
| CJTC-P_2 | 5πMΦ-1≤ CJTC-P <7πMΦ-1 | rad |
| … | … | rad |
| CJTC-P_K Note 1 | 2MΦ-5πMΦ-1≤ CJTC-P<2MΦ-3πMΦ-1 | rad |
| CJTC-P_L Note 1 | ‘Invalid’ | NA |
| Note 1: K = Mφ-2 and L = Mφ-1 |  |  |
| CJTC-P_{MPhi-2} Note 1 | 2MΦ-5πMΦ-1≤ CJTC-P<2MΦ-3πMΦ-1 | rad |
| CJTC-P_{MPhi-1} Note 1 | ‘Invalid’ | NA |
| Note 1: the UE may report invalid value if TRS measurement SNR is such that SNR < -3 dB |  |  |

Simulation results

The simulation results presented here are based on the simulation results presented in our previous papers, For a detailed explanation of the simulation setup, assumptions, and baseline results, readers are referred to the earlier submission [3] and [4].

Table 1. Simulation assumptions for the FO and DO reporting for CJTC simulations.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz |
| SCS | 15kHz, 30kHz |
| Channel Model | TDLC300-10 |
| TRP number | 2 |
| UE antenna configuration | 1x2 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | 4 TRSs per TRP |
| CSI-RS periodicity | 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |
| SNR | -3dB, 0dB, 3dB |
| FO reporting parameters |  |
| Frequency offset between TRPs | 0.1 ppm |
| AFO | 0.2 ppm |
| DO reporting parameters |  |
| Time offset between TRPs | CP/2 |
| AD | CP |

Frequency offset reporting

In the previous paper [4] to verify potential causes for the differences on the simulation results, we presented 2 algorithm options that can be used for FO estimation at the UE side for CJTC reporting.

In the approach used in this paper, the received OFDM signal is processed by one of the estimation methods (Algorithm 1 or Algorithm 2), which estimates the frequency offset FOTRPn for each TRP. The error is defined as difference between true and quantized estimated relative frequency offsets FOTRP1-FOTRP2-quantFOTRP1-FOTRP2.

Table 2 Maximum between 5th and 95th percentile of  CJTC FO estimation error for different frequency offset estimation algorithms

|  |  | Algorithm 1 – correlation based |  | Algorithm 2 – phase ramping |  |
| --- | --- | --- | --- | --- | --- |
| SCS | SNR (dB) | Slow (1401 levels) | Fast (71 levels) | Slow | fast |
| 15KHz | -3 | 29 | 33 | 69 | 92.5 |
|  | 0 | 16 | 18 | 27 | 48 |
|  | 3 | 11 | 11 | 16 | 27 |
| 30KHz | -3 | 55 | 55 | 98 | 127 |
|  | 0 | 32 | 33 | 50 | 87 |
|  | 3 | 20 | 18 | 29 | 49 |

In the results of Table 2 it can be observed that Algorithm 1 presents a very similar performance, even when the number of correlation vectors is reduced from 1401 levels to 71 levels.

On the other hand, algorithm 2 (based on phase ramp calculation) presents a more significant accuracy difference when comparing the fast algorithm, i.e. comparison of consecutive TRS symbols, in relation to the slot one, i.e. comparison of all TRS symbol combination. In the case of algorithm 2, accuracy error of the slow option is at least 1.6 to 2 times larger than the error of Algorithm 1.

Considering that performance difference, it is advisable for companies to evaluate and implement one version of Algorithm 1 in their own simulations.

RAN4 to define CJT-C accuracy based on correlation algorithm described in R4-2514261.

Delay offset reporting

LLS results for the delay offset reporting are provided bellow, more details are available in our previous paper [3].

Table 3: Delay offset estimation error.

| SCS | SNR (dB) | Delay offset (DO) b/w two TRPs | # samples | DO estimation error for AD = CP and MD = 256 |
| --- | --- | --- | --- | --- |
|  |  |  |  | Max (CDF5%, CDF95%) |
| 15KHz | -3 | 0.5CP | 1 | 8 |
|  | 0 |  |  | 8 |
|  | 3 |  |  | 5 |
| 30KHz | -3 | 0.5CP | 1 | 8 |
|  | 0 |  |  | 8 |
|  | 3 |  |  | 8 |

Conclusion

The following Observations and Proposals were made:

Observation 1: Simulations show that CJT provides better gains in cell edge than cell center.

Proposal 1: Define one set of tighten accuracy requirements based on SNR = 0.

Observation 2: Out-of-range is not a condition for reporting an invalid measurement.

Proposal 2: The UE may report invalid FO or PO measurement value if the TRS measurement SNR < -3 dB

Observation 3: For delay offset, the quantisation boundaries are defined directly in the specification, which means these boundaries can be immediately adopted into RAN4 reporting mapping tables without further modification.

Proposal 3: RAN4 to adopt the CJTC reporting mapping table for delay offset, where AD defines the quantization step size, configured by the higher layer parameter valueOfAD, and 𝑀D (MD) is the number of possible reported values, configured by the higher layer parameter valueOfMD as

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC-Dd_0 | 0 ≤ CJTC-Dd <ADMD-1 | Fraction of CP |
| CJTC-Dd_1 | ADMD-1≤ CJTC-Dd <2 ADMD-1 | Fraction of CP |
| CJTC-Dd_2 | 2 ADMD-1≤ CJTC-Dd <3 ADMD-1 | Fraction of CP |
| … | … | Fraction of CP |
| CJTC-Dd_{MD-2} | MD-2 ADMD-1≤ CJTC-Dd <AD | Fraction of CP |
| CJTC-Dd_{MD-1} | AD≤ CJTC-Dd | Fraction of CP |

Proposal 4: RAN4 to adopt the CJTC reporting mapping table for frequency offset, where AFO defines the quantization step size, configured by the higher layer parameter valueOfAFO, and 𝑀FO (MFO) is the number of possible reported values, configured by the higher layer parameter valueOfMFO as

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC-F_0 | 0 ≤ CJTC-F <AFO2MFO-1 | ppm |
| CJTC-F_1 | AFO2MFO-1≤ CJTC-F <3 AFO2MFO-1 | ppm |
| CJTC-F_2 | 3 AFO2MFO-1≤ CJTC-F <5 AFO2MFO-1 | ppm |
| … | … | ppm |
| CJTC-F_{MFO-2} Note 1 | 2 MFO-5AFO2MFO-1≤ CJTC-F | ppm |
| CJTC-F_{MFO-1} Note 2 | ‘Invalid’ | NA |
| Note 1: CJTC-F_{MFO-2} is also used to report measurements above AFO i.e. out of range. Note 2: CJTC-F_{MFO-1} excludes report indication of out of range, where the UE may report invalid value if TRS measurement SNR is such that SNR < -3 dB. |  |  |

Proposal 5: RAN4 to adopt the CJTC reporting mapping table for phase offset, where 𝑀Φ (MPhi) is the number of possible reported values, configured by the higher layer parameter valueOfMPhi as

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC-P_0 | 2MΦ-3πMΦ-1 ≤ CJTC-P <πMΦ-1 or 0 ≤ CJTC-P <3πMΦ-1 | rad |
| CJTC-P_1 | 3πMΦ-1≤ CJTC-P <5πMΦ-1 | rad |
| CJTC-P_2 | 5πMΦ-1≤ CJTC-P <7πMΦ-1 | rad |
| … | … | rad |
| CJTC-P_K Note 1 | 2MΦ-5πMΦ-1≤ CJTC-P<2MΦ-3πMΦ-1 | rad |
| CJTC-P_L Note 1 | ‘Invalid’ | NA |
| Note 1: K = Mφ-2 and L = Mφ-1 |  |  |
| CJTC-P_{MPhi-2} Note 1 | 2MΦ-5πMΦ-1≤ CJTC-P<2MΦ-3πMΦ-1 | rad |
| CJTC-P_{MPhi-1} Note 1 | ‘Invalid’ | NA |
| Note 1: the UE may report invalid value if TRS measurement SNR is such that SNR < -3 dB |  |  |

Proposal 6: RAN4 to define CJT-C accuracy based on correlation algorithm described in R4-2514261.

- References

- R4-2514804, WF on NR_MIMO_Ph5_RRM_Part2, MediaTek inc., Prague, Czech, 13 – 17 October 2025

- R4-2507386, Other RRM requirements for MIMO ph5, Nokia, 3GPP TSG-RAN WG4 Meeting #115, Malta, Malta, 19 – 23 May 2025.

- R4-2510692, Discussion on other RRM requirements for MIMO ph5, Nokia, TSG-RAN WG4 Meeting #116, Bengaluru, India, 25 – 29 August 2025.

- R4-2514261, 3GPP TSG-RAN WG4 Meeting #116 bis, Prague, Czech Republic, 13 – 17 October 2025.

- 3GPP TS 38.214 V19.1.0 (2025-09), NR; Physical layer procedures for data (Release 19).