3GPP TSG RAN WG4 Meeting #117		R4-2522104

Dallas, US, 17th – 21st November 2025

Agenda item:	13.1.2

Source:	Terrestar Solutions, Viasat

Title:	Motivation for proposed Basket WI and non-Basket WI for NTN bands

Release:	Rel-20

Document for:	Discussion

# 1	Introduction

This document aims to outline the scope of the basket WI for NTN.

Release-19 introduced NTN features such as HPUE (PC1 and  PC2), 3MHz bandwidth, RedCap and eRedCap UE, with spectrum related aspects defined only for example NTN bands or for pre-Release-19 defined bands.

NTN bands introduced in Release-19 or not included among the example bands do not support these features.

In accordance with the RAN4 process for creating Basket WI to address band-specific requirements, a new basket WI is proposed for Release-20 to enable these features for additional bands based on operator’s interest.

Additionally, 7 MHz channel BW was introduced in Release-19 for TN for certain bands which couldn’t be fully utilized with 3/5/10/15/20/… MHz channel bandwidths. There is a similar need for specifying a 7 MHz channel for certain NR NTN bands to facilitate full utilization of the satellite spectrum.

This paper presents a proposal for one NTN basket WI and one non spectrum WI.

# 2	Discussion

In Release 19, 3GPP RAN WG4 completed the work item “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” [1] introducing support for high-power user equipment classes—specifically PC2 (26 dBm) and PC1 (31 dBm)—across both NR and IoT NTN bands. While core RF band-agnostic requirements were defined, only a limited set of exemplary bands received additional band-specific requirements, namely L-band n255 and b255, and S-band n256 and b256.

Several other NR NTN/IoT NTN bands, including n250, n251, n253/b253, b249, n252/b252, and n254/b254, were discussed during Release 19 but lacked finalized band-specific RF requirements for high-power operation. Currently, these bands only support PC3, and further specification work is needed to enable PC1 and PC2 support.

The same work item [1] introduced support for a 3 MHz channel bandwidth, limited to pre-Release 19 bands. Specifically, 3 MHz channel support for NTN was defined only for bands n254, n255, and n256. Bands n250, n251, n252, and n253, which were introduced in Release 19, currently lack support for 3 MHz bandwidth.

In Release 19, 3GPP RAN WG4 also completed the work item “Non-Terrestrial Networks (NTN) for NR Phase 3” [3]  introducing support for Release 17 RedCap and Release 18 eRedCap UEs with NR NTN operating in FR1-NTN bands, and only bands already specified prior to Release 19 were completed. NR NTN bands n250, n251, n252, and n253 lack support for Release 17 RedCap and Release 18 eRedCap.

Thus, we propose to initiate a single basket work item to address these gaps by enabling high-power classes PC1 and PC2 for NR NTN/IoT NTN bands n250, n251, n253/b253, b249, n252/b252, and n254/b254, as well as 3 MHz channel bandwidth and RedCap/(e)RedCap support for NR NTN bands n250, n251, n252, and n253.

Finally, 7 MHz channel BW was introduced for TN in Rel-19 [3]. Introducing 7 MHz channel BW will allow n250/n253 operators to fully utilize their uplink spectrum which is 7 MHz wide (shown in figure below). Existing channel bandwidth can result in up to 2 MHz of spectrum remaining unused despite being authorized for MSS service by the regulators.

Figure: DL and UL frequency ranges for n250 and n253

Based on experience in specifying 3 MHz channel bandwidth, the incremental effort is mostly related to the regulatory emissions requirements and associated spectrum related Tx requirements (additional emissions, A-MPR). The non-spectrum aspects generally mirror the work performed on the TN side.

However, since 7 MHz channel bandwidth has not been specified for any NTN bands, to adhere to the RAN4 process, instead of a basket WID, a new non-spectrum WID is proposed.

Thus, we propose initiating a non-spectrum Rel-20 WI to enable support for 7 MHz channel bandwidth in bands n250 and n253.

Proposal 1:	Initiate a new Rel-20 spectrum basket WI to provide support for:

- 3MHz channel H in NR NTN bands: n250, n251, n252, n253.

- Higher power classes for the NR NTN bands n250, n251, n252, n253 and n254

- Higher power classes for the NTN IoT bands b249, b253, b252, b254:

- RedCap and (e)RedCap in NR NTN bands: n250, n251, n252, n253

Proposal 2:	Initiate a new Rel-20 non-spectrum WID to enable support of 7 MHz channel BW for NR NTN bands n250 and n253.

# 3 References

[1] RP-252076 – Rel-19 WID Enhanced requirements and conductive test methodology for NR NTN and IoT NTN

[2] RP-251954 – Rel-19 WID Non-Terrestrial Networks (NTN) for NR Phase 3

[3] RP-251453 – Rel-19 WID for NR FR1 7 MHz Channel Bandwidth