3GPP TSG RAN WG4 Meeting #117	R4-2522105

Dallas, US, November 17 - 21, 2025

Title: 	Considerations for (e)Redcap UEs with 3MHz CBW

Agenda item:	7.4.3.1

Source:	Huawei, HiSilicon

Document for:	Discussion

# Introduction

In the last meeting the below WF[1] was agreed. In this tdoc the remaining issues are addressed.

# Release independent and UE capability Issue 4-1: Whether 3MHz UE CBW should be optional or mandatory in Rel-20 for (e)Redcap FFS whether 3MHz should be optional or mandatory in Rel-20 for (e)Redcap Issue 4-2: Release independent of 3MHz UE CBW for (e)Redcap It is agreed 3MHz UE CBW is release independent from Rel-18 for (e)RedCap UE. Issue 4-3: UE capability FFS in next meeting what UE capabilities of (e)Redcap need to be updated to support 3MHz and sent LS to RAN2.

# Discussion

3MHz UE CBW should be optional or mandatory in Rel-20 for (e)Redcap

On other UE types 3MHz is optional for all supporting bands but n106, because 3MHz requires chip design modifications and it should remain up to chip vendors to decide on whether to support it or not. The case of n106 was an exception as the operator only had 3 MHz in that band and it had no other spectrum for operation.

Proposal 1: 3MHz shall be optional for (e)Redcap UEs supporting n100, n26, n28, n85 and mandatory for n106, on Rel-20.

UE capabilities

In Rel-18 there was a restriction on using 3MHz CBW for (e)RedCap UEs and CA operations. In Rel-19, this restriction was lifted for CA operation using [2]. Currently the following phrase is mentioned on the below listed UE capabilities on TS 38.306, “This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18.”

- support12PRB-CORESET0-r18

- support3MHz-ChannelBW-Asymmetric-r18

- support3MHz-ChannelBW-Symmetric-r18

- support5MHz-ChannelBW-20PRB-CORESET0-r18

- support12PRB-CORESET0-GSCN-41637-r18

Proposal 2: Send an LS to RAN2 to update the following UE capabilities and remove Redcap restrictions:

- support12PRB-CORESET0-r18

- support3MHz-ChannelBW-Asymmetric-r18

- support3MHz-ChannelBW-Symmetric-r18

- support5MHz-ChannelBW-20PRB-CORESET0-r18

- support12PRB-CORESET0-GSCN-41637-r18

# Conclusions

Proposal 1: 3MHz shall be optional for (e)Redcap UEs supporting n100, n26, n28, n85 and mandatory for n106, on Rel-20.

Proposal 2: Send an LS to RAN2 to update the following UE capabilities and remove Redcap restrictions:

- support12PRB-CORESET0-r18

- support3MHz-ChannelBW-Asymmetric-r18

- support3MHz-ChannelBW-Symmetric-r18

- support5MHz-ChannelBW-20PRB-CORESET0-r18

- support12PRB-CORESET0-GSCN-41637-r18

# Reference

[1]	“R4-2515101,WF for [116bis][323] NR_less_than_5MHz_RedCap_UERF, Sony.”

[2]	“R4-2417119, LS on NR channel BW less than 5MHz, Intel.”