3GPP TSG RAN WG4 Meeting #117	R4-2522107

Dallas, US, November 17 - 21, 2025

Title: 	Introduction of US 4.9GHz band RF requirements

Agenda item:	5.11.4

Source:	Huawei, HiSilicon

Document for:	Discussion

# Introduction

In the last meeting the below WF[1] was agreed. In this meeting the FCC mask and how to consider it in 3GPP evaluations will be considered.

- Only FCC “high power device” is considered for 3GPP UE, and two requirements below need to be satisfied. - Peak power spectral density: 21 dBm per one MHz. - High power maximum conducted output power: Channel bandwidth (MHz); High power maximum conducted output power (dBm). 10; 30.0. 15; 31.8. 20; 33.0. 25; 34.0. 30; 34.8. 40; 36.0. 50; 37.0. <Way forward>: Maximum output power tolerance - FFS impact of maximum output power tolerance on maximum output power requirements above. ## <Sub-topic 4-2> Additional requirement for FCC mask M applicable to FCC “high power device” <Agreement>: Resolution bandwidth - Adopt a resolution bandwidth for the FCC part 90 mask to 1% of the configured UE channel bandwidth. - Spec wording is FFS. <Agreement>: Zero dB reference for applicable FCC Emission mask M - The zero dB reference for FCC Emission mask M is calculated based on UE power class maximum output power and the configured UE channel bandwidth. - The zero dB reference is used for mask location and for A-MPR analysis. . <Agreement>: New NS, Impacted frequency range for new NS, Other requirements for new NS - Specify a new NS for the US 4.9GHz band with FCC part 90 including: - The Mask M requirement (note this covers the 5% channel bandwidth edge (inside), the SEM domain and the spurious domain). - Maximum output power limitation in Sub-topic 4-1. <Agreement>: For the next meeting, companies are invited to provide the simulation assumption in order to have a unified understanding of assumption when the simulation campaign starts.

# Discussion

## FCC Mask

FCC has established a mask that has impact on in-band emissions and out-of-band emissions, in contrast to 3GPP that its mask is applied only to out-of-band emissions. Therefore, correct understanding of FCC mask is essential before performing AMPR evaluations properly. Due to the FCC mask some precautions need to be taken into account in resource allocations:

Precautions due to FCC mask M:

The first region of the Mask M starts on the first and last 5% of the CBW (inside the CBW). As shown in Figure 1 the 5% of edge allocated RBs (6 RBs) are impacted by this strict mask. Moreover, it is not a fixed mask (the PSD is not fixed). Therefore, these RBs would require adaptive power emissions, which is not part of the 3GPP requirements. Therefore it is better to avoid RB allocation on the first and last 5% of the CBW.

Figure 1 5% edge RB allocations hitting the FCC mask M

Proposal 1: To respect the FCC Mask M 5% emission of the CBW, and to avoid adaptative power emission on edge RBs (which is not 3GPP compliant), no RB should be allocated on the first and last 5% of the CBW.

Precautions due to FCC in-band emission limits:

High power maximum conducted output power limits provided in the table of the WF does not have any impact on PC3 UEs. On the other hand, the peak power spectral density of 21dBm/MHz can have impact on the emission power. Figure 2 depicts the scenario where with RBstart = 6 (6 was chosen to avoid the FCC mask-M ) and LCRB =1, one can observe that the PSD raises up to 24 dB which would require some power back-off. On the other hand, starting from LCRB= 11 the PSD falls below the 21dBm/MHz, as shown in Figure 3.

Proposal 2: Consider MPR due to PSD limit at 21dBm/MHz for the following RB allocations. Other allocations can be subject to MPR due to FCC mask-M, but they will not be impacted by the 21dBm/MHz PSD limit.

| SCS (KHz) | LCRB |
| --- | --- |
| 15 | <11 |
| 30 | <6 |
| 60 | <3 |

Figure 2  With 1RB allocation the PSD of the transmitter, hits the PSD limit and power back of would be needed.

Figure 3 Starting LCRB= 11 (SCS=15KHz) the PSD limit is respected.

# Conclusions

Proposal 1: To respect the FCC Mask M 5% emission of the CBW, and to avoid adaptative power emission on edge RBs (which is not 3GPP compliant), no RB should be allocated on the first and last 5% of the CBW.

Proposal 2: Consider MPR due to PSD limit at 21dBm/MHz for the following RB allocations. Other allocations can be subject to MPR due to FCC mask-M, but they will not be impacted by the 21dBm/MHz PSD limit.

| SCS (KHz) | LCRB |
| --- | --- |
| 15 | <11 |
| 30 | <6 |
| 60 | <3 |

# Reference

[1]	“R4-2515103, Way Forward for [116bis][322] NR_TDD_4.9GHz_band, Fujitsu.”