3GPP TSG-RAN WG4 Meeting #117	R4-2522123

Dallas, US, November 17 – 21, 2025

Agenda Item:	6.10.2.1

Source:	MediaTek Inc.

Title:	Discussion on perf part for FR2-1 SSB based L3 measurement delay reduction by optimizing BSF

Document for:	Discussion

# Introduction

In this paper we discuss the performance part for R19 RRM phase 5 WI, specifically for Rx beam sweeping factor optimization for L3 measurement delay reduction. The progress in the last meeting of RAN4 is captured in the WF [1].

# Discussion

RAN4 previously agreed on the TC list to be defined for BSF optimization as shown below:

RAN4#115 agreement: Index; Scenario; Number of TC; Configurations and scopes; Company. 1; Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1; - Non-DRX - In TC1/2/3/4, UE is only required to pass TC1+TC4 or TC2+TC3.; CMCC. 3; Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; 1; CATT. 2; Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1; - With DRX, DRX.1 for sub-test1 and DRX.7 for sub-test2 (same as in table A.7.6.1.2.1-2) - In TC1/2/3/4, UE is only required to pass TC1+TC4, or TC2+TC3.; Huawei. 4; Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; 1; Samsung. 5; Scenario 5: Handover; 1; - Intra-frequency handover from FR2 to FR2 - Unknown target cell; OPPO. 1; - Inter-frequency handover from FR2 to FR2 - Unknown target cell. 6; Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection; 1; - Inter-frequency RRC Re-establishment; - Unknown target cell - FR2 to FR2; Intel. 1; - RRC Connection release with redirection - FR2 to FR2.

### Issue 1-1-1: Whether to test FBS quitting Further discuss this issue based on the options in the topic summary R4-2513511.

For issue 1-1-1, some companies proposed to test FBS quitting (i.e., after UE activate multi-Rx for fast beam sweeping). Quitting FBS can be due to 1) T1 timer of 90s expires, 2) UAI indication to switch to single-Rx, or 3) the conditions are not satisfied anymore after UE activate FBS. In any case, UE is not required to be tested whether FBS is deactivated since it is completely up to UE. In other words, UE is only required to be tested when UE is required to activate multi-Rx for FBS, not the other way around.

Proposal 1: Not need to test the quitting of FBS (i.e., after UE activate multi-Rx for fast beam sweeping).

Proposal 2: The TCs are only required to test when UE is required to apply FBS for L3 measurement delay reduction.

### Issue 1-1-2: Design of TCs for FBS Further discuss this issue based on the options in the topic summary R4-2513511. For information: - Test cases for FBS are defined by referring to the framework of legacy TCs for SSB based L3 measurements, i.e., Event A3 triggered measurement report used.

The TC for FBS can be designed using 2 cells (serving cell and neighbour cell). FBS triggering conditions can be set using two power levels of the serving cell and two time periods. During T1, the serving cell’s power can be adjusted to be higher than a threshold. At T2 the power level becomes lower than a threshold, which will trigger to activate FBS. Then the UE need to send L3 measurement report of the neighbour cell based on FBS within the time period of T2. According to the core requirements, a timer of 90s is activated during FBS. However, in the TC a shorter period can be used e.g., 1s or 0.5s.

Proposal 3: TC for FBS can be designed using

- Two cells (serving cell and neighbour cell)

- Two power levels of the serving cell to trigger FBS

Proposal 4: The length of test cases after FBS condition is met can be either 1s or 0.5s

# Summary

In this contribution, the following observations and proposals are concluded:

Proposal 1: Not need to test the quitting of FBS (i.e., after UE activate multi-Rx for fast beam sweeping).

Proposal 2: The TCs are only required to test when UE is required to apply FBS for L3 measurement delay reduction.

Proposal 3: TC for FBS can be designed using

- Two cells (serving cell and neighbour cell)

- Two power levels of the serving cell to trigger FBS

Proposal 4: The length of test cases after FBS condition is met can be either 1s or 0.5s

# References

- R4-2514801, WF on NR_RRM_Ph5_Part2, CATT.