3GPP TSG-RAN WG4 Meeting #117	R4-2522124

Dallas, US, November 17 – 21, 2025

Agenda Item:	6.12.2.2

Source:	MediaTek Inc.

Title: 	Discussion on the performance part for R19 MIMO for other RRM requirements

Document for:	Discussion

# Introduction

In this paper we discuss the performance part for R19 MIMO phase 5, specifically for part 2, which is related to other RRM requirements. In the last meeting, RAN4 discussed the performance requirements for CJT reporting without reaching a conclusion as capture in the WF [1]. Therefore, we provide our view on how to define the performance requirements for CJTC reporting in this paper.

# Discussion

# CJTC calibration reporting.

For CJT calibration, RAN4 narrowed down two alternatives in the last meeting on how to define the performance requirements for DO and FO reporting as shown below [2].

Issue 1-1: Accuracy requirements for delay and frequency offset reporting for CJT calibration Way forward: Alternatives for further discussion: Alt 1: Define one set of [tighten] accuracy requirements based on SNR >= 0 or 3dB Encourage further input on the exact performance requirement Note: “tighten” means it is tighten compared to -3dB side condition Alt 2: Define two sets of requirements with UE capability Encourage input on details of the UE capability

In our view, both alternatives have their own pros and cons. However, we prefer Alt1 since having two UE capabilities as suggested in Alt2 for performance requirements is not a typical scenario in RRM spec. Based on the simulation results provided by all companies in the last meeting in (R4-2512159), it is observed that the accuracy of frequency offset (FO) and delay offset (DO) is highly degraded when the side condition is -3dB. This degradation is also presented in terms of throughput performance in Nokia paper (R4-2510692) in the last meeting. Since from NW prospective it is significant to define high accuracy requirements for the usefulness of the feature, we propose to focus on higher SNR side condition equal to 3dB since it is challenging to meet acceptable accuracy lower SNR based on 1 sample. Given our simulation results in Appendix A, the accuracy for FO and DO based on 1 sample can be defined as shown in the Table 1 and 2, respectively (with added tolerance level). Based on our analysis with respect to the results in (R4-2510692), it seems that the accuracy level provided in the tables can still achieve >95% of the throughput performance for both FO and DO.

Table 1: FO accuracy

| Frequency offset |  |  |
| --- | --- | --- |
| Accuracy based on 1 sample | SNR | SCS |
| Level | dB | kHz |
| 30 | 3 | 15 |
| 55 | 3 | 30 |
| Note: accuracy level refers to quantization level based on the report resolution of MFO = 256 and AFO=0.2ppm |  |  |

Table 2: DO accuracy

| Delay offset |  |  |
| --- | --- | --- |
| Accuracy based on 1 sample | SNR | SCS |
| Level | dB | kHz |
| 14 | 3 | 15 |
| 18 | 3 | 30 |
| Note: accuracy level refers to quantization level based on the report resolution of MD = 256 and AD=CP |  |  |

Proposal 1: Define accuracy requirements for FO and DO for CJT calibration based on 3dB SNR.

Proposal 2: Accuracy requirements for FO and DO for CJT calibration can be defined as in the tables below:

Table 1: FO accuracy

| Frequency offset |  |  |
| --- | --- | --- |
| Accuracy based on 1 sample | SNR | SCS |
| Level | dB | kHz |
| 30 | 3 | 15 |
| 55 | 3 | 30 |
| Note: accuracy level refers to quantization level based on the report resolution of MFO = 256 and AFO=0.2ppm |  |  |

Table 2: DO accuracy

| Delay offset |  |  |
| --- | --- | --- |
| Accuracy based on 1 sample | SNR | SCS |
| Level | dB | kHz |
| 14 | 3 | 15 |
| 18 | 3 | 30 |
| Note: accuracy level refers to quantization level based on the report resolution of MD = 256 and AD=CP |  |  |

In previous discussion in RAN4, it was agreed to define report mapping tables for FO/DO for CJT calibration. Therefore, RAN4 need to discuss the details of the report mapping table. We propose to define the following tables for FO and DO report mapping.

Proposal 3: Report mapping table for frequency/delay/phase offset can be defined based on report resolution MFO = {16, 32, 256}, MD {32, 64, 128, 256} and Mph and {16, 32}.

# Summary

In this paper, the discussion of R19 MIMO is provided. The following observations and proposals are concluded:

Proposal 1: Define accuracy requirements for FO and DO for CJT calibration based on 3dB SNR.

Proposal 2: Accuracy requirements for FO and DO for CJT calibration can be defined as in the tables below:

Table 1: FO accuracy

| Frequency offset |  |  |
| --- | --- | --- |
| Accuracy based on 1 sample | SNR | SCS |
| Level | dB | kHz |
| 30 | 3 | 15 |
| 55 | 3 | 30 |
| Note: accuracy level refers to quantization level based on the report resolution of MFO = 256 and AFO=0.2ppm |  |  |

Table 2: DO accuracy

| Delay offset |  |  |
| --- | --- | --- |
| Accuracy based on 1 sample | SNR | SCS |
| Level | dB | kHz |
| 14 | 3 | 15 |
| 18 | 3 | 30 |
| Note: accuracy level refers to quantization level based on the report resolution of MD = 256 and AD=CP |  |  |

Proposal 3: Report mapping table for frequency/delay/phase offset can be defined based on report resolution MFO = {16, 32, 256}, MD {32, 64, 128, 256} and Mph and {16, 32}.

# Reference

- R4-2514804, WF on NR_MIMO_Ph5_RRM_Part2, MediaTek.

# Appendix A

Simulation assumption and results for CJT calibration:

Table 1. Simulation parameters for timing and frequency offset estimation

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Duplex, SCS | - FDD, 15kHz SCS - TDD, 30kHz SCS |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC300-10 |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm, 0.2ppm 2nd priority: 0.02 ppm, 0.05ppm |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples (TRS resource sets) for measurement | 1, 3, 5 2nd priority: 2, 4, 10 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) |
| CSI-RS periodicity (TRS) | 20ms / 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |

- Timing offset simulation results:

Table 2. Simulation result for timing offset error estimation.

| SCS | SNR | Timing offset (TO) b/w two TRPs | TO estimation error in AD = CP and MD = 256 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Max(CDF5%, CDF95%) #levels 256 |  |  |
|  |  |  | #samples |  |  |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 0.5CP | 29 | 19 | 18 |
| 15KHz | 0 | 0.5CP | 17 | 12 | 10 |
| 15KHz | 3 | 0.5CP | 10 | 7 | 6 |
| 30KHz | -3 | 0.5CP | 30 | 19 | 17 |
| 30KHz | 0 | 0.5CP | 16 | 11 | 10 |
| 30KHz | 3 | 0.5CP | 14 | 9 | 9 |

- Frequency offset simulation results:

Table 3. Simulation result for frequency offset error estimation.

| SCS | SNR | Frequency offset (FO) b/w two TRPs | FO estimation error in AFO = 0.2 ppm and MFO = 256 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Max(CDF5%, CDF95%) #levels 256 |  |  |
|  |  |  | #samples |  |  |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 350 Hz | 65 | 39 | 29 |
| 15KHz | 0 | 350 Hz | 39 | 24 | 18 |
| 15KHz | 3 | 350 Hz | 26 | 16 | 12 |
| 30KHz | -3 | 350 Hz | 133 | 71 | 60 |
| 30KHz | 0 | 350 Hz | 83 | 48 | 37 |
| 30KHz | 3 | 350 Hz | 52 | 31 | 24 |