3GPP TSG-RAN4 Meeting #117	R4-2522125

# Dallas, US, 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on TC#10 for FR1 CJTC-F measurement period and accuracy for R19 MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce a test case to test the measurement period and accuracy requirements defined for frequency offset reporting for CJT calibration in R19 MIMO-ph5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new test case A.6.6.X.2 is introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing frequency offset reporting for CJT calibration will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE 1----------------------------

### A.6.6.X	CJT calibration measurements and accuracy

…

#### A.6.6.X.2	CJTC-F measurement period and accuracy in FR1

##### A.6.6.X.2.1	Test Purpose and Environment

The purpose of the test is to verify that the CJTC-F measurement meets the requirements specified in clause 9.16 for measurement delay and clause 10.1.x for measurement accuracy in FR1 standalone scenario.

The supported test configurations are specified in table A.6.6.X.2.1-1.

Table A.6.6.X.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there is one active serving cell, the FR1 PCell (Cell 1), configured with two TRPs. The test parameters for the Cell 1 are given in table A.6.6.X.2.1-2 and table A.6.6.X.2.1-3 below.

Before the test starts, CSI measurement configuration is provided. The test consists of a single time period T1, during which the UE is triggered via DCI to report CJTC-F measurement results. After 80 ms from the beginning of the test, the DCI trigger comes in slot n (0 for Config 1 and 8 for Config 2) of a frame and UE provides the report back based on the reporting configuration as defined in table A.6.6.X.2.1-3.

Table A.6.6.X.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 | Cell 1 is the PCell |
| RF Channel Number |  | 1, 2 | 1 |  |
| BWchannel | MHz | 1 | 10: NPRB,c = 52 |  |
|  |  | 2 | 40: NPRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
| CP length |  | 1, 2 | Normal |  |
| DRX |  | 1, 2 | OFF |  |
| T1 | s | 1, 2 | [1] |  |

Table A.6.6.X.2.1-3: Cell specific test parameters

| Parameter | Config | Unit | Cell 1 |
| --- | --- | --- | --- |
| SSB ARFCN | 1~2 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.2.1 TDD |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration for TRP1 | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.2 TDD |
| TRS configuration for TRP2 | 1 |  | TRS.1.1 FDD (except k0 should be 1) |
|  | 2 |  | TRS.1.2 TDD (except k0 should be 1) |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.1 ULBWP.1.1 |
| reportConfigType | 1~2 |  | aperiodic |
| reportQuantity | 1~2 |  | CJTC-F |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Noc | 1 | dBm/15 kHz | -98 |
|  | 2 |  |  |
| Note 2 | 1 | dBm/SCS | -98 |
|  | 2 |  | -95 |
| TRS Note 3 | 1~2 | dB | 3 |
| TRS Note 3 | 1~2 | dB | 3 |
| TRS-RSRP Note 3 | 1 | dBm/SSB SCS | -95 |
|  | 2 |  | -92 |
| Io | 1 | dBm/9.36 MHz | -65.3 |
|  | 2 | dBm/38.16 MHz | -59.2 |
| Propagation condition | 1~2 |  | TDL-C 300 ns 10 Hz |
| Antenna configuration | 1~2 |  | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: The configuration applies to both TRP1 and TRP2 |  |  |  |

##### A.6.6.X.2.2	Test requirements

After 80ms from the beginning of the test, the UE shall send CJTC-F measurement report at the 8th slot from the reception of DCI triggering the CJTC-F measurement. The CJTC-F measurement accuracy shall fulfil the requirements specified in clause 10.1.x.

The rate of correct events observed during repeated tests shall be at least 90%.

----------------------END OF CHANGE 1----------------------------