3GPP TSG-RAN WG4 Meeting #117							R4-2522127

Dallas, US, 17th – 21st Nov 2025

Agenda Item:	8.9

Source:	MediaTek Inc.

Title:	Views on 6G spectrum sharing

Document for:	Discussion

# Introduction

Following the 6G SI [1] approved in RAN#108 and the first 6G discussion in the last RAN4 meeting as captured in the WF [2], in this paper, we provide our views on the 6G spectrum sharing with the focus on the following aspect:

- Inter-RAT mobility between 6GR and NR

# Inter-RAT mobility between 6GR and NR

In legacy systems, inter-RAT mobility usually refers to the process of measuring and managing the performance and quality of different radio access technologies within a mobile network. This is crucial for ensuring seamless connectivity and optimal performance as users move between areas covered by different technologies, such as 4G, 5G and later 6G. In addition, we learned in 4G and 5G that synchronization between different RATs is significantly important to ensure successful Inter-RAT measurements and mobility. If RATs are asynchronous, for example, if LTE NW does not know the timing information of the 5G NW, it not possible to configure an appropriate SMTC or measurement gap (MG) offsets for inter-RAT measurements. This means measurements in one RAT are likely to fail since SMTC or MG does not coincide with the transmission of the SSB from the other RAT, as shown in Figure 1.

Figure 1: Asynchronous Inter-RAT measurements

Observation 1: Without LTE/NR timing sync and without LTE NW knowing NR NW timing, appropriate SMTC and MG offsets for inter-RAT measurement cannot be configured correctly.

This issue was discussed in NR and for which a new solution called System Frame Time Difference (SFTD) was introduced in Rel-15, which allows requesting the UE to report the SFN and Frame Timing Difference between two RATs. However, such solution was never used or deployed in the real networks.

Observation 2: SFTD solution was introduced in Rel-15 but never used in real networks.

Therefore, to avoid repeating the legacy mistakes in 6G and spending time on designing complicated solutions which are not likely to be implemented in the real field, we propose that synchronization between RATs to be the baseline assumption in 6G, where timing information of the target cell’s RS can be provided to enable efficient inter-RAT measurement and mobility. In addition, this synchronization does not need to be perfect, in fact as long it can ensure the timing information of inter-RAT’s RS can occur within the measurement configuration provided by the serving RAT it can be sufficient.

Proposal 1: Providing correct timing information for Inter-RAT measurements without UE assistance (e.g., SFTD in legacy) should be the baseline for 6G to enable efficient inter-RAT mobility.

In the last meeting, RAN4 also discussed the following issues as captured in the WF [2]:

#### Issue 1-3-1: Inter-RAT handover interruption - Proposal from companies - Option 1 (CMCC): it is proposed to minimize handover interruption time for inter-RAT handover between NR and 6GR. - Way forward - Discuss whether and how to minimize handover interruption time for inter-RAT handover between NR and 6GR #### Issue 1-3-2: Inter-RAT measurement - Proposal from companies - Option 1 (CMCC): it is proposed to support inter-RAT measurements without gaps, including inter-RAT NR measurement without gap and inter-RAT 6GR measurement without gap, from 6G day-1. - Option 2 (Xiaomi): RAN4 shall study potential inter-RAT RRM measurement impact including w/o and with gap under MRSS scenario. - Way forward - Further discuss

The above two issues, i.e., inter-RAT handover interruption and inter-RAT measurement w/o and with gap, are related to the gapless and interruption discussion in 6G RRM agenda. The outcomes of measurement without gap/gapless and interruption discussion in 6G RRM will impact the discussion on inter-RAT measurements. Therefore, in our view, we should wait for the conclusion on those topics in 6G RRM discussion before making any decision here.

Proposal 2: Wait for 6G RRM discussion and conclusion on gapless and interruption aspects before making decisions on inter-RAT scenario.

# Summary

In this paper, we provide our views on 6G spectrum sharing with the following observations and proposals:

Observation 1: Without LTE/NR timing sync and without LTE NW knowing NR NW timing, appropriate SMTC and MG offsets for inter-RAT measurement cannot be configured correctly.

Observation 2: SFTD solution was introduced in Rel-15 but never used in real networks.

Proposal 1: Providing correct timing information for Inter-RAT measurements without UE assistance (e.g., SFTD in legacy) should be the baseline for 6G to enable efficient inter-RAT mobility.

Proposal 2: Wait for 6G RRM discussion and conclusion on gapless and interruption aspects before making decisions on inter-RAT scenario.

# References

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)

- [2] R4-2514646, WF for [116bis][108] 6G spectrum sharing, CMCC