3GPP TSG RAN WG4 Meeting #117	R4-2522133

Dallas, US, November 17 - 21, 2025

Title: 	PC3 A-MPR requirements for 6MHz channel bandwidth

Agenda item:	7.1.5.2

Source:	Huawei, HiSilicon

Document for:	Discussion

# Introduction

In the RAN4 116bis meeting the WF was agreed, where the following configurations were considered for A-MPR evaluations:

## 2.4 NS_06, NS_14, NS_15, NS_43/43U Agreement: - Conduct A-MPR evaluation (if needed) for - NS_06 for n12 (PC3) - For information: Current spec states “For power class 3 operation on bands n12, n13, n14, n85 and n110, no A-MPR is applicable. - NS_06 for n85 (PC3 and PC2) - For information: Current spec states “For power class 3 operation on bands n12, n13, n14, n85 and n110, no A-MPR is applicable. - NS_14 for n5 (PC3 and PC2): A-MPR evaluation is not needed for 6MHz leveraging the experience of 7 MHz - NS_15 for n5 (PC3 and PC2)

In this document we are presenting our PC3 A-MPR evaluations using these configurations

# Discussion

The PA was calibrated based on the following assumptions:

- PC3

- 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc, Image: 28dBc, CIM3: 60dBc, CIM5: 70dBc

- ASE, SEM and ACLR are considered as gating factors (EVM was excluded).

In our plots the back off values below MPR level are in gray, as no A-MPR needed for them.

## NS_15

The following requirements should be considered by the UE when BS issues NS_15:

As an example, the PC3 A-MPR evaluations for QPSK with 6 MHz at 846 MHz is shown in Figure 3, the remaining evaluations can be found in the Annex.

Figure 3 NS_15 PC3  power back off for 6MHz CBW with QPSK and Fc= 846 MHz

Proposal 1: Consider the following A-MPR regions and values for NS_15 with 6MHz CBW.

Table 1: A-MPR regions for NS_15 (Power Class 3)

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 6MHz | 843 < Fc ≤ 846 | ≥4.5 | ≥ 0 | A7 |
|  |  | ≥3.06, ≤4.5 | ≥1.44 | A2 |
|  | 840 < Fc ≤ 843 | ≥4.68 | ≥2.88 | A9 |
|  |  | ≥5.4 | =0.180 | A9 |

Table 6.2.3.24-2: A-MPR for NS_15 (Power Class 3)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner |  |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 | ≤ 8.5 | ≤ 5 | ≤ 4 | ≤ 2 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 | ≤ 10 | ≤ 6.5 | ≤ 4.5 | ≤ 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 | ≤ 10 | ≤ 6.5 | ≤ 4.5 | ≤ 2.5 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  | ≤ 10 | ≤ 6.5 | ≤ 4.5 | ≤ 2.5 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  | ≤ 10 | ≤ 9 | ≤ 5 | ≤ 3.5 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | ≤ 11 | ≤ 6.5 | ≤ 6 | ≤ 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | ≤ 11 | ≤ 6.5 | ≤ 6 | ≤ 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  | ≤ 11 | ≤ 6.5 | ≤ 6 | ≤ 4 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  | ≤ 11 | ≤ 9.5 | ≤ 6.5 | ≤ 5 |

# Conclusion

Proposal 1: Consider the following A-MPR regions and values for NS_15 with 6MHz CBW.

Table 1: A-MPR regions for NS_15 (Power Class 3)

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 6MHz | 843 < Fc ≤ 846 | ≥4.5 | ≥ 0 | A7 |
|  |  | ≥3.06, ≤4.5 | ≥1.44 | A2 |
|  | 840 < Fc ≤ 843 | ≥4.68 | ≥2.88 | A9 |
|  |  | ≥5.4 | =0.180 | A9 |

Table 6.2.3.24-2: A-MPR for NS_15 (Power Class 3)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner |  |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 | ≤ 8.5 | ≤ 5 | ≤ 4 | ≤ 2 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 | ≤ 10 | ≤ 6.5 | ≤ 4.5 | ≤ 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 | ≤ 10 | ≤ 6.5 | ≤ 4.5 | ≤ 2.5 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  | ≤ 10 | ≤ 6.5 | ≤ 4.5 | ≤ 2.5 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  | ≤ 10 | ≤ 9 | ≤ 5 | ≤ 3.5 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | ≤ 11 | ≤ 6.5 | ≤ 6 | ≤ 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | ≤ 11 | ≤ 6.5 | ≤ 6 | ≤ 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  | ≤ 11 | ≤ 6.5 | ≤ 6 | ≤ 4 |

# Annex