3GPP TSG-RAN4 Meeting #117	R4-2522158

# Dallas, TX, USA Nov 17th – Nov 21st 2025

Source: 	Qualcomm Incorporated

Title: 	Remaining Issues of FR2-1 RX Beam Sweeping Factor Optimization

Agenda item:	4.15.1

Document for:	Discussion

# Introduction

RAN4 has been discussing FR2-1 RX beam sweeping factor optimization during Rel-19. This Tdoc provides our views regarding the remaining open issues of this topic.

# Discussion

Following issue was kept open during the last meeting.

Issue 1-1-1: Condition of re-entering FSB after exiting <Way Forward>: - Further discuss in next meeting considering the below options: - Option1: Define T2 for re-evaluation after the UE exits FBS mode. - Option2: Not to define T2 for re-evaluation after the UE exits FBS mode. - Option 2.1: Need to update core requirements. - Option 2.2: Not need to update core requirements.

RAN4 RRM session has introduced the parameter T1 that allows UE to exit fast beam sweeping (FBS) mode a certain time duration after triggering FBS based on channel condition. Proponents of T1 mentioned that UE may need to exit FBS due to concerns regarding battery power. Now, RAN4 is discussing whether to define T2 that allows UE to re-evaluate channel conditions and enter FBS a certain time after existing FBS mode. Power related concerns are purely UE internal and they are transparent to external entities. If UE has exited FBS due to high power consumption concern, it is not clear how RAN4 can determine an appropriate time duration T2 after which UE’s power consumption related concerns will be resolved. Hence, RAN4 does not have any proper mechanism to define T2 for re-evaluation after the UE exists FBS mode. It should be left to UE implementation if and how to restart FBS after it has exited FBS mode.

Observation 1: Proponents of T1 mentioned that UE may need to exit FBS due to concerns regarding battery power.

Observation 2: If UE has exited FBS due to high power consumption concern, it is not clear how RAN4 can determine an appropriate time duration T2 after which UE’s power consumption related concerns will be resolved.

Proposal 1: RAN4 does not define T2 for re-evaluation after the UE exits FBS mode.

RAN4 made the following agreements during the last two meetings [1] [2][3]

### Issue 1-1-5: Revision of the agreement on applicability requirement Agreement in Tuesday ad-hoc: - The FBS enhancement applies for the case when the target and serving carrier frequency are same. - FFS in maintenance which CC to apply the FBS when the target and serving carrier frequency are on different. Issue 1-1-2: Multi-band supported by FBS <Agreement>: - The serving carrier shall be a FR2-1 carrier. - For inter-frequency measurement, when serving carrier and target carrier are in different bands, use the FBS value reported for the band of the target carrier in the requirements. Online discussion Session Chair: It is RAN4 common understanding that FBS requirement apply to intra-frequency measurement and inter-frequency measurement within measurement gap.

Following issue was kept open during the last meeting

Issue 1-1-4: Applicability of FBS when the target and serving carrier frequency are on different <Way Forward>: - Further discuss the options: - Option 1: When the target and serving carrier frequency are different, FBS applies to target carrier only. - Option 2: When the target and serving carrier frequency are different, FBS applies to the serving carrier and target carrier. - Option 3: Introduce UE capability indicating UE can support FBS on both target and serving carrier frequency.

It is not clear what above WF tries to resolve that have not already been resolved during previous meetings. RAN4 already agreed that UE should use the FBS value reported for the band of the target carrier during inter-frequency measurement when serving and target carrier are in different bands. On the other hand, when serving and target carrier are same, UE should use the value reported for the band of the serving and target carrier for intra-frequency neighbor cell measurements.

When UE gets configured to perform both intra-frequency and inter-frequency measurements where measurement objects of intra and inter frequency don’t overlap in time domain and the band corresponding inter-frequency MOs is different from UE’s serving band, UE simply uses the FBS value of each individual band to measurement MO of that band. That means, UE should use the FBS corresponding to serving carrier during neighbor cell intra-frequency measurement and the FBS corresponding to the band containing the inter-freq MOs during inter-freq measurement.

Proposal 2: When UE gets configured to perform both intra-frequency and inter-frequency measurements where the band corresponding to inter-frequency MOs is different from UE’s serving band,

- UE uses the FBS reported for the serving carrier during intra-frequency neighbor cell measurement

- UE uses the FBS reported for the band containing inter-freq MOs during inter-freq measurement.

# Conclusions:

Observation 1: Proponents of T1 mentioned that UE may need to exit FBS due to concerns regarding battery power.

Observation 2: If UE has exited FBS due to high power consumption concern, it is not clear how RAN4 can determine an appropriate time duration T2 after which UE’s power consumption related concerns will be resolved.

Proposal 1: RAN4 does not define T2 for re-evaluation after the UE exits FBS mode.

Proposal 2: When UE gets configured to perform both intra-frequency and inter-frequency measurements where the band corresponding to inter-frequency MOs is different from UE’s serving band,

- UE uses the FBS reported for the serving carrier during intra-frequency neighbor cell measurement

- UE uses the FBS reported for the band containing inter-freq MOs during inter-freq measurement.

# References

- R4-2514800, “WF on RRM Requirements for NR_RRM_Ph5_Part1_v04”, 3GPP TSG-RAN WG4 Meeting #116bis.

- RAN4 RRM Session Repor, 3GPP TSG RAN WG4 #116bis.

- R4-2512125, WF on RRM requirements for NR_RRM_Ph5_Part2, 3GPP TSG-RAN WG4 #116