3GPP TSG-RAN4 Meeting #117	        R4-2522243 Dallas, Texas, United States, 17th November – 21st November 2025

Source: 	Qualcomm Inc.

Title: 	6 MHz Channel Bandwidth A-MPR Analysis

Agenda Item:	7.1.5.2

Document for:	Discussion

WI Code:	NR_UE_RF_Ph5-Core

# Introduction

Based on the WF [1] agreed in RAN4#116bis meeting, in this contribution initial results of A-MPR analysis for network signalling label NS_15 are presented.

# Simulation Study

For analyzing the impact on Tx performance, we simulated the impact on achievable output power. A power class 2 (PC2) transmitter was calibrated to just meet the ACLR with DFT-s-OFDM waveform with Start_RB = 0 and L_CRB = 100 and MPR = 1 dB. Other parameters in the simulation set up are as follows :

- Channel Bandwidth = 6 MHz

- SCS = 15 kHz

- Maximum # of RBs = 30

- Band of Operation : n5 (Uplink : 824 – 849 MHz)

- LO leakage = 28 dBc

- Image = 28 dBc

- CIM3 = 60 dBc

- Network Signalling Label : NS_15

- Waveforms : CP-OFDM and DFT-s-OFDM

- Modulation : QPSK

Preliminary simulated results are presented below, further corroboration with bench measurements will be performed.

NS_15 :

Carrier frequency of 846 MHz is used to evaluate A-MPR behavior over the entire allocation space of 6 MHz channel bandwidth. The results are presented below for DFT-s-OFDM and CP-OFDM QPSK waveforms.

Figure 1: A-MPR behavior for NS_15 @ Fc = 846 MHz for (a) DFT-s-OFDM and (b) CP-OFDM

Further study is required to complete the analysis over the range of carrier frequencies where A-MPR is impacted and for other sets of waveforms as highlighted in the tables 1 and 2 below.

Observation 1: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_15 is ≤ 9 dB.

Proposal 1: A-MPR for NS_15 label should be studied for all the remaining waveforms and at other carrier frequencies where it could impact A-MPR within n5 band.

Proposal 2: Study A-MPR for PC3 UE in band n5 where NS_15 label is applicable.

Table 1: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.44 | >0 | A5 |
|  |  | ≤ 0.18 | ≤ 0.36 | A6 |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
| 6 MHz | Fc = 846 MHz | TBD | TBD | TBD |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |

Table 2: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  |

Observation 2: Network signalling label NS_06 is applicable for bands n12 and n85.

Proposal 3: Study A-MPR for both PC2 and PC3 UEs in bands n12, n85 where NS_06 network signaling label is applicable.

# Conclusions

Following observations and proposals were made:

Observation 1: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_15 is ≤ 9 dB.

Proposal 1: A-MPR for NS_15 label should be studied for all the remaining waveforms and at other carrier frequencies where it could impact A-MPR within n5 band.

Proposal 2: Study A-MPR for PC3 UE in band n5 where NS_15 label is applicable.

Observation 2: Network signalling label NS_06 is applicable for bands n12 and n85.

Proposal 3: Study A-MPR for both PC2 and PC3 UEs in bands n12, n85 where NS_06 network signaling label is applicable.

# References

[1]	R4-2515084, “Way Forward for [116bis][329] NR_UE_RF_Ph2_Parrt2 – 6 MHz channel”, Samsung, Nokia, Ericsson, Skyworks, Apple, Huawei, CATT, ZTE, Qualcomm, RAN4 #116bis, Prague, Czech Republic, 13th – 17th October, 2025.