3GPP TSG-RAN WG4 Meeting #117	R4-2522387

# Dallas, USA,  17th – 21st November 2025								(revision of R4-2520566)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0096 | rev | 1 | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for the measurement method in ETSI requirements for the 1610-1626.5MHz range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Rohde & Schwarz, Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_LS_band-Core |  |  |  |  |  | Date: |  |  | 2025-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For a UE operating in 1610-1626.5MHz frequency range ETSI defines additional in-band and out-of-band emission requirements, which are captured in ETSI EN 301441. However, when these requirements were captured in the RAN4 specifications, the measurement method was not captured accordingly, which may leave to a potential ambiguity on whether it is mean/average or peak limit. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 6.2A.1 and 6.2B.1, additional clarifications are provided that the ETSI PSD limit is for UE power spectral density with either mean/average or peak limit measurement method and that this limit applies to any frequency within the operating Tx range. In addition to that, "EIRP” is changed to “power" to align table headers with same table headers in TS 38.101-5. Additional clarifications are provided for the NS flags NS_04N, NS_05N, NS_11N, NS_12N capturing which measurement method should be applied. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will remain unclear which measurement method should be applied to ETSI emission requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1, 6.2B.1, 6.5A.3.3.2, 6.5A.3.3.3, 6.5A.4.4.5, 6.5B.3.3.3, 6.5B.3.3.4, 6.5B.4.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2A.1	UE maximum output power for category M1

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 6.2A.1-1: UE Power Class

| E-UTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 |  |  | 23 | +/-2 | 20 | +/-2 |
| 255 |  |  | 23 | +/-2 | 20 | +/-2 |
| 254 |  |  | 23 | +/-2 | 20 | +/-2 |
| 253 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

The UE shall meet the following additional requirements for maximum transmission power density specified in Table 6.2A.1-2 when NS is signalled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2A.1-2: Additional requirements for transmit power density

| E-UTRA Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density | Measurement method |
| --- | --- | --- | --- | --- | --- |
| 254 | NS_04N | 1.4 | 1610 - 1618.25 | 27dBm/4kHz | Mean spectral limit (note 2) |
|  | NS_05N | 1.4 | 1618.25 - 1626.5 |  |  |
|  | NS_11N | 1.4 | 1610 - 1618.25 | 15dBm/4kHz | Peak spectral limit (note 2) |
|  | NS_12N | 1.4 | 1618.25 - 1626.5 |  |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. NOTE 2: Indicated limit is applied to any frequency of the 1610-1626.5MHz band. |  |  |  |  |  |

********** NEXT CHANGED SECTION **********

### 6.2B.1	UE maximum output power for category NB1 and NB2

Category NB1 and NB2 UE Power Classes are specified in Table 6.2B.1-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75 kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2B.1-1: UE Power Class

| E-UTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2 | 20 | +/-2 |
| 255 | 23 | +/-2 | 20 | +/-2 |
| 254 | 23 | +/-2 | 20 | +/-2 |
| 253 | 23 | +/-2 | 20 | +/-2 |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

The UE shall meet the following additional requirements for maximum transmission power density specified in Table 6.2B.1-2 when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2B.1-2: Additional requirements for transmit power density

| E-UTRA Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density | Measurement method |
| --- | --- | --- | --- | --- | --- |
| 254 | NS_04N | 0.2 | 1610 - 1618.25 | 27dBm/4kHz | Mean spectral limit (note 2) |
|  | NS_05N | 0.2 | 1618.25 - 1626.5 |  |  |
|  | NS_11N | 0.2 | 1610 - 1618.25 | 15dBm/4kHz | Peak spectral limit (note 2) |
|  | NS_12N | 0.2 | 1618.25 - 1626.5 |  |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. NOTE 2: Indicated limit is applied to any frequency of the 1610-1626.5MHz band. |  |  |  |  |  |

********** NEXT CHANGED SECTION **********

##### 6.5A.3.3.2	Requirements for network signalling value "NS_04N" and "NS_11N"

When "NS_04N" or "NS_11N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.2-1 for any channel bandwidth configured within 1610-1618.25MHz.

Table 6.5A.3.3.2-1: Additional requirements for "NS_04N" and "NS_11N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
| ± 0-160 | -2 | 30kHz | Average |
| ± 160-2300 | -2 to -26 |  |  |
| ± 2300-18500 | -26 |  |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

##### 6.5A.3.3.3	Requirements for network signalling value “NS_05N" and "NS_12N"

When "NS_05N" or "NS_12N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.3-1 for any channel bandwidth configured within 1618.25-1626.5MHz.

Table 6.5A.3.3.3-1: Additional requirements for "NS_05N" and "NS_12N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
| ± 0-160 | -5 | 30kHz | Average |
| ± 160-225 | -5 to -8.5 |  |  |
| ± 225-650 | -8.5 to -15 |  |  |
| ± 650-1365 | -15 |  |  |
| ± 1365-1800 | -23 to -26 |  |  |
| ± 1800-16500 | -26 |  |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

********** NEXT CHANGED SECTION **********

##### 6.5A.4.4.5	Minimum requirement (network signalled value "NS_04N", "NS_05N", "NS_11N" and "NS_12N")

When "NS_04N" or "NS_05N" or "NS_11N" or "NS_12N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.5-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2-1 from the edge of the channel bandwidth.

Table 6.5A.4.4.5-1: Additional out-of-band requirements for "NS_04N", "NS_05N", "NS_11N" and "NS_12N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
|  | BWchannel |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Average (note 2) |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
|  |  |  | Average |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 2: In the sub-band 1573.42MHz to 1580.42 MHz the average measurement time is 20ms. |  |  |  |

********** NEXT CHANGED SECTION **********

##### 6.5B.3.3.3	Requirements for network signalling value "NS_04N" and "NS_11N"

As specified in 6.5A.3.3.2.

##### 6.5B.3.3.4	Requirements for network signalling value “NS_05N" and "NS_12N"

As specified in 6.5A.3.3.3.

********** NEXT CHANGED SECTION **********

##### 6.5B.4.4.5	Minimum requirement (network signalled value "NS_04N", "NS_05N", "NS_11N" and "NS_12N")

When "NS_04N" or "NS_05N" or "NS_11N" or "NS_12N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.5-1 where BWchannel is replaced with 200 kHz, and FOOB (MHz) is replaced with 1.7MHz.