3GPP TSG-RAN WG4 Meeting #117											     R4-2522638

Dallas, US, Nov. 17-21, 2025

Title: 	WF on NR_MIMO_Ph5_RRM_Part2

Agenda Item:	6.12.1

Source: 	MediaTek inc.

Document for:	Approval

# <Topic 1: Performance requirements for CJT reporting>

Issue 1-1: Accuracy requirements for delay and frequency offset reporting for CJT calibration

Agreement:

- For FO, define two set of accuracy requirements with UE capability based on 3dB SNR, for 0.2ppm and 256 levels

  - For 15kHz SCS, [33] and [3]

  - For 30kHz SCS, [68] and [4]

- For DO, define two set of accuracy requirements with UE capability based on 3dB SNR, for CP and 256 levels

  - For 15kHz SCS, 14 and [3];

  - For 30kHz SCS, 18 and [4]

- Note: Achieving better accuracy requires advanced UE implementation and hence UE capability to be introduced.

  - The Note will not be captured in the CR.

Issue 1-2: On UE capability in Issue 1-1

Agreement:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_MIMO_Ph5 | 62-1 | UE accuracy for CJTC delay offset reporting | Support of high accuracy reporting for CJTC-Dd | 59-2-3-1 | Yes | N/A | UE does not support the requirements for high accuracy reporting for CJTC-Dd | Per band and per BC, or, Per UE Note: Both “Per band and per BC” and “Per UE” are feasible from RAN4 perspective, and the decision is up to RAN2 | No | FR1 only | N/A | The high accuracy reporting for CJTC-Dd is defined in TS 38.133 clause 10 | Optional |
| 62. NR_MIMO_Ph5 | 62-2 | UE accuracy for CJTC frequency offset reporting | Support of high accuracy reporting for CJTC -F | 59-2-3-2 | Yes | N/A | UE does not support the requirements for high accuracy reporting for CJTC-F | Per band and per BC, or, Per UE Note: Both “Per band and per BC” and “Per UE” are feasible from RAN4 perspective, and the decision is up to RAN2 | No | FR1 only | N/A | The high accuracy reporting for CJTC-F is defined in TS 38.133 clause 10 | Optional |

Issue 1-3: How to define report mapping tables for CJTC reporting?

Agreement:

- Discuss directly in the CR

# <Topic 2: Performance requirements for asymmetric DL sTRP/UL mTRP scenarios>

Issue 2-1: Whether applicability rule for asymmetric DL sTRP/UL mTRP scenarios TCs is needed?

Agreement:

- For asymmetric TRPs, do not define applicability rules between EN-DC and NR-SA.