# 3GPP TSG-RAN WG4 Meeting #117                                                      R4-2522641

# Dallas, USA, Nov. 17-21, 2025

Agenda item:			6.14.1

Source:	Moderator (CMCC)

Title:	WF on Ambient_IoT_RRM

Document for:	Approval

Topic #2: RRM performance requirements for device 1 (6.14.3)

Issue 2-1-1: test procedures for A-IoT RRM test

Agreement:

  - Step 1: TE transmits an R2D message carrying A-IoT Paging message with access type as CBRA

    - Number of Access Occasions: 1

    - Time Resource Indication and Frequency Resource Indication: configured as 1

  - Step 2: device transmits an D2R message carrying Access Random ID (Msg1). The D2R transmission shall meet the transmit timing requirements

  - Step 3: TE monitors occasions whether there is Access Random ID reflected by device,

    - TE transmit Access trigger message if there is no Access Random ID detected, until and Access Random ID is detected or the all-access occasions has passed

    - If an Access Random ID is received, TE transmits Random ID Response which is identical to Access Random ID

      - To verify TD2R_min, the Random ID response is transmitted exactly TD2R_min after the end of access random ID backscattered by the DUT

  - Step 4: device transmits an D2R message (MSG3). The D2R transmission shall meet the transmit timing requirements

Issue 2-1-2: OTA related aspects

Agreement:

  - To capture in TS 38.191 that the RRM test is only performed at peak direction

    - for RRM test cases at peak direction, the test case is performed at following conditions:

      - Step 1: CW polarization with minimum value between EIStotalθ,ϕ,PolCW=θ and EIStotalθ,ϕ,PolCW=ϕ

      - Step 2: Reader polarization with minimum value between EISDUT(PolMeas=θ;PolCW=θ) and EISDUT(PolMeas=ϕ;PolCW=θ) if θ polarization of CW is selected in step 1, or the reader polarization with minimum value between EISDUT(PolMeas=θ;PolCW=ϕ) and EISDUT(PolMeas=ϕ;PolCW=ϕ) if ϕ polarization of CW is selected in step 1.

      - Step 3: The received power at DUT’s antenna is set as [10] dB higher than the reference sensitivity requirement.

  - Further discuss whether to capture the above agreement in a dedicated (sub-)section or as part of the test case.

Issue 2-1-3: Test parameters

Agreement:

  - RAN4 to adopt the following parameters for the RRM test case.

    - For R2D transmission, re-use FRC A-FR1-B1-1 in clause B.2.1 of 38191,

    - For R2D transmission, re-use FRC A-FR1-A1-1 in clause B.1 of 38194 with following update

      - Remove “modulation” and “SFO” for RRM test,

      - In addition, define the small frequency shift factor as ‘8’

    - Set “Number of Access Occasions” in Paging message to 1.