3GPP TSG-RAN WG4 Meeting #117	R4-2522648

Dallas, US, 17th – 21st November 2025

Agenda item:			6.17.1

Source:	Nokia

Title:	Ad-hoc minutes for NR_Mob_Ph4_RRM

Document for:	Information

# Ad-Hoc minutes

This summary includes the proposals and CRs for NR mobility enhancement phase 4 NR_Mob_Ph4.

The following discussion order is recommended for the topics:

RRM Core part CLTM (AI 4.23)

Issue 1-1-1: Whether to include unknown cell case on SSB triggered CLTM

Issue 1-1-2: How to capture the Known / Unknown condition in the specification

Performance part for event triggered reporting (AI 6.17.2.1):

Issue 2-1-1: Whether to send DCI in every slot after the event condition is met.

(New) Issue 2-1-2: Discussion on whether delta approach is used for SSB based test cases

Performance part for CSI-RS L1-RSRP (AI 6.17.2.2):

Issue 3-1-1: Whether to send DCI in every slot after the event condition is met

Issue 3-1-2: Test case design for event triggered CSI-RS based L1-RSRP when SSB L1-RSRP measurement is not performed

R4-2520840	Topic summary for [117][215] NR_Mob_Ph4_RRM_Part1

Type: other		For: Information
					Source: Moderator (Nokia)

R4-2520841	Topic summary for [117][216] NR_Mob_Ph4_RRM_Part2

Type: other		For: Information
					Source: Moderator (China Telecom)

# Topic #1 RRM Core requirements

Companies’ contributions summary

| TDoc | Title | Source | Proposals |
| --- | --- | --- | --- |
| R4-2520104 | Discussion on RRM core maintenance for Rel-19 LTM | CATT | Observation 1: CLTM is different with legacy CHO, as there is no concept of known case in legacy CHO requirements, whereas there was definition of known neighbor cell in LTM. Proposal 1: It is better to distinguish between known and unknown target cell cases in the CLTM measurement time requirements when an SSB-based L1-RSRP measurement is used in the event, and specification change is needed. |
| R4-2520930 | Discussion on CLTM | Huawei, HiSilicon | Proposal 1: For SSB based L1-RSRP measurement used with CLTM, RAN4 can define requirements for both known and unknown cases. Proposal 2: The definition of known cell in CLTM needs to be defined, since the known condition for LTM stated in section 6.3.1 TS38.133 requires UE to send a valid L1/L3 report, and it is not applicable for CLTM. |
| R4-2521145 | Discussion on Conditional Intra-CU LTM | ZTE Corporation, Sanechips | Observation 1: For the definiation of Tmeasure in CHO, both known cell and unknown cell are considered, and the requirements defined based on the worst case. Proposal 1: For SSB based L1-RSRP measurement used with CLTM, support both unknown and known case. And further discuss whether it is necessary to change existing specification. |
| R4-2521526 | Discussion on Remaining Issues of Rel-19 Mobility Enhancements | Nokia | Proposal 1: Both known and unknown target cell cases should be covered in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event. |

Draft CRs to RRM core requirements

| TDoc | Title | Source | Impacted Sections and summary of the change | Actions / recommendations |
| --- | --- | --- | --- | --- |
| R4-2520473 | CR for LTM event-triggered L1 reporting | Nokia | 6.3.1 LTM PCell Cell Switch 6.3.1.1 Introduction This is a resubmission of RAN4#116bis endorsed draft CR R4-2514052. Changes: 1. Clarify the agreed applicability rule related to event-triggered reporting and on leave reporting: - Event-triggered L1 report for LTM candidate cell may be SSB or CSI-RS based, so change “SSB” to “RS” to cover both. - Clarify that the on leave report needs to be for the same RS as the event-triggered report, because without this clarification any on leave report even for a different candidate cell would be covered, which does not make sense. 2. Add MAC spec reference for the event-triggered L1 reporting MAC-CE. | Straightforward correction. If companies have comments, flag this to moderator early. Otherwise no revision needed and the CR can be endorsed directly. |
| R4-2521171 | Correction on Conditional Intra-CU LTM | ZTE Corporation, Sanechips | 6.3.2.2.3 Interruption time Both ‘TLTM-IU’ and ‘TCLTM-IU’ are used in clause 6.3.2.2.3. and they should be same. Change ‘TCLTM-IU’ to ‘TLTM-IU’ in the interruption delay requirement. | Same section changes as in R4-2521527. Recommended to merge to R4-2521527 |
| R4-2521527 | CR for Conditional Intra-CU LTM | Nokia | - Capture resubmitted changes from draft CR R4-2514867 endorsed in RAN4#116bis. - Distinguish between known and unknown target cell cases in CLTM measurement time requirements when an SSB-based L1-RSRP measurement is used in the event. | Revise, and merge changes from R4-2521171 Consider resolution on issues: Issue 1-3-1: Whether to include unknown cell case on SSB triggered CLTM And Issue 1-3-2: How to capture the Known / Unknown condition in the specification |
| R4-2522038 | Clearn up for CSI-RS based Intra-frequency L1-RSRP measurements for neighbour cell | Apple | Correct the format of the requirement wording and title. | Straightforward correction. If companies have comments, flag this to moderator otherwise no revision needed. |

## Conditional Intra-CU LTM

Issue 1-1-1: Whether to include unknown cell case on SSB triggered CLTM

Issue 1-3-1: Whether to include unknown cell case on SSB triggered CLTM Recommended WF: Discuss the following: - For SSB based L1-RSRP measurement used with CLTM. Discuss and confirm which scenario is supported: - Option 1: Support only known case (specification change needed) - Option 2: Support both unknown and known case (specification change needed) - Option 3: Support only unknown case (no specification change).

Proposals:

- Proposal 1 (CATT): It is better to distinguish between known and unknown target cell cases in the CLTM measurement time requirements when an SSB-based L1-RSRP measurement is used in the event, and specification change is needed.

  - Observation: CLTM is different with legacy CHO, as there is no concept of known case in legacy CHO requirements, whereas there was definition of known neighbor cell in LTM.

- Proposal 2 (Huawei): For SSB based L1-RSRP measurement used with CLTM, RAN4 can define requirements for both known and unknown cases.

- Proposal 4 (ZTE): For SSB based L1-RSRP measurement used with CLTM, support both unknown and known case. And further discuss whether it is necessary to change existing specification.

  - Observation 1: For the definition of Tmeasure in CHO, both known cell and unknown cell are considered, and the requirements defined based on the worst case.

- Proposal 5 (Nokia): Both known and unknown target cell cases should be covered in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event.

Recommended Way Forward

- Agree the following:

  - Define requirements for both known and unknown cases for CLTM when an SSB-based L1-RSRP measurement is used in the event.

Continuing discussion from online session

MTK: why we need this agreement. Both are covered in the existing CR.

QC: In legacy, Rel-18 LTM is only applicable to known.

HW: In Rel-19 CR, it looks like we already included the unknown case.

ZTE: Whether we need to update the requirement.

HW: known case is typical. Ok to define both known and unknown.

QC: We can achieve it in 6G. UE needs to measure every cell in equal priority.

MTK: For C-LTM and subsequent C-LTM, the same requirements are applied. Both known and unknown are typical.

Nokia: In the current spec, the measurement delay is based on unknown condition. known case is more typical. The known condition is for L1 measurement.

E///: PSS/SSS detection is typically for unknown case. known case is typical for LTM. both are typical for C-LTM.

QC: not configure too many cells which UE has not detected yet.

Understanding of the current spec for Rel-19 C-LTM:

- Understanding #1: both known and unknown cases are already covered with the same requirement (MTK, ZTE)

- Understanding #2: only unknown case is covered (HW, Nokia, E///)

- Understanding #3: only known case is covered (QC)

When an SSB-based L1-RSRP measurement is used in the event:

- Both known and unknown cases should be covered for CLTM, FFS any spec update is required (MTK, ZTE, HW, Nokia, E///)

- Only known case is covered (QC)

Ad-hoc discussion

QC: Add configuration before CLTM to avoid fully blind config

HW: Doesn’t that mean the cell needs to be known?

QC: The time may be longer than 5s

Tentative Agreement:

- Define requirements for both known and unknown cases for CLTM when an SSB-based L1-RSRP measurement is used in the event.

- The requirements apply when CLTM is configured after L3 measurement report from the configured CLTM candidate cell(s)

  - Exact wording can be discussed in CR

Issue 1-1-2: How to capture the Known / Unknown condition in the specification

Depending if there is an agreement to have both known and unknown in the issue Issue 1-1-1.

Proposals:

- Proposal 1 (Huawei): The definition of known cell in CLTM needs to be defined, since the known condition for LTM stated in section 6.3.1 TS38.133 requires UE to send a valid L1/L3 report, and it is not applicable for CLTM.

- Proposal 2 (ZTE): For SSB based L1-RSRP measurement used with CLTM, support both unknown and known case. And further discuss whether it is necessary to change existing specification.

  - Observation 1: For the definition of Tmeasure in CHO, both known cell and unknown cell are considered, and the requirements defined based on the worst case.

- Proposal 3 (Nokia): Both known and unknown target cell cases should be covered in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event.

  - CR R4-2521527:

    - For intra-frequency target cell is known according to the definition in clause 9.14.2

    - For inter-frequency target cell is known according to the definition in clause 9.15.2

Recommended WF:

Discuss the following:

- Option 1: Use known condition from 9.14.2 (intra) / 9.15.2 (inter):

  - For intra-frequency (9.14.2): The cell is considered as known if the following conditions are met in this requirement:

    - The UE has performed L3 measurement on the target cell during the last 5 seconds, and

    - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2.

  - For inter-frequency (9.15.2): The inter-frequency neighbor cell is considered as known if the following conditions are met in this requirement:

    - The UE has performed L3 measurement on the target cell during the last 5 seconds, and

    - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.3.

  - Otherwise, the cell is unknown.

- Option 2: Define new known condition for CLTM in section 6.3.2.

Discussion:

MTK: The cell may become unknown after the UE has fulfilled the known condition, why this needs to be discussed?

QC: share the same view

Oppo: Maybe it still makes sense to consider known only.

Nokia: Both options are testable

MTK: In reality, unknown may still be the most realistic case

Nokia: This is similar to any RAN4 requirement

MTK: If network is sure, why it uses CLTM

HW: detection requirements are quite relaxed

QC: needs to consider also how the test case is done

Nokia: Cell doesn’t need to necessarily be fully off

Tentative agreement:

- FFS how capture known in the specification.

# Topic #2 Event triggered L1 measurement reporting performance part

Companies’ contribution summary

| TDoc | Title | Source | Proposals |
| --- | --- | --- | --- |
| R4-2521920 | Discussion on performance requirements for event-triggered L1 measurement reporting | Ericsson | Proposal 1: RAN4 to agree that TE sends DCI in every slot after the event condition is changed in the test. |

Issue 2-1-1: Whether to send DCI in every slot after the event condition is met

Technical background from 	R4-2521920. Check also Issue 3-1-1 as it is more or less the same.

The event triggered measurement reporting delay is defined as the time between an event that will trigger an event triggered measurement report and the point when the UE starts to transmit the first UL transmission to report the measurement results over the air interface. This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

In the test configuration, using SR as the end point is not verifiable, since the UE can send the SR at any arbitrary time, and the test equipment (TE) cannot determine when the measurement period has actually ended. While using a special purpose SR could be an alternative, but not all UEs may support it. To ensure consistent testing across all UEs, we propose scheduling a DCI with UL grant in every slot once the event is triggered.

Proposals:

- Proposal 1 (Ericsson): 	RAN4 to agree that TE sends DCI in every slot after the event condition is changed in the test.

Recommended WF:

- Discuss the above

Online Discussion:

Nokia: why we don’t use the legacy approach? what’s the difference here

E///: In legacy, the ending point is L3 measurement report, here the ending point is SR in PUCCH.

Nokia: ending point for the core requirement? We can check.

MTK: Similar concern as Nokia. Why not use the legacy way. PUCCH can be configured periodically. No issue.

HW: We understand the motivation of P1. SR in PUCCH is also used in legacy. No need to update this in Rel-19.

QC: UE is allowed to report earlier than the RAN4 requirement.

E///: Special SR is based on UE capability.

HW: Legacy way seems ok

E/// The difference between legacy procedure is L3 report and in this the ending point is SR

Ad-hoc Discussion:

Tentative agreement:

- FFS for the design where TE sends DCI in every slot after the event condition is changed in the test.

Draft CRs for RRM performance requirements

| TDoc | Title | Source | Actions / recommendations |
| --- | --- | --- | --- |
| R4-2520112 | Draft CR on test case for inter-f SSB based L1-RSRP measurement with MG with event triggered reporting | CATT | Upload to the CR folder during the meeting as early as possible for review. |
| R4-2520113 | Draft CR on test case for inter-f SSB based L1-RSRP measurement without MG with event triggered reporting | CATT |  |
| R4-2520477 | DraftCR TC for SSB-based FR2 event-triggered reporting without MG | Nokia |  |
| R4-2520931 | TC for CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement with event triggered reporting or periodic reporting | Huawei, HiSilicon |  |
| R4-2521164 | Test case for Intra-frequency SSB based L1-RSRP measurement in FR2 with event triggered reporting or periodic reporting | ZTE Corporation, Sanechips |  |
| R4-2521173 | Test case for Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM in FR2 with event triggered reporting or periodic reporting | ZTE Corporation, Sanechips |  |
| R4-2521921 | Intra-frequency SSB based L1-RSRP measurement in FR1 with event triggered reporting or periodic reporting | Ericsson |  |

# Topic #3 CSI-RS based L1 measurement Performance part

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| TDoc | Title | Source | Proposals |
| --- | --- | --- | --- |
| R4-2521922 | Discussion on performance requirements for CSI-RS based L1 measurement | Ericsson | - RAN4 to agree that TE sends DCI in every slot after the event condition is changed in the test. - For the event triggered CSI-RS based L1-RSRP by skipping SSB based L1-RSRP, the UE is expected to report the event-triggered L1-RSRP within 160 ms (assuming CSI-RS periodicity of 20ms) from the start of T2. |

Issue 3-1-1: Whether to send DCI in every slot after the event condition is met

Technical background from 	R4-2521922

The event triggered measurement reporting delay is defined as the time between an event that will trigger an event triggered measurement report and the point when the UE starts to transmit the first UL transmission to report the measurement results over the air interface. This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

In the test configuration, using SR as the end point is not verifiable, since the UE can send the SR at any arbitrary time, and the test equipment (TE) cannot determine when the measurement period has actually ended. While using a special purpose SR could be an alternative, but not all UEs may support it. To ensure consistent testing across all UEs, we propose scheduling a DCI with UL grant in every slot once the event is triggered.

Proposals:

- Proposal 1 (Ericsson): 	RAN4 to agree that TE sends DCI in every slot after the event condition is changed in the test.

Tentative agreement:

- FFS for the design where TE sends DCI in every slot after the event condition is changed in the test.

Issue 3-1-2: Test case design for event triggered CSI-RS based L1-RSRP when SSB L1-RSRP measurement is not performed

Technical background from R4-2521922:

- UE may need time to detect the event, it is assumed that the measurement is performed after T2 begins

- Considering the beam sweeping factor, the UE may realize the event condition after completing a full sweep, which could take up to 8 × 20 ms = 160 ms

- Assuming DCI is scheduled every slot

- UE reports the event-triggered L1-RSRP within 160 ms from the start of T2

Proposals:

- Proposal 1 (Ericsson):	For the event triggered CSI-RS based L1-RSRP by skipping SSB based L1-RSRP, the UE is expected to report the event-triggered L1-RSRP within 160 ms (assuming CSI-RS periodicity of 20ms) from the start of T2.

Recommended WF:

- Discuss the above proposal.

  - With 20ms CSI-RS periodicity, the UE transmits the event triggered L1-RSRP measurement report 160ms from the start of T2.

Draft CRs for RRM performance requirements

| TDoc | Title | Source |  |
| --- | --- | --- | --- |
| R4-2520677 | CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 with periodic reporting | Apple | Upload to the CR folder during the meeting as early as possible for review. |
| R4-2520678 | CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 with event periodic reporting | Apple |  |
| R4-2520932 | CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR2 | Huawei, HiSilicon |  |
| R4-2521797 | CSI-RS based L1 RSRP measurement for neighbour cell in FR1 with event triggered reporting or periodic reporting | Nokia |  |
| R4-2521798 | draftCR for CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 | Nokia |  |
| R4-2521923 | CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement with event triggered reporting or periodic reporting | Ericsson |  |
| R4-2522300 | CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 with periodic reporting | Apple |  |
| R4-2522301 | CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 with event periodic reporting | Apple |  |