33GPP TSG-WG4 Meeting #117	 R4-2522715

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on L1-RSRP/SINR measurement requirement for SDL SCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LBCA_Sw-Core |  |  |  |  |  | Date: |  |  | 2025/11/7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Change 1: The IE name should be unified as LowBandCA-via-Switching-r19 Change 2: Noutside_MG definition: from “or”to “and” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the UE capability IE name Correct the Noutside_MG definition: from “or”to “and” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM L1-RSRP/L1-SINR requirements to support LB CA switching is incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5, 9.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ...38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Start of Change 1

9.5	L1-RSRP measurements for Reporting

9.5.4	L1-RSRP measurement requirements

9.5.4.1	SSB based L1-RSRP Reporting

When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

When groupBasedBeamReporting-r17 is configured, the UE shall be capable of performing L1-RSRP measurements based on the two configured SSB resource sets for L1-RSRP, and the UE physical layer shall be capable of reporting group-based L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

When there is no L1-RSRP measurement on LTM neighbor cell(s) to measure within active BWP, the value of TL1-RSRP_Measurement_Period_SSB is defined in table 9.5.4.1-1 for FR1, the value of TL1-RSRP_Measurement_Period_SSB is defined in table 9.5.4.1-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is not configured, and defined in table 9.5.4.1-2A for FR2-1 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is configured, and defined in table 9.5.4.1-3 for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	The value of N in table 9.5.4.1-2A is 2, 4 or 6 in FR2-1 for UE supporting fastBeamSweepingMultiRx-r18 according to the conditions described in clause 3.6.19.

-	The value of N in table 9.5.4.1-2 is 8.

-	The value of N in table 9.5.4.1-3 is 8.

For UE supporting On-demand SSB operation and when OD-SSB transmission is indicated, the EMP for L1-RSRP are

-	the periodicity of OD-SSBs in the configured DL BWP if no first SSB transmission is configured, or

-	the periodicity of the union of OD-SSBs and first SSBs if first SSBs are transmitted and the OD-SSB activation is indicated.

For a UE supporting LowBandCA-via-Switching-r19, or for a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent measurement gap(s) with NCSG measurement gap(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrent GAP and musim-GapPreference-r17 and when concurrent gaps or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured,

-	an SSB or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8 and 9.1.10,

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs , periodic MUSIM gap(s) and per-FR GAPs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

Ntotal is the total number of SSB resource occasions within the window, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window.

For UEs supporting LowBandCA-via-Switching-r19 and configured with low NR inter-band carrier aggregation:

-	for the PCell L1-RSRP measurement, Ntotal also includes SSB resource occasions that overlap with the SDL SCell ON duration within the window,

-	for the SCell L1-RSRP measurement, Ntotal also includes SSB resource occasions that overlap with the PCell ON duration within the window, as defined by the configured switching pattern, and

-	Noutside_MG is the number of SSB resource occasions within the window W

-	that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, and

-	that are not overlapped with SDL SCell ON duration corresponding to the LB CA switching pattern, for the PCell L1-RSRP measurement for a UE supporting LowBandCA-via-Switching-r19with low NR inter-band carrier aggregation configured or

-	that are not overlapped with PCell ON duration corresponding to the LB CA switching pattern, for the SCell L1-RSRP measurement for a UE supporting LowBandCA-via-Switching-r19 with low NR inter-band carrier aggregation configured, and

-	L1-RSRP requirement in this clause is not applied when Noutside_MG = 0.

-	Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped GAP occasion, non-dropped MUSIM gap occasion nor any SMTC occasion within the window W.

-	for UEs supporting LowBandCA-via-Switching-r19, switching pattern periodicity is the periodicity of the RRC configured semi-static switching pattern; otherwise, it is not applicable.

-	an SSB or an SMTC occasion is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion.

-	TL1 is periodicity of the target SSB.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor concurrentMeasGapsPreMG-r18 nor concurrentMeasGapsNCSG-r18 or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s) and concurrent measurement gap(s) with NCSG(s) and UE does not support musim-GapPreference-r17 or when no MUSIM gaps are configured,

For FR1,

-	P=11-TSSBxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB.

For FR2,

P1= 11-TSSBTSMTCperiod, when SSB is not overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P1 is PL1_sharing*Psharing factor, when SSB is not overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P1=11-TSSBxRP-TSSBTSMTCperiod, when SSB is partially overlapped with GAP and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TSSB < 0.5*TSMTCperiod

-	P is Psharing factor1-TSSBxRP, when SSB is partially overlapped with GAP and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TSSB = 0.5*TSMTCperiod

-	P1= 11-TSSBmin(TSMTCperiod,xRP), when SSB is partially overlapped with GAP (TSSB < xRP) and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P is  PL1_sharing*Psharing factor1-TSSBxRP, when SSB is partially overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

-	If SSB resource from the cell with different PCI is configured for L1-RSRP measurement, and P2 is valid accoding to clause 9.13.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain)

-	P = P11-P1*TSSBP2*TSSB_CDP ,   if P1*TSSB < P2*TSSB_CDP.

-	P = P1, if P1*TSSB > P2*TSSB_CDP.

-	P = 2*P1, if P1*TSSB = P2*TSSB_CDP.

-	Otherwise, P = P1

Where:

-	TSSB = ssb-periodicityServingCell of the serving cell

-	TSMTCperiod = the configured SMTC period

-	TSSB_CDP = SSB periodicity of the cell with PCI different from serving cell

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	PL1_sharing = 2, if SSB resource from the cell with different PCI is configured for L1-RSRP measurement, and Psharing_factor,CDP is used in clause 9.13.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain). PL1_sharing = 1, otherwise.

-	TSSB = ssb-periodicityServingCell

-	TSMTCperiod = the configured SMTC period

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	an SSB or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	If the UE is configured with Pre-MG, an SSB or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	Otherwise, when NCSG measurement gap only is configured,

-	an SSB or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

The value of TL1-RSRP_Measurement_Period_SSB is defined in table 9.5.4.1-4 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18 in FR1, table 9.5.4.1-5 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18 in FR1, table 9.5.4.1-6 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18 in FR2 and table 9.5.4.1-7 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18 in FR2 when there is L1-RSRP measurement on LTM neighbor cell(s) to measure within active BWP, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8 in table 9.5.4.1-6 and table 9.5.4.1-7.

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

For a window W of duration max (TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

-	PL1_sharing is defined as

- 	When number of neighboring cells configured with SSB based L1-RSRP measurement is 1

-	PL1_sharing = 2, if any symbol of the SSBs from serving cell and neighbor cell are overlapping or adjacent (in time domain)

-	PL1_sharing = 1, otherwise

-	When number of neighboring cells configured with SSB based L1-RSRP measurement is more than 1

-	PL1_sharing = 3.

Note:	RRM requirements are not applicable when L1 measurement on the cells with different PCI and LTM L1 measurement on neighbour cells are both configured.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

For UE supporting measurement gap occasion cancellation, the UE is not required to perform SSB measurements during the cancelled gap occasions.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and GAP configurations does not meet previous conditions.

When UE is configured with aperiodic MUSIM gap and the aperiodic MUSIM gap is overlapping with SSB resource occasion for L1-RSRP, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and SSB resource occasions for L1-RSRP are fully overlapped with MUSIM gap(s) or fully overlapped with the union of MUSIM gap(s) and GAPs, no requirement applies for the SSB based L1-RSRP measurement.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of the SSB index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise, K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of the SSB index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 9.5.4.1-2A: Measurement period TL1-RSRP_Measurement_Period_SSB configured with groupBasedBeamReporting-r17 for FR2-1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*N)*TDRX |
| Note1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of the SSB index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 9.5.4.1-3: Measurement period TL1-RSRP_Measurement_Period_SSB configured with highSpeedMeasFlagFR2-r17 for FR2-1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N1Note2)*TSSB) |
| DRX cycle ≤ 80 ms | max(TReport, ceil(M*P*N1Note2*M2)*max(TDRX,TSSB)) |
| 80 ms< DRX ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*N)*TDRX |
| Note1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: Scaling factor N1 = 2 when highSpeedMeasFlagFR2-r17 = set1 or scaling factor N1 = 6 when highSpeedMeasFlagFR2-r17 = set2, if UE does not support simultaneousReceptionTwoQCL-r18 or when highSpeedDeploymentTypeFR2-r17 is not configured as bidirectional. Scaling factor N1 = 1.5 when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N1 = 4 when highSpeedMeasFlagFR2-r17 is configured to set2, if UE supports simultaneousReceptionTwoQCL-r18 and when highSpeedDeploymentTypeFR2-r17 is configured as bidirectional. Note 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 9.5.4.1-4: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: The requirements apply if the actual RTD between serving cell and neighbor cell is not larger than CP. |  |

Table 9.5.4.1-5: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. |  |

Table 9.5.4.1-6: Measurement period TL1-RSRP_Measurement_Period_SSB in FR2 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB) ) |
| DRX cycle > 320 ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements apply if the actual RTD between serving cell and neighbor cell is not larger than CP. |  |

Table 9.5.4.1-7: Measurement period TL1-RSRP_Measurement_Period_SSB in FR2 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*PL1_sharing*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

9.5.4.2	CSI-RS based L1-RSRP Reporting

When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured CSI-RS resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_CSI-RS.

When groupBasedBeamReporting-r17 is configured, the UE shall be capable of performing L1-RSRP measurements based on the two configured CSI-RS resource sets for L1-RSRP, and the UE physical layer shall be capable of reporting group-based L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_CSI-RS.

The value of TL1-RSRP_Measurement_Period_CSI-RS is defined in table 9.5.4.2-1 for FR1 and in table 9.5.4.2-2 for FR2, where

-	For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	For aperiodic CSI-RS resources M=1

-	For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set) for table 9.5.4.2-2, where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set.

-	For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided with QCL-TypeD for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided qcl-info is configured for all resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	-	For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clauses 10.1.19.2 and 10.1.20.2 if number of resources in the resource set is smaller than maxNumberRxBeam. The requirements apply provided qcl-info is configured with QCL-TypeD for all resources in the resource set.

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1=0

1>	else (if UE supports supportSBFD and SBFD is configured by the network)

2>	if higher layer parameter timeRestrictionForChannelMeasurement is configured

3>	L1=0

2>	else (if higher layer parameter timeRestrictionForChannelMeasurement is not configured)

3>	if UE is configured to report L1-RSRP for SBFD symbols

4>	When DRX is not configured, L1 is the number of occasions of the CSI-RS resource that are overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS

4>	When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS resource is overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS

3>	if UE is configured to report L1-RSRP for non-SBFD symbols

4>		When DRX is not configured, L1 is the number of occasions of the CSI-RS resource that are overlapping with SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS, when DRX is not configured

4>		When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS resource is overlapping with SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS

For a UE supporting LowBandCA-via-Switching-r19or for a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent measurement gap(s) with NCSG measurement gap(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrent measurement gap and musim-GapPreference-r17 and when concurrent gaps or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured,

-	a CSI-RS or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clauses 9.1.8 and 9.1.10,

-	P value for a CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE measurement gaps or NCSGs, MUSIM gap(s)and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window W, including those overlapped with measurement gap occasions, MUSIM gap occasions or SMTC occasions within the window W.

For UEs supporting LowBandCA-via-Switching-r19 and configured with low NR inter-band carrier aggregation:

-	for the PCell L1-RSRP measurement, Ntotal also includes CSI-RS resource occasions that overlap with the SDL SCell ON duration within the window,

-	for the SCell L1-RSRP measurement, Ntotal also includes CSI-RS resource occasions that overlap with the PCell ON duration within the window, as defined by the configured switching pattern, and

-	Noutside_MG is the number of CSI-RS resource occasions within the window W

-	that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, and

-	that are not overlapped with SDL SCell ON duration corresponding to the LB CA switching pattern, for the PCell L1-RSRP measurement for a UE supporting LowBandCA-via-Switching-r19 with low NR inter-band carrier aggregation configured or

-	that are not overlapped with PCell ON duration corresponding to the LB CA switching pattern, for the SCell L1-RSRP measurement for a UE supporting LowBandCA-via-Switching-r19 with low NR inter-band carrier aggregation configured, and

-	L1-RSRP requirement in this clause is not applied when Noutside_MG = 0.

-	Navailable is the number of CSI-RS resource occasions that are not overlapped with any non-dropped GAP occasions, non-dropped MUSIM gap occasion nor any SMTC occasion within the window W.-	for UEs supporting LowBandCA-via-Switching-r19, switching pattern periodicity is the periodicity of the RRC configured semi-static switching pattern; otherwise, it is not applicable.

-	a CSI-RS or an SMTC occasion is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

-	TL1 is periodicity of the target CSI-RS.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor concurrentMeasGapsPreMG-r18 nor concurrentMeasGapsNCSG-r18 or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s) and concurrent measurement gap(s) with NCSG measurement gap(s), and UE does not support musim-GapPreference-r17 or when no MUSIM gaps are configured.

For FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For FR2,

-	P=1, when CSI-RS is not overlapped with a GAP and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when CSI-RS is not overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P=Psharing factor, when CSI-RS is not overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=1, when aperiodic CSI-RS resource is not overlapped with GAP

-	P= 11-TCSI-RSxRP-TCSI-RSTSMTCperiod, when CSI-RS is partially overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5*TSMTCperiod

-	P=31-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TCSI-RS = 0.5*TSMTCperiod

-	P= 11-TCSI-RSmin⁡(TSMTCperiod,xRP), when CSI-RS is partially overlapped with GAP (TCSI-RS < xRP) and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P=Psharing factor1-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

Where:

-	Psharing factor = 1, if the CSI-RS configured for L1-RSRP measurement outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

TSMTCperiod = the configured SMTC period.

TCSI-RS = the periodicity of CSI-RS configured for L1-RSRP measurement

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a CSI-RS or an SMTC occasion is considered to be as overlapped with the GAP if it overlapps a measurement gap occasion, and

-	xRP = MGRP

-	If the UE is configured with Pre-MG, a CSI-RS or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	Otherwise, when NCSG measurement gap only is configured,

-	a CSI-RS or an SMTC occasion is considered to be as overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

For UE supporting measurement gap occasion cancellation, the UE is not required to perform CSI-RS measurements during the cancelled gap occasions.

When UE is configured with aperiodic MUSIM gap and the aperiodic MUSIM gap is overlapping with CSI-RS resource occasion for L1-RSRP, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and CSI-RS resource occasions for L1-RSRP are fully overlapped with MUSIM gap(s) or fully overlapped with the union of MUSIM gap(s) and GAPs, no requirement applies for the CSI-RS based L1-RSRP measurement.

Table 9.5.4.2-1: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR1

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P)* max(TCSI-RS, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *(M+L1)*P)*max(TDRX,TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. NOTE 3: K = 1 when TCSI-RS ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. NOTE 4: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 or measurementEnhancementCA-r17. NOTE 5: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 9.5.4.2-2: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N)* max(TCSI-RS, Tproc) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P*N)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. NOTE 3: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

End of Change 1

Start of Change 2

9.8	L1-SINR measurements for Reporting

9.8.4	L1-SINR measurement requirements

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

The UE shall be capable of performing L1-SINR measurements with the CSI-RS resource configured as CMR and no dedicated resource configured as IMR for L1-SINR computation, and the UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only.

The value of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only is defined in table 9.8.4.1-1 for FR1 and in table 9.8.4.1-2 for FR2, where

For the value of M,

-	For periodic and semi-persistent CSI-RS resources as CMR, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise;

-	For aperiodic CSI-RS resources as CMR, M=1.

For the value of N in FR2

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured for all resources in the resource set.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requriements apply provided qcl-info is configured for all resources in the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clauses 10.1.28.1 and 10.1.28.3 if number of resources in the resource set is smaller than maxNumberRxBeam. The requriements apply provided qcl-info is configured for all resources in the resource set.

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1=0

1>	else (if UE supports supportSBFD and SBFD is configured by the network)

2>	if higher layer parameter timeRestrictionForChannelMeasurement is configured

3>	L1=0

2>	else (if higher layer parameter timeRestrictionForChannelMeasurement is not configured)

3>	if UE is configured to report L1-SINR for SBFD symbols

4>	When DRX is not configured, L1 is the number of occasions of the CSI-RS resource as CMR that are overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-SINR_Measurement_Period_CSI-RS_CMR_Only

4>	When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS resource as CMR is overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-SINR_Measurement_Period_CSI-RS_CMR_Only

3>	if UE is configured to report L1-SINR for non-SBFD symbols

4>		When DRX is not configured, L1 is the number of occasions of the CSI-RS resource as CMR that are overlapping with SBFD symbols during TL1-SINR_Measurement_Period_CSI-RS_CMR_Only

4>		When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS resource as CMR is overlapping with SBFD symbols during TL1-SINR_Measurement_Period_CSI-RS_CMR_Only

For a UE supporting LowBandCA-via-Switching-r19, or for a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent GAP(s) with NCSG(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrent measurement gap and musim-GapPreference-r17 and when concurrent GAPs or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured,

-	a CSI-RS or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to 9.1.8 and 9.1.10,

-	P value for a CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs or periodic MUSIM gap(s) and per-FR GAPs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window,

For UEs supporting LowBandCA-via-Switching-r19 and configured with low NR inter-band carrier aggregation:

-	for the PCell L1-SINR measurement, Ntotal also includes CSI-RS resource occasions that overlap with the SDL SCell ON duration within the window,

-	for the SCell L1-SINR measurement, Ntotal also includes CSI-RS resource occasions that overlap with the PCell ON duration within the window, as defined by the configured switching pattern, and

-	Noutside_MG is the number of CSI-RS resource occasions within the window W

-	that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion,and

-	that are not overlapped with SDL SCell ON duration corresponding to the LB CA switching pattern, for the PCell L1-SINR measurement for a UE supporting LowBandCA-via-Switching-r19with low NR inter-band carrier aggregation configured or

-	that are not overlapped with PCell ON duration corresponding to the LB CA switching pattern, for the SCell L1-SINR measurement for a UE supporting LowBandCA-via-Switching-r19 with low NR inter-band carrier aggregation configured, and

-	L1-SINR requirement in this clause is not applied when Noutside_MG = 0.

-	Navailable is the number of CSI-RS resource occasions that are not overlapped with any non-dropped GAP occasion, non-dropped MUSIM gap occasion nor any SMTC occasion within the window W.

-	for UEs supporting LowBandCA-via-Switching-r19, switching pattern periodicity is the periodicity of the RRC configured semi-static switching pattern; otherwise, it is not applicable.

-	a CSI-RS or an SMTC occasion is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion.

-	TL1 is periodicity of the target CSI-RS.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor concurrentMeasGapsPreMG-r18 nor concurrentMeasGapsNCSG-r18 or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s) and concurrent GAP(s) with NCSG(s) and UE does not support musim-GapPreference-r17 or when no MUSIM gaps are configured,

For the value of P in FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAP configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For the value of P in FR2,

-	P=1, when CSI-RS is not overlapped with GAP and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when CSI-RS is not overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P is Psharing factor,, when CSI-RS is not overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P= 11-TCSI-RSxRP-TCSI-RSTSMTCperiod, when CSI-RS is partially overlapped with [measurement gap] and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5*TSMTCperiod

-	P=Psharing factor1-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP  and TSMTCperiod = xRP and TCSI-RS = 0.5*TSMTCperiod

-	P= 11-TCSI-RSmin⁡(TSMTCperiod,xRP), when CSI-RS is partially overlapped with GAP (TCSI-RS < xRP) and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P=Psharing factor1-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP  (TSMTCperiod < xRP)

Where:

Psharing factor = 1, if the CSI-RS configured for L1-SINR measurement outside gap is

not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	TSMTCperiod = the configured SMTC1 period or SMTC2 period if configured.

-	TCSI-RS = the periodicity of CSI-RS configured for L1-SINR measurement

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a CSI-RS is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	If the UE is configured with Pre-MG, a CSI-RS reourse or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	Otherwise, when NCSG measurement gap only is configured,

-	a CSI-RS is considered to be overlapped with the GAP  if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Note: The overlap between CSI-RS for L1-SINR measurement and SMTC means that CSI-RS for L1-SINR measurement is within the SMTC window duration.

For UE supporting measurement gap occasion cancellation, the UE is not required to perform CSI-RS measurements during the cancelled gap occasions.

When UE is configured with aperiodic MUSIM gap and the aperiodic MUSIM gap is overlapping with CSI-RS resource occasion for L1-SINR, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and CSI-RS resource occasions for L1-SINR are fully overlapped with MUSIM gap(s) or fully overlapped with the union of MUSIM gap(s) and GAPs, no requirement applies for the CSI-RS based L1-SINR measurement.

Longer evaluation period would be expected if the combination of CSI-RS, SMTC occasion and GAP configurations does not meet previous conditions.

Table 9.8.4.1-1: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P)* max(TCSI-RS, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P)*max(TDRX,TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. NOTE 3: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 9.8.4.1-2: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N)* max(TCSI-RS, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P*N)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. NOTE 3: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

#### 9.8.4.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured

The UE shall be capable of performing L1-SINR measurements with the SSB configured as CMR and dedicated resource configured as IMR for L1-SINR computation, in which the NZP-CSI-RS or CSI-IM resource configured as dedicated IMR shall be 1-to-1 mapped to SSB configured as CMR, with the same periodicity. The UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_SSB_CMR_IMR.

The requirements in this clause are not applicable if NZP-CSI-RS or CSI-IM resource configured as dedicated IMR is scheduled with different periodicity as SSB configured as CMR.

For UE supporting On-demand SSB operation and when OD- SSB transmission is indicated, the EMP for L1-SINR are

-	the periodicity of OD-SSBs in the configured DL BWP if no first SSB transmission is configured, or

-	the periodicity of the union of OD-SSBs and first SSBs if first SSBs are transmitted and the OD-SSB activation is indicated.

The value of TL1-SINR_Measurement_Period_SSB_CMR_IMR is defined in table 9.8.4.2-1 for FR1 and in table 9.8.4.2-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured, and defined in table 9.8.4.2-3 for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured, where

For the value of M

-	For periodic or semi-persistent NZP CSI-RS or CSI-IM resource as dedicated IMR, M=1 if the higher layer parameters timeRestrictionForChannelMeasurements and/or timeRestrictionForInterferenceMeasurements are configured, and M=3 otherwise;

For the value of N in FR2

-	N = 2, 4 or 6 in FR2-1 for UE supporting fastBeamSweepingMultiRx-r18, according to the conditions described in clause 3.6.19,

-	N = 8 otherwise.

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1=0

1>	else (if UE supports supportSBFD and SBFD is configured by the network)

2>	if higher layer parameter timeRestrictionForChannelMeasurement is configured

3>	L1=0

2>	else (if higher layer parameter timeRestrictionForChannelMeasurement is not configured)

3>	if UE is configured to report L1-SINR for SBFD symbols

4>		When DRX is not configured, L1 is the number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-SINR_Measurement_Period_SSB_CMR_IMR

4>		When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS/CSI-IM resource as IMR is overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-SINR_Measurement_Period_SSB_CMR_IMR

3>	if UE is configured to report L1-SINR for non-SBFD symbols

4>	When DRX is not configured, L1 is the number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_SSB_CMR_IMR

4>	When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS/CSI-IM resource as IMR is overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_SSB_CMR_IMR

P is defined as the maximum value between PCMR and PIMR, i.e., P = max(PCMR, PIMR), where

-	the value of PCMR shall be derived in the same way as the value of P used for SSB based L1-RSRP measurement in clause 9.5.4.1, in which the occasions and period of the SSB for CMR shall be used instead.

-	the value of PIMR shall be derived in the same way as the value of P used for CSI-RS based L1-RSRP measurement in clause 9.5.4.2, in which the occasions and period of the NZP CSI-RS for NZP-IMR or CSI-IM for ZP-IMR shall be used instead.

For UE supporting measurement gap occasion cancellation, the UE is not required to perform SSB measurements during the cancelled gap occasions.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet previous conditions.

For L1-SINR measurement with SSB as CMR and CSI-RS or CSI-IM as IMR, the requirement shall apply if the CSI-RS is configured as IMR with repetition field as “repetition = OFF” or CSI-IM is configured as IMR.

For L1-SINR measurement with SSB as CMR and CSI-RS/CSI-IM as IMR, no requirement shall apply if SSB occasions for CMR or CSI-RS/CSI-IM occasions for IMR are fully overlapped with the configured measurement gap

Table 9.8.4.2-1: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P)* max(TSSB, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P)*max(TDRX,TSSB, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of the SSB index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. NOTE 3: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for IMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 9.8.4.2-2: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N)* max(TSSB, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TSSB, Tproc)) |
| DRX cycle > 320 ms | ceil(1.5*(M+L1)*P*N)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR measurement if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of the SSB index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. NOTE 3: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for IMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 9.8.4.2-3: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR configured with highSpeedMeasFlagFR2-r17 for FR2-1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N1Note 3)* max(TSSB, Tproc)) |
| DRX cycle ≤ 80 ms | max(TReport, ceil((M+L1)*P*N1Note 3*M2)*max(TDRX,TSSB, Tproc)) |
| 80 ms< DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*(M+L1)*P*N)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. NOTE 3: N1 = 2 when highSpeedMeasFlagFR2-r17 = set1; N1 = 6 when highSpeedMeasFlagFR2-r17 = set2. NOTE 4: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 NOTE 5: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for IMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured

The UE shall be capable of performing L1-SINR measurements with the CSI-RS resource configured as CMR and dedicated resource configured as IMR for L1-SINR computation, in which the NZP-CSI-RS or CSI-IM resource configured as dedicated IMR shall be 1-to-1 mapped to CSI-RS resource configured as CMR, with the same periodicity. The UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR.

The requirements in this clause are not applicable if NZP-CSI-RS or CSI-IM resource configured as dedicated IMR is scheduled with different periodicity as CSI-RS resource configured as CMR.

The value of TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR is defined in table 9.8.4.3-1 for FR1 and in table 9.8.4.3-2 for FR2, where

For the value of M,

-	M=1 shall be applied if

-	aperiodic NZP-CSI-RS as CMR or dedicated IMR, or

-	aperiodic CSI-IM as dedicated IMR, or

-	periodic and semi-persistent NZP-CSI-RS as CMR or dedicated IMR and the higher layer parameters timeRestrictionForChannelMeasurement and/or timeRestrictionForInterferenceMeasurements are configured, or

-	periodic and semi-persistent CSI-IM as dedicated IMR and the higher layer parameters timeRestrictionForChannelMeasurement and/or timeRestrictionForInterferenceMeasurements are configured;

-	M=3 otherwise.

For the value of N in FR2

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured for all resources in the resource set.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requriements apply provided qcl-info is configured for all resources in the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.28.1 and 10.1.28.3 if number of resources in the resource set is smaller than maxNumberRxBeam. The requriements apply provided qcl-info is configured for all resources in the resource set.

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1=0

1>	else (if UE supports supportSBFD and SBFD is configured by the network)

2>	if higher layer parameter timeRestrictionForChannelMeasurement is configured

3>	L1=0

2>	else (if higher layer parameter timeRestrictionForChannelMeasurement is not configured)

3>	if UE is configured to report L1-SINR for SBFD symbols

4>	When DRX is not configured, L1 is the maximum between number of occasions of the CSI-RS resource as CMR that are overlapping with dynamic UL transmission or with non-SBFD symbols and number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with dynamic UL transmission or with non-SBFD symbols, during TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR

4>	When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS resource as CMR or at least one occasion of the CSI-RS/CSI-IM resource as IMR is overlapping with dynamic UL transmission or with non-SBFD symbols, during TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR

3>	if UE is configured to report L1-SINR for non-SBFD symbols

4>	When DRX is not configured, L1 is the maximum between number of occasions of the CSI-RS resource as CMR that are overlapping with non-SBFD symbols and number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR

4>	When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS resource as CMR or at least one occasion of the CSI-RS/CSI-IM resource as IMR is overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR

P is defined as the maximum value between PCMR and PIMR, i.e., P = max(PCMR, PIMR), where

-	The value of PCMR and PIMR shall be derived in the same way as the value of P used for CSI-RS based L1-RSRP measurement in clause 9.5.4.2, in which the occasions and period of the CSI-RS for CMR and NZP CSI-RS for NZP-IMR or CSI-IM for ZP-IMR shall be used instead respectively.

For UE supporting measurement gap occasion cancellation, the UE is not required to perform CSI-RS measurements during the cancelled gap occasions.

Longer evaluation period would be expected if the combination of CSI-RS, SMTC occasion and measurement gap configurations does not meet previous conditions.

For L1-SINR measurement with CSI-RS as CMR and CSI-RS as IMR, the requirement shall apply only if CSI-RS resources as CMR and IMR are configured with the same repetition field and the number of CSI-RS resources in the resource sets for CMR and IMR are same.

For L1-SINR measurement with CSI-RS as CMR and CSI-IM as IMR, the requirement shall apply only if the number of CSI-RS resources in the resource set for CMR and the number of CSI-IM resources in the resource set for IMR are same.

For L1-SINR measurement with CSI-RS as CMR and CSI-RS/CSI-IM as IMR, no requirement shall apply if CSI-RS occasions for CMR or CSI-RS/CSI-IM occasions for IMR are fully overlapped with the configured measurement gap.

Table 9.8.4.3-1: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P)*max(TCSI-RS, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P)*max(TDRX,TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. NOTE 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. NOTE 4: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CMR or IMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 9.8.4.3-2: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N)* max(TCSI-RS, Tproc) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P*N)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. NOTE 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. NOTE 4: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CMR or IMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

End of Change 2