3GPP TSG-RAN4 Meeting #117	R4-2522723

# Dallas, US, November 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 6132 | Rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for LTE_terr_bcast_Ph2_demod |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_Ph2-Perf |  |  |  |  |  | Date: |  |  | 2025-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It has been agreed to introduce new demod test cases for time-frequency interleaving |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introducing new demod test cases for time-frequency interleaving - Introducing new reference measurement channels for time-frequency interleaving - Defining a new channel model for 50 Hz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreed demod tests can not be defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.1, A.3.1.1, A.3.8.1 FDD, B.2.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==== Start of Change #1 ====

10.4.1	Minimum requirement for PMCH decoding

10.4.1.3		Minimum requirement with 1.25kHz subcarrier spacing

The receive characteristic of MBMS is determined by the BLER.

For the parameters specified in Table 10.4.1.3-1 and Table A.3.8.1-11, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.4.1.3-2.

Table 10.4.1.3-1: Test Parameters for Testing

| Parameter |  | Unit |  |
| --- | --- | --- | --- |
| Downlink power allocation | ρA | dB | 0 |
|  | ρB | dB | 0 (Note 1) |
|  |  | dB | 0 |
| Noc at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode in PCell |  |  | 1 |
| Subcarrier spacing for MBSFN cell |  | kHz | 1.25 kHz |
| Bandwidth |  | MHz | 8 (Note 2) |
| Note 1: PB = 0. Note 2: For both Pcell and Scell. |  |  |  |

Table 10.4.1.3-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | MBSFN RS type | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | N/A | OP.1 FDD | AWGN | 1x1 | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 8 | R.108-1 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 12.2 |  |
|  | MBMS Dedicated Cell | 8 | R.108-1 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-2) | 1x1 | 1 | 10.6 |  |
| 2 | PCell | 10 | N/A | N/A | OP.1 FDD | AWGN | 1x1 | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 8 | R.108-2 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 3 |  |
|  | MBMS Dedicated Cell | 8 | R.108-2 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-2) | 1x1 | 1 | 2.4 |  |

==== End of Change #1 ===

==== Start of Change #2 ====

### A.3.1.1	Overview of DL reference measurement channels

Table A.3.1.1-1O: Overview of DL reference measurement channels (PMCH Performance)

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.8.1-1 | R.40 FDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-1 | R.37 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-2 | R.38 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-3 | R.39-1 FDD | 5 | 64QAM | 2/3 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-3 | R.39 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-4 | R.81-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-4 | R.81-2 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-5 | R.82-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-6 | R.83-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-6 | R.83-2 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-7 | R.84-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-8 | R.85-1 FDD | 3 | QPSK | 1/3 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-8 | R.85-2 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-8 | R.85-3 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-1 FDD | 10 | 64QAM | 0.48 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-2 FDD | 10 | 64QAM | 0.52 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-3 FDD | 6 | 64QAM | 0.49 | 30 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-4 FDD | 7 | 64QAM | 0.48 | 35 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-5 FDD | 8 | 64QAM | 0.48 | 40 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-6 FDD | 6 | 64QAM | 0.54 | 30 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-7 FDD | 7 | 64QAM | 0.53 | 35 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-8 FDD | 8 | 64QAM | 0.52 | 40 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-10 | R.107 FDD | 10 | 16QAM | 0.46 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-10 | R.107-1 FDD | 6 | 16QAM | 0.46 | 30 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-10 | R.107-2 FDD | 7 | 16QAM | 0.46 | 35 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-10 | R.107-3 FDD | 8 | 16QAM | 0.46 | 40 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-11 | R.108-1 FDD | 8 | 16QAM | 0.41 | 40 |  |  |  |
| FDD | Table A.3.8.1-11 | R.108-2 FDD | 8 | QPSK | 0.3 | 40 |  |  |  |
| TDD |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.8.2-1 | R.40 TDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-1 | R.37 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-2 | R.38 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-3 | R.39-1 TDD | 5 | 64QAM | 2/3 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-3 | R.39 TDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |

==== End of Change #2 ====

==== Start of Change #3 ====

Table A.3.8.1-11: Fixed Reference Channel for subcarrier spacing 1.25 kHz with LTE based 5G terrestrial broadcast MBMS dedicated cell with time-frequency interleaving

| Parameter | PMCH |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.108-1 FDD | R.108-2 FDD |
| Channel bandwidth | MHZ | 8 | 8 |
| Subcarrier spacing | kHz | 1.25 | 1.25 |
| Allocated resource blocks |  | 39 | 39 |
| Allocated slots per 40 ms (Note 1) |  | 32 | 39 |
| Time domain spreading and scaling (M, N) |  | (8, 4) | (32, 16) |
| Frequency domain interleaving |  | Enabled Codebook cycling disabled | Enabled Codebook cycling disabled |
| Modulation |  | 16QAM | QPSK |
| Target coding rate |  | 0.41 | 0.3 |
| Information bit payload (Note 2) |  |  |  |
| For each slot | Bits | 31704 | 45352 |
| Number of code blocks per subframe |  | 6 | 6 |
| Binary channel bits per subframe |  |  |  |
| For each slot | Bits | 19200 | 9600 |
| Transport block size before scaling with N | Bits | 7992 | 2825 |
| Transport block size after scaling with N | Bits | 31704 | 45353 |
| MBMS UE category |  |  |  |
| Note 1: First subframe every 40 ms is allocated for non-MBMS transmission Note 2: Zero OFDM symbols are reserved for PDCCH and no CRS allocated as per TS 36.211 [4] Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 bits is allocated to each Code Block (otherwise L = 0 bits) |  |  |  |

==== End of Change #3 ====

==== Start of Change #4 ====

B.2.6	MBSFN Propagation Channel Profile

### B.2.6.2	Subcarrier spacing 1.25kHz

Table B.2.6.2-1 shows propagation conditions that are used for the MBSFN performance requirements in multi-path fading environment in an extended delay spread environment for subcarrier spacing as 1.25kHz.

Table B.2.6.2-1: Propagation Conditions for Multi-Path Fading Environments for MBSFN Performance Requirements in an extended delay spread environment with subcarrier spacing 1.25kHz

| Extended Delay Spread |  |
| --- | --- |
| Maximum Doppler frequency [5Hz] |  |
| Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 |
| 30 | -1.5 |
| 150 | -1.4 |
| 310 | -3.6 |
| 370 | -0.6 |
| 1090 | -7.0 |
| 49960 | -10 |
| 49990 | -11.5 |
| 50110 | -11.4 |
| 50270 | -13.6 |
| 50330 | -10.6 |
| 51050 | -17.0 |
| 109960 | -20 |
| 109990 | -21.5 |
| 110110 | -21.4 |
| 110270 | -23.6 |
| 110330 | -20.6 |
| 111050 | -27.0 |

Table B.2.6.2-2: Propagation Conditions for Multi-Path Fading Environments for MBSFN Performance Requirements in an extended delay spread environment with subcarrier spacing 1.25kHz and 50 Hz Doppler frequency

| Extended Delay Spread |  |
| --- | --- |
| Maximum Doppler frequency [50Hz] |  |
| Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 |
| 30 | -1.5 |
| 150 | -1.4 |
| 310 | -3.6 |
| 370 | -0.6 |
| 1090 | -7.0 |
| 49960 | -10 |
| 49990 | -11.5 |
| 50110 | -11.4 |
| 50270 | -13.6 |
| 50330 | -10.6 |
| 51050 | -17.0 |
| 109960 | -20 |
| 109990 | -21.5 |
| 110110 | -21.4 |
| 110270 | -23.6 |
| 110330 | -20.6 |
| 111050 | -27.0 |

==== End of Change #4 ====