3GPP TSG-RAN4 Meeting #117	R4-2522732

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6226 | rev | - | Current version: | 18.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_Mob_Enh2-Core) CR to 38.133 on Rel-18 LTM PDCCH ordered RACH core requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: The following agreement is not implemented currently in LTM specification: RAN4#115 agreement (R4-2508453) <Agreement>: The minimum gap from the PDCCH-order to the RO, in RAN4, is re-defined as ‘Max(X ms, the current RAN4 minimum timeline requirement).’ - X = 3.5 for intra-BWP early PRACH - X = 5.5 for inter-BWP early PRACH Change 2: Fixing unsearchable (MS Word) symbols for PDCCH ordered RACH to searchable format. Change 3: Currently when collision happens due to MG, the UE is allowed to drop the transmission of the RACH. This is unwanted behaviour from the network side as it is invisible for the network whether the PRACH transmission failed or UE dropped it. <Agreement on reply LS> - RACH shall be prioritized over SSB, if the collision is outside MG. - If collision happens within MG, it is up to UE implementation, i.e., UE may transmit or drop RACH when the RACH occasion colliding with the MG occasion. < Agreement > - Further discuss the definition of RACH occasion colliding with the MG occasion. The requirements corrected such that UE always transmit RACH after PDCCH ordered RACH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2C.1, 6.2.2C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 6.2.2C	PDCCH ordered Random Access for LTM

#### 6.2.2C.1	Introduction

The requirements in this clause apply for a UE capable of rach-EarlyTA-Measurement-r18 configured to trigger RACH procedure toward target LTM candidate cell before receiving cell switch command MAC-CE.

The requirements in this clause apply if PDCCH order-based RACH on candidate cell is triggered based on L1-RSRP report or L3-RSRP beam-level report in FR1, or based on L1-RSRP report in FR2.

The requirements in this clause for PDCCH ordered-based RACH to an inter-frequency target cell is only applicable, when

-	network has configured UE to perform SSB based L3 measurement with beam measurement reporting or L1 measurement for the target cell before the PDCCH order, or

-	the SFN of the serving cell from which PDCCH order is received and the SFN of the target cell are the same.

After receiving PDCCH ordered RACH command, the UE shall transmit RACH preamble no later than the first RACH occasion which is not colliding with measurement gap.

-     RACH collision occurs if measurement gap overlaps with RACH occasion, RF retuning time to target cell or RF retuning time back to serving cell.

-     If the RACH occasion is colliding with MG, it is up to UE implementation whether to transmit or drop the RACH transmission.

#### 6.2.2C.2	PDCCH ordered Random Access delay

PDCCH ordered RACH delay TLTM-PDCCH-order is the delay from PDCCH ordered RACH command until the time when the UE shall complete the RACH transmission to the LTM candidate cell.

UE may perform preamble transmission from NT,2 + ∆BWPSwitching + ∆Delay + Tswitch + TSSB + ∆RF/BB preparation delay and shall complete RACH transmission in TLTM-PDCCH-order, where

TLTM-PDCCH-order = Max (Tmin-time-gap, NT,2 + ∆BWPSwitching + ∆Delay + Tswitch + TSSB + ∆RF/BB preparation + Tfirst-valid-RO).

Where:

-    Tmin-time-gap is the minimum time gap between the PDCCH ordered RACH command to the first candidate cell RACH occasion UE shall be able to transmit (RO), where:

-    If PRACH resource is within the active UL BWP of serving cell(s), the Tfirst-RO is 3.5ms

-    If PRACH resource is outside of the active UL BWP of serving cell(s), the Tfirst-RO is 5.5ms

-     NT,2, ∆BWPSwitching, ∆Delay and Tswitch are defined in TS 38.213 [3] clause 8.1

-     TSSB 	TSSBis the time for T/F tracking and acquiring full timing information of the target cell

-    TSSB 	TSSB= 0, provided:

-	L1-RSRP measurement is configured and

-	The target LTM candidate cell is on an FR1 carrier whose TCI state associated with SSB indicated in the PDCCH-order is activated, and the time between the MAC-CE activating the target TCI state and the PDCCH is larger than TCI state activation delay defined in clause 8.25.3, and

-	L1-RSRP measurement period of candidate cell is not larger than 160 ms, or

-	The time between the MAC-CE activating the target TCI state and the PDCCH is not larger than TCI state activation delay defined in clause 8.25.3 + 160ms.

-	Otherwise, TSSBTSSBTfirst-SSB_RACHTSSB-proc = Tfirst-SSB-RACH + TSSB-proc, where

-	Tfirst-SSB-RACH Tfirst-SSB_RACHis the time to the first SSB occasion, after 1slot from the end of the slot of the PDCCH, when SSB is to be measured outside MG

-	Tfirst-SSB-RACH Tfirst-SSB_RACHis the time to the first SSB occasion, overlapped with MGL after 2ms and 1slot from the end of the slot of the PDCCH if the SSB to be measured within MG.

-	TSSB-proc TSSB-proc= 2 ms, which is the time for SSB processing;

-	∆RF/BB preparation ∆RF/BB preparationis the time for RF and baseband preparation. The following apply:

-	If PRACH bandwidth is outside active UL BWP of serving cell(s) but within one of configured UL BWPs of an active serving cell, the ∆RF/BB preparation ∆RF/BB preparationis the DCI based BWP switching delay specified in clause 8.6.

-	If PRACH bandwidth is not within any of the configured UL BWPs of any active serving cell, ∆RF/BB preparation ∆RF/BB preparation equals to capability value reported in pdcch-RACH-PrepTimeList-r18.

-	Otherwise, ∆RF/BB preparation ∆RF/BB preparation = 0.

-	 Tfirst-valid-RO is the uncertainty of the available PRACH occasion

==============End of change==============