3GPP TSG-RAN4 Meeting #117	R4-2522774

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6062 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on UEIBR Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The IE names in UEIBR core requirements don’t match RRC spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated to use IE names from 38.331 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will not be correct mapping to corresponding parameters |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.3.4, 9.13.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### 9.5.3.4	Event Triggered Reporting for the UE initiated beam management

For a UE configured with a CSI-ReportConfig with csi-ReportUE-IBR-r19 and with dl-OrJointTCI-StateList，with or without eventCountWindow-r19 configuration, the reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively.

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

When eventCountWindow-r19 is not configured,

The event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE transmits first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than TL1-meas_basic + Tfirst_UL_channel, where:

TL1-meas_basic is the maximum of L1-RSRP measurement periods of the reference signals of the beams corresponding to the event

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventTypeUE-IBR-r19 set to ‘event2-r19’, UE shall perform L1-RSRP measurement based on the L1-RSRP measurement period of current beam and new beam(s), respectively.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventTypeUE-IBR-r19 set to ‘event1-r19’, UE shall perform L1-RSRP measurement based on current beam L1-RSRP measurement period.

If the newBeamResourceSet-r19 is based on SSB, and if the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is based on SSB:

- the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-1 or 9.5.4.1-2 assuming TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state for the current beam.

- the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-1 or 9.5.4.1-2 assuming TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

If the reference signal in the indicated TCI state and newBeamResourceSet-r19 are all based on CSI-RS:

- the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-1 or 9.5.4.2-2 assuming TReport. = 0, and TCSI-RS is the periodicity of the reference signal in the indicated TCI state for the current beam.

- the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-1 or 9.5.4.2-2 assuming TReport. = 0, and TCSI-RS is the periodicity of the the reference signal configured by the newBeamResourceSet-r19 for new beams.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventTypeUE-IBR-r19 set to ‘event7-r19’, UE shall perform L1-RSRP measurement based on L1-RSRP measurement period of the beams in active TCI state list and new beams.

If the newBeamResourceSet-r19 is based on SSB, and if the SS/PBCH block that is QCLed with the reference signal corresponding to the activated TCI states with the highest valueOfQ-r19 is also based on SSB:

- the L1-RSRP measurement period of each beam of activated TCI states refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-1 or 9.5.4.1-2 assuming TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal in the activated TCI states or the SS/PBCH block which is QCLed with the reference signal in the activated TCI states.

- the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-1 or 9.5.4.1-2 assuming TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

If the reference signal in the valueOfQ-r19 highest L1-RSRP out of the reference signals among the activated TCI states and newBeamResourceSet-r19 are all based on CSI-RS:

- the L1-RSRP measurement period of each beam of activated TCI states refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-1 or 9.5.4.2-2 assuming TReport. = 0, and TCSI-RS is the periodicity of the reference signal in the activated TCI states.

- the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-1 or 9.5.4.2-2 assuming TReport. = 0, and TCSI-RS is the periodicity of the the reference signal configured by the newBeamResourceSet-r19 for new beams.

Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next PUCCH transmission occasion

Otherwise, if eventCountWindowh-r19 is provided, the event triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel, where

- eventInstanceCount-r19 is configured by the network

- TL1-meas_basic is the measurment period above based on the configuration,

- L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindow-r19).

- The above requriement is applicable when eventDetectionTimeWindow-r19 is not less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic . The requirement does not apply when eventDetectionTimeWindow-r19 is less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic.

==============Second change==============

#### 9.13.3.4	Event triggered reporting for UE initiated beam management

For a UE configured with a CSI-ReportConfig with csi-ReportUE-IBR-r19 and with dl-OrJointTCI-StateList, with or without eventCountWindow-r19 configuration the reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2.

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

When eventCountWindow-r19 is not configured,

The event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report after condition is met over the air and the point when the UE transmits first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than TL1-meas_basic + Tfirst_UL_channel.

TL1-meas_basic is the maximum of L1-RSRP measurement periods of the reference signals of beams corresponding to the event

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventTypeUE-IBR-r19 set to ‘event2-r19’, UE shall perform L1-RSRP measurement based on the L1-RSRP measurement period of current beam and new beam(s) respectively.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventTypeUE-IBR-r19 set to ‘event1-r19’, UE shall perform L1-RSRP measurement based on the L1-RSRP measurement period of current beam.

If the SS/PBCH block is QCLed with the reference signal in the indicated TCI state, the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_SSB_CDP in Table 9.13.4.1-1 or 9.13.4.1-2 with TReport. = 0, and TSSB_CDP is the periodicity of the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state for the current beam

If the reference signals in newBeamResourceSet-r19 are based on SSB for a cell with additional PCI, the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB_CDP in Table 9.13.4.1-1 or 9.13.4.1-2 with TReport. = 0, and TSSB_CDP is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventTypeUE-IBR-r19 set to ‘event7-r19’, UE shall perform L1-RSRP measurement based on L1-RSRP measurement period of current beam and new beam respectively.

If the SS/PBCH block is QCLed with the reference signal corresponding to the activated TCI states with the highest valueOfQ-r19  the L1-RSRP measurement period of the beam refers to TL1-RSRP_Measurement_Period_SSB_CDP as specified in table 9.13.4.1-1 or 9.13.4.1-2 assuming TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal corresponding to the activated TCI states with the highest valueOfQ-r19

If the reference signals in newBeamResourceSet-r19 are based on SSB for a cell with additional PCI, the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB_CDP in Table 9.13.4.1-1 or 9.13.4.1-2 with TReport. = 0, and TSSB_CDP is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

Tfirst UL channel is the time to first PUCCH after L1 reporting condition is triggered

Otherwise, if eventCountWindow-r19 is provided, the event triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel, where

- eventInstanceCount-r19 is configured by the network

- TL1-meas_basic is the measurement period above based on the configuration

- L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindow-r19).

- The above requriement is applicable when eventDetectionTimeWindowLength-r19 is not less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic . The requirement does not apply when eventDetectionTimeWindowh-r19 is less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic.

==============End of change==============