3GPP TSG-RAN4 Meeting #117          		R4-2522778

Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on TC Event-2, FR1, Inter-cell SSB based, no window time based, mode-A (TC5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the CR split in the WF (R4-2514803), this draftCR provides test case on Event-2, FR1, Inter-cell SSB based, no window time based, mode-A (TC5) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce test case on Event-2, FR1, Inter-cell SSB based, no window time based, mode-A (TC5). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.x.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of 1st change >>

A.6.6.x.1	Event triggered reporting for UE initiated beam management for UE configured with Inter-cell SSB based L1-RSRP measurement on FR1 when DRX is not used

A.6.6.x.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event based on L1-RSRP measurement . This test will partly verify the event triggered reporting for UE initiated beam management in clause 9.13.3.4 configured with event2-r19, modeA-r19 and without eventCountWindow-r19 configuration.

Table A.6.6.x.1.1-1: Applicable NR configurations for inter-cell SSB based Event triggered reporting for UE initiated beam management in FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

A.6.6.x.1.2	Test Parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a cell with different PCI from Cell 1 (Cell 2). The test parameters are given in table A.6.6.x.1.2-1and table A.6.6.x.1.2-2 below.

SSB#0 is transmitted on Cell 1. SSB#1 is transmitted on Cell 2. In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSB#1. SSB#0 is for current beam measurement and SSB#1 is for new beam measurement. The test consists of two successive time periods, with time duration of T1 and T2 respectively. At the beginning of T2, SSB#1 starts transmission and the UE is configured for L1-RSRP measurement on SSB#1, and the RSRP configuration of SSB#1 is better than SSB#0 to trigger the event-2 reporting. The test has higher layer parameter timeRestrictionForChannelMeasurements configured in CSI-ReportConfig and additionalPCIList configured in CSI-SSB-ResourceSet.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the the SSB#0 for Cell 1.

Table A.6.6.x.1.2-1: General test parameters

| Parameter |  | Config |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
| Active cell |  | 1~3 |  |  | Cell 1 |
| A cell with additional PCI |  | 1~3 |  |  | Cell 2 |
| RF Channel Number |  | 1~3 |  |  | 1: Cell 1 and Cell 2 |
| SSB ARFCN |  | 1~3 |  |  | freq1 |
| Duplex mode |  | 1 |  |  | FDD |
|  |  | 2 |  |  | TDD |
|  |  | 3 |  |  | TDD |
| TDD Configuration |  | 1 |  |  | N/A |
|  |  | 2 |  |  | TDDConf.1.1 |
|  |  | 3 |  |  | TDDConf.2.1 |
| BWchannel |  | 1 |  | MHz | 10: NPRB,c = 52 |
|  |  | 2 |  |  | 10: NPRB,c = 52 |
|  |  | 3 |  |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  |  | SR.1.1 FDD |
|  |  | 2 |  |  | SR.1.1 TDD |
|  |  | 3 |  |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  |  | CR.1.1 FDD |
|  |  | 2 |  |  | CR.1.1 TDD |
|  |  | 3 |  |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  |  | CCR.1.1 FDD |
|  |  | 2 |  |  | CCR.1.1 TDD |
|  |  | 3 |  |  | CCR.2.1 TDD |
| SSB configuration |  | 1 |  |  | SSB.3 FR1 for Cell 1 and Cell 2 |
|  |  | 2 |  |  | SSB.3 FR1 for Cell 1 and Cell 2 |
|  |  | 3 |  |  | SSB.4 FR1 for Cell 1 and Cell 2 |
| OCNG Patterns |  | 1~3 |  |  | OP.1 |
| Initial BWP Configuration |  | 1~3 |  |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  |  | SMTC.1 |
| TRS Configuration |  | 1 |  |  | TRS.1.1 FDD |
|  |  | 2 |  |  | TRS.1.1 TDD |
|  |  | 3 |  |  | TRS.1.2 TDD |
| DRX configuration |  | 1~3 |  |  | off |
| eventTypeUE-IBR-r19 |  | 1~3 |  |  | event2-r19 |
| eventThreshold-r19 |  | 1~3 |  |  | 1 |
| reportTransmissionMode-r19 |  | 1~3 |  |  | modeA-r19 |
| currentBeamReport-r19 |  | 1~3 |  |  | enabled |
| Timing offset between two SSBs associated to different PCI |  | 1~3 |  |  | <CP |
| T1 |  | 1~3 |  | s | 5 |
| T2 |  | 1~3 |  | s | 1 |
| CP length |  | 1~3 |  |  | Normal |
| PUCCH-Resource-r19 | periodicityAndOffset | 1~2 |  | sl4, 3 |
|  | pucch-Cell |  |  | Cell 1 |
|  | periodicityAndOffset | 3 |  | sl8, 4 |
|  | pucch-Cell |  |  | Cell 1 |
| EPRE ratio of PSS to SSS |  | 1~3 |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Propagation condition |  | 1~3 |  |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |

Table A.6.6.x.1.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 of Cell 1 |  | SSB#1 of Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 7 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -87.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -84.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 7 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.6.x.1.3	Test Requirements

The UE shall send a first PUCCH message using the new UCI type, no later than 20 ms plus X slot from the beginning of time period T2.

- X is 4 slots for Config 1 and 2.

- X is 8 slots for Config 3.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

## << End of 1st change >>