3GPP TSG-RAN4 Meeting # 117	R4-2522802

# Dallas, USA, Nov.17 – 21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on test cases of L1-CLI-RSSI measurement in FR1/FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the work split in WF R4-2515166, the test cases of L1-CLI-RSSI measurement in FR1/FR2 are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the test cases for: L1-CLI-RSSI measurement in FR1 L1-CLI-RSSI measurement in FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases of Rel-19 SBFD are incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.x.2 A.7.6.x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

A.6.6.x.2	L1-CLI-RSSI measurement with DRX with SBFD

A.6.6.x.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-CLI-RSSI measurement with UE configured with two DL subbands in SBFD DUD operation. The RSSI resource CLI-RSSI-MeasResource is configured across two DL subbands. This test will verify the CLI-RSSI measurement requirements in clause 9.18.3 with the testing configurations for NR cells in table A.6.6.x.2.1-1.

Table A.6.6.x.2.1-1: Applicable NR configurations for FR1 L1-CLI-RSSI test

| Configuration | Description |
| --- | --- |
| 1 | NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex mode |

A.6.6.x.2.2	Test Parameters

One cell is deployed in the test, which are FR1 PCell (Cell 1). The test parameters for PCell is given in table A.6.6.x.2.2-1 and A.6.6.x.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-CLI-RSSI measurement on CLI-RSSI-MeasResource and report aperiodically. The test consists of two successive time periods, with time duration of T1 and T2, respectively. The higher layer parameter timeRestrictionForChannelMeasurements is configured to UE in the test. At 640ms after the beginning of time period T2, the DCI triggering L1-CLI-RSSI reporting is sent to UEand UE provides the report back based on the reporting configuration.

There is no measurement gap configured in the test. The L1-CLI-RSSI measurement resource CLI-RSSI-MeasResource configuration is in table A.6.6.x.2.2-3.

Table A.6.6.x.2.2-1: General test parameters for L1-CLI-RSSI reporting for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | NR Cell 1 |  |
| RF Channel Number |  | 1 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.14 FR1 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| CLI-RSSI-MeasResource |  | 1 | A.6.6.x.2.2-3 |  |
| CP length |  | 1 | Normal |  |
| DRX |  | 1 | DRX.7 |  |
| Time offset between DL from serving cell and OCNG from test system | s | 1 | 17.67 |  |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 2 |  |

Table A.6.6.x.2.2-2: NR Cell specific test parameters for L1-CLI-RSSI reporting for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.2.4 |  |
| SBFD configuration |  | 1 | SBFD.2 FR1 |  |
| PDSCH RMC configuration |  | 1 | SR.2.1 TDD |  |
| PUSCH parameters |  | 1 | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.2.1 TDD |  |
| OCNG Patterns Note 3 |  | 1 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |
| on L1-CLI-RSSI measurement resource Note 2 | dBm/30 kHz | 1 | -113 | -105 |
| on L1-CLI-RSSI measurement resource Note 2 | dBm/SCS | 1 | -113 | -105 |
| Io on L1-CLI-RSSI measurement resource | dBm/98.28 MHz | 1 | -77.84 | -69.84 |
| Io on L1-CLI-RSSI measurement resource | dBm/1.08 MHz | 1 | -97.44 | -89.44 |
| Propagation Condition |  | 1 | AWGN |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |  |

Table A.6.6.x.2.2-3: CLI-RSSI-MeasResource measurement resource configuration for measurement reporting

| RSSI-Resource | cli-RSSI-MeasResourceId-r19 | 0 |
| --- | --- | --- |
|  | startPRB-r19 | 0 |
|  | nrofPRBs-r19 | 273 |
|  | startSymbol-r19 | 3 |
|  | nrofSymbols-r19 | 11 |
|  | cli-RSSI-PeriodicityAndOffset-r19 | sl640, 1 |

A.6.6.x.2.3	Test Requirements

Within 650 ms from the beginning of time period T2, the UE shall send L1-CLI-RSSI report. The nominal RSSI used to evaluate the requirement shall be based on Io. The UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots.

The rate of correct events observed during repeated tests shall be at least 90%.

==============Next change==============

A.7.6.x.2	L1-CLI-RSSI measurement with DRX with SBFD

A.7.6.x.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-CLI-RSSI measurement with UE configured with two DL subbands in SBFD DUD operation. The RSSI resource CLI-RSSI-MeasResource is configured across two DL subbands. This test will verify the CLI-RSSI measurement requirements in clause 9.18.3 with the testing configurations for NR cells in table A.7.6.x.2.1-1.

Table A.7.6.x.2.1-1: Applicable NR configurations for FR2 L1-CLI-RSSI test

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SCS, 200 MHz bandwidth, TDD duplex mode |

A.7.6.x.2.2	Test Parameters

One cell is deployed in the test, which are FR2 PCell (Cell 1). The test parameters for PCell is given in table A.7.6.x.2.2-1 and A.7.6.x.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-CLI-RSSI measurement on CLI-RSSI-MeasResource and report aperiodically. The test consists of two successive time periods, with time duration of T1 and T2, respectively. The higher layer parameter timeRestrictionForChannelMeasurements is configured to UE in the test. At 5ms after the beginning of time period T2, the DCI triggering L1-CLI-RSSI reporting is sent to UE and UE provides the report back based on the reporting configuration.

There is no measurement gap configured in the test. The L1-CLI-RSSI measurement resource CLI-RSSI-MeasResource configuration is in table A.7.6.x.2.2-3.

Table A.7.6.x.2.2-1: General test parameters for L1-CLI-RSSI reporting for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | NR Cell 1 |  |
| RF Channel Number |  | 1 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.28 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| CLI-RSSI-MeasResource |  | 1 | A.7.6.x.2.2-3 |  |
| CP length |  | 1 | Normal |  |
| DRX |  | 1 | OFF | Non-DRX |
| Time offset between DL from serving cell and OCNG from test system | s | 1 | 17.67 |  |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 1 |  |

Table A.7.6.x.2.2-2: NR Cell specific test parameters for L1-CLI-RSSI reporting for PCell in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| SBFD configuration |  | 1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |
| PUSCH parameters |  | 1 | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |
| OCNG Patterns Note 3 |  | 1 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |
| Propagation Condition |  | 1 | AWGN |  |
| NOTE 1: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |  |

Table A.7.6.x.2.2-3: NR OTA Cell specific test parameters for CLI-RSSI event triggered reporting for PCell in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| AoA setup |  | 1 | Setup 1 defined in A.3.15.1 |  |
| Beam assumption Note 3 |  | 1 | Fine | Fine |
| on CLI-RSSI measurement resource Note 2 | dBm/15 kHz | 1 | -119 | -108 |
| on CLI-RSSI measurement resource Note 2 | dBm/SCS | 1 | -110 | -99 |
| Io on CLI-RSSI measurement resource | dBm/95.04 MHz | 1 | -81.01 | -70.01 |
| Io on CLI-RSSI measurement resource | dBm/1.08 MHz | 1 | -100.46 | -89.46 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |

Table A.7.6.x.2.2-4: CLI-RSSI-MeasResource measurement resource configuration for measurement reporting

| RSSI-Resource | cli-RSSI-MeasResourceId-r19 | 0 |
| --- | --- | --- |
|  | startPRB-r19 | 0 |
|  | nrofPRBs-r19 | 66 |
|  | startSymbol-r19 | 3 |
|  | nrofSymbols-r19 | 11 |
|  | cli-RSSI-PeriodicityAndOffset-r19 | sl40, 25 |

A.7.6.x.2.3	Test Requirements

Within 7ms from the beginning of time period T2, the UE shall send L1-CLI-RSSI report The nominal RSSI used to evaluate the requirement shall be based on Io. The UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots.

The rate of correct events observed during repeated tests shall be at least 90%.

==============End of change==============