3GPP TSG-RAN4 Meeting # 117	R4-2522815

# Dallas, USA, 17th Nov – 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR Mapping Tables for CJTC reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 has agreed to introduce mapping tables for CJT calibration reporting for frequency and time offset. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added mapping tables for delay offset and frequency offset reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mapping of measurement will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 10.1.X 	CJT Calibration Delay Offset Measurement Report Mapping

The reporting range of delay offset is defined from 0 to AD, where AD∈0.5CP,CP. The number of levels is defined by MD, where MD∈{32, 64, 128, 256}. The mapping of measured quantity is defined in tables 10.1.4X-1. The range in the signaling may be larger than the guaranteed accuracy range.

Table 10.1.X-1 Mapping of cjtc-Dd

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC_D_00 | 0 ≤ CJTC-Dd < AD MD-1 | Fraction of CP |
| CJTC_D _01 | AD MD-1≤ CJTC-Dd < 2*AD MD-1 | Fraction of CP |
| CJTC_D _02 | 2*AD MD-1≤ CJTC-Dd < 3*AD MD-1 | Fraction of CP |
| … | … | … |
| CJTC_D _MD-2 | (MD-2)*AD MD-1≤ CJTC-Dd < AD | Fraction of CP |
| CJTC_D _MD-1 | AD £ CJTC-Dd | Fraction of CP |

### 10.1.Y	 CJT Calibration Frequency Offset Measurement Report Mapping

The reporting range of frequency offset is defined from 0 to AFO, where AFO∈0.1ppm,0.2ppm. The number of levels is defined by MFO, where MFO∈{16, 32, 256}. The mapping of measured quantity is defined in tables 10.1.Y-1. The range in the signaling may be larger than the guaranteed accuracy range.

Table 10.1.Y-1 Mapping of cjtc-F

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC_F_00 | 0 ≤ CJTC-F < AFO 2(MFO-1) | ppm |
| CJTC_F _01 | AFO 2(MFO-1)≤ CJTC-F < 3*AFO 2(MFO-1) | ppm |
| CJTC_F _02 | 3*AFO 2(MFO-1)≤ CJTC-F < 5*AFO 2(MFO-1) | ppm |
| … | … | … |
| CJTC_F _MFO-2 | (2*MFO-5)*AFO 2(MFO-1)≤ CJTC-F | ppm |
| CJTC_F _MFO-1 | 'Invalid' | ppm |

==============End of change==============