3GPP TSG-RAN4 Meeting #117	R4-2522825

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on introducing TC for OD-SIB1 based Cell reselection FR1 to FR1 target Cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Per agreement in R4-2514809, this CR is to introduce TC for OD-SIB1 based Cell reselection FR1 to FR1 target Cell |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce TC for OD-SIB1 based Cell reselection FR1 to FR1 target Cell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements can not be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.1.1.X1 (new) and A.6.1.1.X2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.6.1.1.X1	Cell reselection to FR1 intra-frequency NR cell with OD-SIB1

A.6.1.1.X1.1	Test Purpose and Environment

This test is to verify the requirement for the intra-frequency NR cell reselection requirements specified in clause 4.2.2.3.

A.6.1.1.X1.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.6.1.1.X1.2-1, A.6.1.1.X1.2-2 and A.6.1.1.X1.2-3. The test consists of one time period with time duration of T1. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2. Cell2 is an NES Cell with OD-SIB1, and OD-SIB1-Config of Cell2 is provided in SIB26 of Cell1.

Table A.6.1.1.X1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.1.1.X1.2-2: General test parameters for intra-frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell 1 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell 2 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell 1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 40 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.X1.2-3: Cell specific test parameters for intra-frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T1 |
| TDD configuration |  | 1 | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC |  | 1 | SR.1.1 FDD | SR.1.1 FDD |
| configuration |  | 2 | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET |  | 1 | CR.1.1 FDD | CR.1.1 FDD |
| RMC configuration |  | 2 | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD | CCR.1.1 FDD |
| RMC configuration |  | 2 | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in clause A.3.2.1 | OP.1 defined in clause A.3.2.1 |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 | DLBWP.0.1 |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 | ULBWP.0.1 |
| RLM-RS |  | 1, 2, 3 | SSB | SSB |
| Qrxlevmin | dBm/SCS | 1, 2 | -130 | -130 |
|  |  | 3 | -127 | -127 |
| Pcompensation | dB | 1, 2, 3 | 0 | 0 |
| Qhysts | dB | 1, 2, 3 | 0 | 0 |
| Qoffsets, n | dB | 1, 2, 3 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 |  | SS-RSRP |  | SS-RSRP |
|  | dB | 1 | -3.11 | 2.79 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
|  | dB | 1 | 13 | 16 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -82 |
|  |  | 2 | -85 | -82 |
|  |  | 3 | -82 | -79 |
| Io | dBm/9.36 MHz | 1 |  | -52.21 |  | -52.21 |
|  | dBm/9.36 MHz | 2 |  | -52.21 |  | -52.21 |
|  | dBm/38.16 MHz | 3 |  | -46.12 |  | -46.12 |
| Treselection | s | 1, 2, 3 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | 60 | 60 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

A.6.1.1.X1.3	Test Requirements

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to an already detected cell shall be less than Tevaluate, NR_Intra + TSI-NR + TOD-SIB1.

Tevaluate, NR_Intra	See table 4.2.2.3-1 in clause 4.2.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

TOD-SIB1 is the time for OD-SIB1 acuqisition, whichi is assumed to be [1] second in the test case.

The rate of correct cell reselections observed during repeated tests shall be at least 90 %.

<End of Change 1>

<Start of Change 2>

A.6.1.1.X2	Cell reselection to FR1 inter-frequency NR cell with OD-SIB1

A.6.1.1.X2.1	Test Purpose and Environment

This test is to verify the requirement for the inter-frequency NR cell reselection requirements specified in clause 4.2.2.3.

A.6.1.1.X2.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.6.1.1.X2.2-1, A.6.1.1.X2.2-2 and A.6.1.1.X2.2-3. The test consists of one time period with time duration of T1. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2. Cell2 is an NES Cell with OD-SIB1, and OD-SIB1-Config of Cell2 is provided in SIB26 of Cell1.

Table A.6.1.1.X2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.1.1.X2.2-2: General test parameters for inter-frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell 1 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell 2 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell 1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 40 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.X2.2-3: Cell specific test parameters for inter-frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T1 |
| TDD configuration |  | 1 | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC |  | 1 | SR.1.1 FDD | SR.1.1 FDD |
| configuration |  | 2 | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET |  | 1 | CR.1.1 FDD | CR.1.1 FDD |
| RMC configuration |  | 2 | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD | CCR.1.1 FDD |
| RMC configuration |  | 2 | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in clause A.3.2.1 | OP.1 defined in clause A.3.2.1 |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 | DLBWP.0.1 |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 | ULBWP.0.1 |
| RLM-RS |  | 1, 2, 3 | SSB | SSB |
| Qrxlevmin | dBm/SCS | 1, 2 | -130 | -130 |
|  |  | 3 | -127 | -127 |
| Pcompensation | dB | 1, 2, 3 | 0 | 0 |
| Qhysts | dB | 1, 2, 3 | 0 | 0 |
| Qoffsets, n | dB | 1, 2, 3 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 |  | SS-RSRP |  | SS-RSRP |
|  | dB | 1 | 14 | 12 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
|  | dB | 1 | 14 | 12 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -86 |
|  |  | 2 | -84 | -86 |
|  |  | 3 | -81 | -83 |
| Io | dBm/9.36 MHz | 1 |  | -55.88 |  | -57.78 |
|  | dBm/9.36 MHz | 2 |  | -55.88 |  | -57.78 |
|  | dBm/38.16 MHz | 3 |  | -49.79 |  | -51.69 |
| Treselection | s | 1, 2, 3 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | 60 | 60 |
| SnonintrasearchP | dB | 1, 2, 3 | 50 | 50 |
| Threshx, highP | dB | 1, 2, 3 | 48 | 48 |
| Threshserving, lowP | dB | 1, 2, 3 | 44 | 44 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

A.6.1.1.X2.3	Test Requirements

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to an already detected cell shall be less than Tevaluate, NR_Inter + TSI-NR + TOD-SIB1.

Tevaluate, NR_Inter	See table 4.2.2.4-1 in clause 4.2.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

TOD-SIB1 is the time for OD-SIB1 acuqisition, whichi is assumed to be [1] second in the test case.

The rate of correct cell reselections observed during repeated tests shall be at least 90 %.

<End of Change 2>