3GPP TSG-RAN WG4 Meeting #117	R4-2522827

Dallas, USA, Nov. 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6209 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for network energy savings enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CR for Enhancements of network energy savings. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Interruption at OD-SSB activation/deactivation in NE-DC section is missing. - SCell activation Change the SFN offset and SSB offset to in the same helf frame of the serving cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirements for Enhancements of network energy savings would be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.2.x, 8.3.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------- CHANGE #1 ------------------------------

##### 8.2.3.2.x	Interruptions at OD-SSB activation/deactivation

The requirements in this clause shall apply for the UE configured with E-UTRA PSCell and one SCell.

When OD-SSB on one deactivated SCell in MCG is activated or deactivated the UE is allowed an interruption on any serving cell in MCG, provided that the OD-SSB activation/deactivation command and SCell activation/deactivation command are not received at the same time:

-	of up to X2 slots, if the active serving cell is not in the same band as any of the SCells on which OD-SSB is being activated or deactivated, or

-	of up to Y2 slots if the active serving cells are in the same band as any of the SCells on which OD-SSB is being activated or deactivated, provided the cell specific reference signals from the active serving cells and the SCells on which OD-SSB is being activated or deactivated are available in the same slot.

Where X2 and Y2 are specified in table 8.2.3.2.x-1.

Table 8.2.3.2.x-1: Interruption length X2 and Y2 at OD-SSB activation/deactivation

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 1 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |  |

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #2 ------------------------------

### 8.3.19	OD-SSB based SCell Activation Delay Requirement for Deactivated SCell

The requirements in this clause shall apply for the UE supporting On-demand SSB operation and configured with at least one downlink OD-SSB SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one OD-SSB SCell is being activated.

The requirements in the clause 8.3.x1 apply, provided the following conditions are met:

-	an OD-SSB transmission is activated in a configured DL BWP where a first SSB transmission is not configured, or

-	an OD-SSB transmission is activated in a configured DL BWP where a first SSB transmission is configured and the OD-SSB cannot be used to obtain SIB1.

Where the first SSB transmission is specified as in TS 38.213 and have the following relations with the OD-SSB transmission:

-	are with the same physical cell identity, and

-	are in the same frequency location, and

-	are with same sub-carrier spacing, and

-	are with the same downlink transmit power, and

-	are quasi co-located with respect to Doppler shift, Doppler spread, average gain, average delay, delay spread and, when applicable, spatial RX parameters, when they have a same SSB index.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall perform SCell activation procedure based on the transmitted SSBs, where the transmitted SSBs are

-	OD-SSBs being transmitted in the configured DL BWP if no first SSB transmission is configured, or

-	the union of OD-SSBs and first SSBs if first SSBs are transmitted and the time instance when receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command, or

-	first SSBs only if first SSBs are transmitted but no OD-SSB transmission during the SCell activation.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tactivation_time is the SCell activation delay in milliseconds.

If the SCell is known and belongs to FR1, Tactivation_time is:

-	TFirst, OD-SSB + T∆ + 5 ms, if the measurement period of the SCell being activated is equal to or smaller than 2400 ms.

-	TFirst, OD-SSB + Trs, OD-SSB + T∆ + 5 ms, if the measurement period of the SCell being activated is larger than 2400 ms.

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met

-	only one SSB is indicated in ‘od-ssb-PositionInBurst’ if provided; otherwise, in ssb-PositionsInBurst, or

-	 multiple SSBs are indicated in ‘od-ssb-PositionInBurst’ if provided; otherwise, in ssb-PositionsInBurst and TCI indication of SSBs is provided in same MAC PDU with SCell activation,

provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time is:

-	TFirst, OD-SSB + 2* Trs,OD-SSB + 5 ms, if the following conditions are met,

-	the SCell is contiguous to an active serving cell in the same band, and

-	its od-ssb-PositionInBurst if provided; otherwise, ssb-PositionsInBurst provided is the same as the one of contiguous FR1 active serving cell, and

-	OD-SSB is in the same half-frame on the SCell and the SSB of the contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is smaller than or equal to 260 ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6 dB;

-	TFirst, OD-SSB + 3*Trs, OD-SSB + 5 ms, otherwise.

Otherwise, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time is:

-	6 ms + TFirst, OD-SSB + 2*Trs, OD-SSB + TL1-RSRP, measure, OD-SSB + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming, OD-SSB + 2 ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

-	3 ms + TFirst, OD-SSB + 2*Trs, OD-SSB + TL1-RSRP, measure, OD-SSB + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming, OD-SSB, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

-	However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

-	the SCell is contiguous to an active serving cell in the same band, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its od-ssb-PositionInBurst if provided; otherwise, ssb-PositionsInBurst provided is same as the one of contiguous FR1 active serving cell, and

-	OD-SSB is in the same half-frame on the SCell and the SSB of the contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is larger than 260 ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6 dB;

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirst OD_SSB + 5ms provided:

-	the SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3, and

-	the parameter od-ssb-PositionsInBurst if provided; otherwise, ssb-PositionsInBurst provided if the SCell being activated is same for the serving cell(s), and

-	the SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell.

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1 or in FR2:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3 ms + max(Tuncertainty_MAC + TFineTiming, OD-SSB + 2 ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0, if

-	UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time, or

-	Provided this is the initial activation after SCell addition, and

-	TCI-ActivatedConfig is configured for the SCell, and UE receives the SCell activation command and semi-persistent CSI-RS activation command at the same time.

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max(Tuncertainty_MAC + 5 ms + TFineTiming, OD-SSB, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0, if

-	UE receives the SCell activation command and TCI state activation commands at the same time, or

-	Provided this is the initial activation after SCell addition, and

-	UE receives the SCell activation command and TCI-ActivatedConfig is configured for the SCell.

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, then Tactivation_time is:

-	6 ms + TFirst, OD-SSB + 23*Trs, OD-SSB  + TL1-RSRP, measure, OD-SSB + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming, OD-SSB  + 2 ms, Tuncertainty_SP), or

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, then Tactivation_time is:

-	3 ms + TFirst OD-SSB + 23*Trs, OD-SSB + TL1-RSRP, measure, OD-SSB + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming, OD-SSB), (Tuncertainty_RRC + TRRC_delay)}.

where,

Trs, OD-SSB is the periodicity of the transmitted SSB.

T∆ is the additional time for fine timing tracking and acquiring full timing information of the SCell. T∆ = 2*Trs, OD-SSB ms for SCell operating with 12 PRB SSB BW. Otherwise, T∆ = 0 ms.

TFirst, OD-SSB is the time to the end of the first complete transmitted SSB burst after slot n + THARQ+3msNR slot length, where the transmitted SSB is

-	OD-SSB in the configured DL BWP if no first SSB transmission is configured, or

-	the union of OD-SSB and first SSB if first SSB is transmitted and the time instance when receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command, or

-	first SSB only if first SSB is transmitted but no OD-SSB transmission during the SCell activation.

TFineTiming, OD-SSB is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete transmited SSB burst corresponding to the TCI state.

TL1-RSRP, measure, OD-SSB is the transmited SSB based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB based on applicability as defined in clause 9.5.4 assuming M=1 and TReport=0

TL1-RSRP, report is delay of acquiring CSI reporting resources.

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

-	the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) or the SSB used for SCell activation which has the same SSB index of the SSB measured also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with the indexes or the SSBs used for SCell activation with the same indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown.

The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The starting point of an interruption window on SpCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length  and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is:

-	0, if Tactivation_time is 3 ms;

-	TFirst, OD-SSB, for any scenario where Tactivation_time includes TFirst, OD-SSB;

-	Tuncertainty_MAC + TFineTiming, OD-SSB, for any scenario where Tactivation_time includes only TFineTiming, OD-SSB and no TFirst, OD-SSB.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The requirements in this clause and requirements on interruption due to SCell activation in clause 8.2 apply provided that the first SSB and the OD-SSB of the to-be-activated SCell is within the first active DL BWP of the SCell.

No OD-SSB based SCell activation requirement applies if OD-SSB is transmitting upon UE receiving the SCell activation command but NW deactivates the OD-SSB earlier than the slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

-------------------- END OF CHANGES--------------------------