3GPP TSG-RAN WG4 Meeting #117	R4-2522943

# Dallas, TX, USA, 17th – 21st, November, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | - | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | dCR to TS 38.141-2 on SBFD TX conformance tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | dCR to TS 38.141-2 to add TX radiated conformance test for SBFD-capable BS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add TX radiated conformance test clauses for SBFD-capable BS, including - General clause for radiated TX conformance testing - Radiated TX power for sBFD - OTA BS output power for SBFD - Ouput power dynamics for SBFD - Transmit ON/OFF power for SBFD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The relevant clauses of TX radiated conformance tests for SBFD-capable BS are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

## 9.2 Radiated transmitter characteristics for SBFD-capable BS

### 9.2.1	General

### 9.2.2	Radiated transmit power for SBFD

The minimum requirement for BS type 1-H, BS type 1-O and BS type 2-O is specified in TS 38.104 [2], clause 12.5.2. The test purpose defined in clause 6.2.3 shall apply. The method of test defined in clause 6.2.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.2.5 shall apply.

### 9.2.3	OTA base station output power for SBFD

The minimum requirement for BS type 1-H, BS type 1-O and BS type 2-O is specified in TS 38.104 [2], clause 12.5.3. The test purpose defined in clause 6.3.3 shall apply. The method of test defined in clause 6.3.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.3.5 shall apply.

### 9.2.4	OTA output power dynamics for SBFD

#### 9.2.4.1	General

The requirements in clause 9.2.4 apply during SBFD symbol. Transmit signal quality (as specified in clause 9.2.6) shall be maintained for the output power dynamics requirements of this clause.

The OTA output power requirements are directional requirements and apply to the beam peak directions over the OTA peak directions set.

#### 9.2.4.2		OTA RE power control dynamic range

The minimum requirement for BS type 1-H, BS type 1-O and BS type 2-O is specified in TS 38.104 [2], clause 12.5.4.2. The test method defined in clause 6.4.2.3 shall apply.

#### 9.2.4.3	OTA total power dynamic range

The minimum requirement for BS type 1-H, BS type 1-O and BS type 2-O is specified in TS 38.104 [2], clause 12.5.4.3. The test purpose defined in clause 6.4.3.3 shall apply. The method of test defined in clause 6.4.3.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.4.3.5 shall apply.

### 9.2.5	Transmit ON/OFF power for SBFD

#### 9.2.5.1	Transmitter OFF power for SBFD

The minimum requirement for BS type 1-H, BS type 1-O and BS type 2-O is specified in TS 38.104 [2], clause 12.5.5.1. For the transmitter OFF power requirements for non-SBFD symbols, the same requirement as specified in clause 6.5.1 shall be applied. Requirement is not applicable for SBFD symbols.

==============End of change==============