3GPP TSG-RAN4 Meeting #117	R4-2522944

# Dallas, USA,  17th – 21st November 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | draftCR | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS 38.141-1 on test requirements for SBFD-capable BS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of test requirements for SBFD-capable BS to TS 38.141-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 9.2.5 Transmitted signal quality for SBFD 9.2.6 Unwanted emissions for SBFD 9.2.7 Transmitter intermodulation for SBFD 9.2.8 Transient period between SBFD and non-SBFD operation 9.2.9 In-channel adjacent subband leakage for SBFD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SBFD-capable BS will not have test requirements for Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses introduced to specification: 9.2.5, 9.2.6, 9.2.7, 9.2.8, 9.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 9.2.5	Transmitted signal quality for SBFD

9.2.5.1		General

The general description for transmitted signal quality as clause 6.5.1 shall be applied.

9.2.5.2		Frequency error for SBFD

The minimum requirement for BS type 1-H is specified in TS 38.104 [2], clause 12.2.5.2, and shall apply per single-band connector. The test purpose defined in clause 6.5.2.3 shall apply. The method of test defined in clause 6.5.2.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.5.2.5 shall apply.

9.2.5.3		Modulation quality for SBFD

The minimum requirement for BS type 1-H is specified in TS 38.104 [2], clause 12.2.5.2, and shall apply per single-band connector. The test purpose defined in clause 6.5.3.3 shall apply. The method of test defined in clause 6.5.3.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.5.3.5 shall apply.

9.2.5.4		Time alignment error for SBFD

The minimum requirement for BS type 1-H is specified in TS 38.104 [2], clause 12.2.5.2, and shall apply per single-band connector. The test purpose defined in clause 6.5.4.3 shall apply. The method of test defined in clause 6.5.4.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.5.4.5 shall apply.

==============Next change==============

### 9.2.6 	Unwanted emissions for SBFD

9.2.6.1		General

The general description for unwanted emissions as clause 6.6.1 shall be applied.

9.2.6.2		Occupied bandwidth

The minimum requirement for BS type 1-H is specified in TS 38.104 [2], clause 12.2.6.2, and shall apply per single-band connector. The test purpose defined in clause 6.6.2.3 shall apply. The method of test defined in clause 6.6.2.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.6.2.5 shall apply.

9.2.6.3		Adjacent Channel Leakage Power Ratio (ACLR) for SBFD

The minimum requirement for BS type 1-H is specified in TS 38.104 [2], clause 12.2.6.2, and shall apply per single-band connector. The test purpose defined in clause 6.6.3.3 shall apply. The method of test defined in clause 6.6.3.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.6.3.5 shall apply.

9.2.6.4		Operating band unwanted emissions for SBFD

The minimum requirement for BS type 1-H is specified in TS 38.104 [2], clause 12.2.6.2, and shall apply per single-band connector. The test purpose defined in clause 6.6.4.3 shall apply. The method of test defined in clause 6.6.4.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.6.4.5 shall apply.

9.2.6.5		Transmitter spurious emissions for SBFD

The minimum requirement for BS type 1-H is specified in TS 38.104 [2], clause 12.2.6.2, and shall apply per single-band connector. The test purpose defined in clause 6.6.5.3 shall apply. The method of test defined in clause 6.6.5.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.6.5.5 shall apply.

==============Next change==============

### 9.2.7	Transmitter intermodulation for SBFD

The minimum requirement for BS type 1-H is specified in TS 38.104 [2], clause 12.2.7.2, and shall apply per single-band connector. The test purpose defined in clause 6.7.3 shall apply. The method of test defined in clause 6.7.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.7.5 shall apply.

==============Next change==============

### 9.2.8	Transient period between SBFD and non-SBFD operation

### 9.2.8.1	Definition and applicability

Transmitter transient period between SBFD and non-SBFD operation requirements apply only to SBFD-capable BS.

The transmitter transient period between SBFD and non-SBFD operation is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period between SBFD and non-SBFD operation is illustrated in figure 9.2.8.1-1.

Transmitter output power frequency regions for transition

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 9.2.8.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period between SBFD and non-SBFD operation

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases:

Table 9.2.8.1-1: Transmitter transient period between SBFD and non-SBFD operation

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| SBFD symbol to DL symbol | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| SBFD symbol to UL symbol | SBFD DL subband | BS transmitter ON-to-OFF |
| DL symbol to SBFD symbol | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| UL symbol to SBFD symbol | SBFD DL subband | BS transmitter OFF-to-ON |

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

9.2.8.2	Minimum requirement

The minimum requirement for BS type 1-H is specified in TS 38.104 [2], clause 12.2.8.2.

9.2.8.3	Test purpose

The purpose of this test is to verify the transmitter transient periods between SBFD and non-SBFD operation are within the limits of the minimum requirements.

9.2.8.4	Method of test

##### 9.2.8.4.1	Initial conditions

Test environment:

-	normal; see annex B.2.

RF channels to be tested for single carrier: see clause 4.9.1.

RF bandwidth positions to be tested for multi-carrier: see clause 4.9.1.

##### 9.2.8.4.2	Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.3.1 for BS type 1-H. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in annex D.3.1 for BS type 1-H. All TAB connectors not under test shall be terminated.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

2)	For switching from SBFD symbol to DL symbol or from DL symbol to SBFD symbol, use TM1.1 to represent non-SBFD DL operation, and use TM1.1_SBFD_DU/UD/DUD to represent SBFD operation.

For switching from SBFD symbol to UL symbol or from UL symbol to SBFD symbol, use TM1.1_SBFD_DU/UD/DUD to represent SBFD operation.

For single carrier set the TAB connector under test to transmit according to the test configuration NRTC_SBFD in clause 4.8 using the corresponding test models or set of physical channels in clause 4.9.2 at manufacturers declared rated carrier output power per TAB connector (PSBFD,rated,c,TABC, D.134).

For a connector under test declared to be capable of multi-carrier operation (D.132, D.133, D.135) set the connector under test to transmit on all carriers configured using the test configuration NRTC_SBFD and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models or set of physical channels in clause 4.9.2.

3)	Measure the mean power spectral density over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the TAB connector centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period – 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

9.2.8.5	Test requirements

The measured mean power spectral density according to clause 6.4.2.4.2 shall be less than -83 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The measured mean power spectral density according to clause 6.4.2.4.2 shall be less than -82.5 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 7.125 GHz in licensed spectrum only.

==============Next change==============

### 9.2.9	In-channel adjacent subband leakage for SBFD

9.2.9.1	Definition and applicability

In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of SBFD UL subband within NR carrier.

The requirement shall apply during the transmitter ON period.

9.2.9.2	Minimum requirement

The minimum requirement for BS type 1-H is specified in TS 38.104 [2], clause 12.2.9.3.

9.2.9.3	Test purpose

To verify that the in-channel adjacent subband leakage requirement shall be met as specified by the minimum requirement.

9.2.9.4	Method of test

##### 9.2.9.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: see clause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier: see clause 4.9.1.

##### 9.2.9.4.2	Procedure

For BS type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.3.1 for BS type 1-H. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the single-band connector under test to measurement equipment as shown in annex D.3.1 for BS type 1-H. All connectors not under test shall be terminated.

The measurement device characteristics shall be:

-	Measurement filter bandwidth: defined in clause 9.2.9.5.

-	Detection mode: true RMS voltage or true average power.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1.

2)	For a connectors declared to be capable of single carrier operation only (D.132), set the representative connectors under test to transmit according to the test configuration NRTC_SBFD in clause 4.8 using the corresponding test models NR-FR1-TM 1.1_SBFD_DU/UD/DUD in clause 4.9.2 at rated carrier output power PSBFD,rated,c,TABC for BS type 1-H (D.134).

For a connector under test declared to be capable of multi-carrier operation (D.132, D.133,D.135) set the connector under test to transmit on all carriers configured using the test configuration NRTC_SBFD and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models NR-FR1-TM 1.1_SBFD_DU/UD/DUD or set of physical channels in clause 4.9.2.

3)	Measure ICASL.

4)	Repeat the test with the channel set-up according to NR-FR1-TM 1.2_SBFD_DU/UD/DUD in clause 4.9.2.

9.2.9.5	Test requirements

##### 9.2.9.5.1	General requirements

The ACLR requirements in clause 9.2.9.5.2 shall apply as described in clauses 9.2.9.5.3.

##### 9.2.9.5.2	Limits and basic limits

For SBFD operation in the unpaired spectrum, the ICASL shall be less than or equal to the value specified in table 9.2.9.5.2-1.

Table 9.2.9.5.2-1: SBFD-capable base station ICASL absolute basic limit

| SBFD BS category / BS class | ICASL absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

Table 9.2.9.5.2-2: Filter parameters for the UL subband

| RAT | Filter on the assigned UL subband and corresponding filter bandwidth |
| --- | --- |
| NR | Square (the declared bandwidth of UL subband) |

#### 9.2.9.5.3	SBFD-capable BS type 1-H

The ICASL requirements are that for each TAB connector TX min cell group and each applicable basic limit in clause 12.2.9.2, the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed a BS limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell).

NOTE:	Conformance to the in-channel adjacent subband leakage requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the in-channel adjacent subband leakage power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The  in-channel adjacent subband leakage power at each TAB connector shall be less than or equal to the limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

==============End of change==============