3GPP TSG-RAN WG4 Meeting #117		    								               R4-2522978

Dallas, USA, November 17-21, 2025

Agenda item:			7.8.1

Source:	Moderator (Apple)

Title:	Way Forward for [117][320] NR_low_band_CA_switching

Document for:	Approval

# Way Forward

Issue 1: Common understanding of the UE reference architecture

RAN4 has reached the common understanding of the following aspects related to the UE reference architecture:

-	For CA_n28A-n67A:

-	Specify requirements assuming a filter implementation that takes the Region 1 frequency range restriction for band n28 into account

-	The following note can be added to the specification in relation to this band combination: “the range of operation in band n28 is restricted to 703-733 MHz UL and 758-788 MHz DL”

-	For other potential NR_LBCA_Sw_Enh CA combinations where the frequency gap between DL bands is  not small enough or there is potential interference in the gap, RAN4 assumes the following are needed for Case 3 operation:

-	A CR to TR38.768 is needed to capture these aspects, together with those agreed during RAN4 #116bis in R4-2515156, in the next meeting

Issue 2: Switching time mask between Case 1 and Case 3

-	The following updates to the big picture of the switching time mask (to be introduced into the TR) are needed:

-	;Companies are encouraged to check the following proposals based on the paper R4-2520092.

Figure 1 The whole picture on switching time mask for RTDS2P ≤ 0 Figure 2 The whole picture on switching time mask for RTDS2P > 0 1) From UL/DL FDD band to Dual-DL FDD+SDL bands: FFS 2) From Dual-DL FDD+SDL bands to UL/DL FDD band: Actual switching gap length ≥ ceil[ ( Tswitch + TTA * 106+ l * RTDS2P + FDD UL leading transient period ) / (1/14*103) ] UL transient period is assumed as 10us. Where RTDS2P is the receiving timing difference among the closest DL slot timing boundaries between SCell and PCell, with the range of negative MRTD to positive MRTD. And MRTD = 3us assuming that the FDD and SDL carriers are colocated. In addition, RTDS2P = - RTDP2S. If RTDS2P ≥ 0, then l = 1; if RTDS2P < 0, then l = 0.

-	A CR to TR38.768 is needed to capture these aspects in the next meeting

Issue 3: Open issues related to the switching time mask for switching between Case 1 and Case 3

-	companies are encouraged to share views on if there is RAN1 impact to clarify the switching pattern for Case 3 in the next meeting

-	After approving the big picture of the switching time mask (see Issue 2), RAN4 may send a liaison with the information to RAN1 with a request to take the information into account during the next meeting.

-	RAN4 can further discuss whether an update of the RMC is needed to test switching between Case 1 and Case 3 with consideration of RAN1 impact conclusion.

Issue 4: Summary of impacted Rx requirements

-	Inter-band CA operating bands (already defined in Rel-19)

-	NR CA configurations and bandwidth combinations (already defined in Rel-19)

-	∆TIB,c for Inter-band CA (No need to define in Rel-20)

-	Transmit ON/OFF time mask for inter-band CA (reuse in Rel-19)

-	∆RIB,c due to CA

-	Blocking requirement

Issue 5: REFSENS and ΔRIB,c

-	For CA_n28A-n67A, specifying ΔRIB,c to accommodate potential higher insertion losses for the combined filter relative to the single-band filters; the relaxation value is FFS

-	For CA_n12A-n29A, FFS if we can reuse single-carrier REFSENS or not.

-	Consider the fact that if the case 3 operation is affected by blocker, both PCell (n12) and SCell (n29) are impacted, while for Rel-19 LBCA switching, only SCell is affected.

Issue 6: ACS and blocking

-	For CA_n28A-n67A:

-	Considering the limitation of wider DL filters in suppressing closer UL, the performance of ACS can be further analyzed

-	Specify the IBB and OBB blocking requirements based on the Region 1 restricted operating range of band n28

-	For CA_n12A-n29A:

-	Specify ACS, IBB, OBB based on unrestricted operation in both bands

-	NOTE: unrestricted operation subject to BS-BS coexistence discussion in maintenance