3GPP TSG-RAN WG4 Meeting #117	R4-2523041

Dallas, USA, November 17 – November 21, 2025

Title: 	           Way Forward for [116bis][316] NR_UE_RF_Ph5

Agenda Item:	7.1.4

Source: 	Murata, Skyworks, MediaTek, Qualcomm, Apple, Vivo, Nokia, […]

Document for:	Approval

# 1. Background

Various Contributions were made on the method to predict PC1.5 2TX RSD for FDD bands.

- Proposals

  - Option 1: Utilize 1Tx PC2 RSD to derive PC1.5 RSD via the following steps (Apple)

    - Derive the interfering Tx noise in primary path from the existing 1Tx PC2 RSD specifications

    - Add 3dB to the calculated PC2 Tx noise to generate the noise for PC1.5

    - Use the resulting PC1.5 Tx interfering noise from above to calculate the RSD for PC1.5

  - Option 2: Add extra 3 dB to 2Tx PC2 RSD to get PC1.5 2Tx RSD (Huawei)

  - Option 3: Only consider duplexer isolation not antenna isolation (LGE)

  - Option 4: Calculate PC1.5 RSD via the following steps (Murata)

    - Derive the interfering Tx noise in primary path from the existing 1 Tx PC3 REFSENS and 1Tx PC2 RSD specifications. Assume 6dB interference balance between primary and diversity.

    - Utilize the same tx noise as the primary and diversity path are symmetrical, to generate the noise for PC1.5

    - Use the resulting PC1.5 Tx interfering noise from above to calculate the RSD for PC1.5

  - Option 5: Calculate PC1.5 RSD via the following steps (Skyworks)

    - Derive the interfering Tx noise in primary path from the existing 1 Tx PC3 REFSENS specifications. Assume 6dB interference balance between primary and diversity.

    - Add 3dB to the calculated PC3 Tx noise and apply it to both primary and diversity branch i.e., with an imbalance of 0 dB,

    - Use the resulting PC1.5 Tx interfering noise from above to calculate the RSD for PC1.5

  - Option 6: Add extra X dB to PC3 RSD to get PC1.5 2Tx RSD according to supporting CBWs for example bands (MediaTek)

# 2. WF

The objective of this WF 2-fold:

Firstly, investigate if it is possible to have a unified approach to predict 2TX PC1.5 RSD from the 1TX/2TX PC2 RSD in the specification or directly from PC3. The approach could be either a procedure or a simple delta value that can be applied to other FDD bands. Determine the feasibility of applying a method or applying a simple delta value.

Consider the following starting point assumptions:

- Baseline PC3 REFSENS scalable to the desired BW assuming no TX OOB and TX blocking effect.

- Existing 1TX/2TX PC2 RSD agreed vales in the specification.

- Existing RFFE architecture used for PC2 1TX/2TX RSD. (ie. Existing filter isolations still apply)

- Interference Imbalance [Consider 6dB, 8dB as starting values].

The following methodology examples were already provided this meeting:

Methodology example 1:

- For PC3 reference, a 6dB main to diversity imbalance is assumed and based on smallest CBW ideal REFSENS scaling, from which the equivalent de-sense and Main PC3 UL interference is calculated.

- Increase the main UL interference by 3dB and apply the increased interference level to both main and diversity RX chains to derive the 2TX PC1.5 RSD.

Methodology example 2:

- For PC3 reference, a 6dB main to diversity imbalance is assumed and based on smallest CBW ideal REFSENS scaling, from which the UL interference is calculated to arrive at the PC2 1TX RSD.

- Log sum the {UL interference + 6dB reduced UL interference} and apply the interference log sum to both the main and diversity RX chains to derive the 2TX PC1.5 RSD.

Methodology example 3: […]

Secondly, determine if a generalized LUT approach is possible to predict the 1TX PC2, 2TX PC2, and 2TX PC1.5 RSD. There could be deviations from the already agreed values in the specification for 1TX PC2 and 2TX PC2, and it is up to the group to decide whether the deviation is acceptable for the sake of simplicity.

- Baseline PC3 REFSENS scalable to the desired BW assuming no TX OOB and TX blocking effect.

- Existing RFFE architecture used for PC2 1TX/2TX RSD. (ie. Existing filter isolations still apply)

- Interference Imbalance [Consider 6dB, 8dB as starting values].

Thirdly, consider the following aspects:

- Evaluation criteria for the method: Check that the derived PC1.5 2Tx RSD is larger than the PC2 1Tx and 2Tx RSD.

- When UL interference cannot be extracted from the PC3 REFSENS but a PC2 RSD is specified, consider using PC2 as a starting point as an exception.

- Whether the method can support future PC2 1Tx and 2Tx and PC1.5 1Tx RSD evaluation.

# 3. REFERENCES

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2520224 | Discussion on general methodology for FDD PC1.5 2Tx RSD | Murata Manufacturing Co Ltd. |
| R4-2520290 | Influence of antenna coupling and ACLR on 4x26dBm and 2x26dBm HPUE back-off | Skyworks Solutions Inc. |
| R4-2520580 | On the introduction of PC1.5 with 2Tx for FDD bands | Apple |
| R4-2520732 | Discussion on HPUE for NR single carrier operation | vivo |
| R4-2520770 | Discussion on RF requirements for Rel-20 HPUE | ZTE Corporation, Sanechips |
| R4-2520823 | TN HP related RF requirements for both HHUE and FWA | MediaTek Korea Inc. |
| R4-2521517 | Discussion for Rel-20 HPUE RF requirement | LG Electronics France |
| R4-2522135 | Discussion on HPUE RF requirements | Huawei, HiSilicon |