# 3GPP TSG-RAN WG4 Meeting #117	R4-2523097

Dallas, US, 17th –21th Nov, 2025

Agenda item:			11.4

Source:	Huawei, HiSilicon

Title:	Way Forward for [117][324] LS_BDaT regarding AAS modeling paramters

Document for:	Approval

# Introduction

ECC PT1 has submitted an LS in RP-252992 (R4-2513058) on parameters for AAS BS operating in bands below 1 GHz. The related co-existence work for AAS BS below 1 GHz will be carried out in ECC PT1 and captured in a new ECC Reports that would form basis for the update of the regulatory technical conditions in ECC Decision (15)01, ECC Decision (09)03, and ECC Decision (06)13. ECC PT1 has informed that it has started work towards authorisation of AAS BS in the following frequency bands below 1 GHz:

- 694-791 MHz (referred below as 700 MHz),

- 791-862 MHz (referred below as 800 MHz),

- 880-960 MHz (referred below as 900 MHz).

ECC PT1 expects that AAS BSs in the three bands under study would be deployed in existing sites and hence the deployment parameters would be similar to non-AAS. However, ECC PT1 does not have information about the technical characteristics of AAS BSs in these frequency bands. Therefore, ECC PT1 asked RAN4 for guidance on the following items:

- Unwanted emissions characteristics of AAS BSs operating in the 700 MHz, 800 MHz and 900 MHz bands.

- Whether 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz under consideration.

- A model and associated parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz, both in-band and out of band for frequency bands under consideration.

The following table summarizes antenna model parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz, which were submitted to RAN4#117:

| Parameter | Value |
| --- | --- |
| Antenna pattern model | Extended AAS subarray model |
| Element gain (dBi) | 6.4 (with 2dB Ohmic loss) |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) | Òption 1: 2×4 Option 2: 2x5 |
| Horizontal/Vertical radiating sub-array or element spacing | Horizontal: 0.5 of wavelength for H, Vertical: Option 1: 2.8 of wavelength for V Option 2: 3.5 of wavelength for V |
| Number of element rows in sub-array | 4 |
| Vertical element separation in sub-array | 0.7 of wavelength for V |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Mechanical down-tilt (degrees) | 6 for urban/suburban, 3 for rural |
| Base station horizontal coverage range (degrees) | ±60 |
| Vertical steering range (degree) | 90-100 |

In this WF we capture agreements on the model and associated parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz.

# Topic #1: Model and associated parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz

The antenna model in section 7.1 of TR 38.922 can be used in 700 MHz, 800 MHz and 900 MHz operating bands for further discussion in RAN4. The associated parameters for AAS model are FFS.